3GPP TSG-WG SA2 Meeting #140E	S2-2005013

Elbonia, July 17 – 21, 2020	(revision of S2-20xxxx)

Source:	Qualcomm Incorporated

Title:	Interim conclusion for Key issue #12: NWDAF-assisted RFSP policy

Document for:	Approval

Agenda Item:	8.1

Work Item / Release:	FS_eNA_ph2 / Rel-17

Abstract: This contribution proposes overall evaluation on Key Issue #12.

# 1. Introduction

This contribution proposes interim conclusion on KI#12: NWDAF-assisted RFSP policy.

# 2. Text Proposal

It is proposed to capture the following new text in TR 23.700-91.

* * * * Start of Changes * * * *

# 7	Overall Evaluation

Editor's note:	This clause will provide evaluation of different solutions.

## 7.x	Evaluation for Key Issue #12

Key Issue 12 is about NWDAF-assisted RFSP policy, there are 5 solutions which are solution #4, Solution #40, Solution #41, Solution #42, Solution #43 and Solution #44 proposed for this key issue.

In the current spec about RFSP, AMF decides the RFSP index in use based on the subscribed RFSP index and the allowed NSSAI of the UE. AMF also reports the subscribed RFPS index to PCF, PCF is enabled to modify the PFSP Index used by AMF based on operator policies and slice instance information. There are different procedures proposed in these 5 solutions respectively and Solution #43 proposed a call flow that AMF establishes the AM policy control association with PCF. PCF checks the operator policies and retrieves the AM policy control data from UDR and sends the analytics subscribe to NWDAF which is align with the current RFSP mechanism. It is proposed to discuss the procedure based on the proposal in solution #43.

Regarding the proposal about analytics request for RFSP, there are two different views in all these 5 solutions, solution #4, solution #40 and solution #43 propose to reuse the existing analytics ID (e.g. solution #4 and solution #43 propose to  reuse the Service Experience analytics ID while solution #40 proposes to reuse the User Data Congestion Analytics ID), solution #41 and solution #42 proposed to define new analytics ID for this new key issue and the input and output are proposed respectively. It is easy to apply to this key issue and limited standard impact if the Rel-16 defined analytics ID are enough for RFSP analytics. But whether we need to define new input need further study.

# 8	Conclusions

Editor's note:	This clause is intended to list conclusions that have been agreed during the course of the study item activities. This should also capture the guiding principles and documentation approach for creating CRs to normative specifications within the responsibility of SA WG2.

## 8.x	Key issue #12: NWDAF-assisted RFSP policy

Interim conclusion:

The procedure to collect analytics from NWDAF to PCF and allocate the RFSP for an application considering analytics is based on the proposal in clause 6.43.4 in solution #43. It is FFS whether to define a new analytics ID and the corresponding input and output to the new analytics ID or reuse the Rel-16 supported ananlytics IDs for the RFSP analytics.

* * * * End of Changes * * * *