SA WG2 Meeting #140E	S2-2005030

19 AUG –  2 SEP 2020

Source:	III

Title:	KI #21, New Sol: NWDAF assisting in user plane performance

Document for:	Approval

Agenda Item:	8.1

Work Item / Release:	FS_eNA_ph2/ Rel-17

Abstract of the contribution: This contribution proposes a new solution for NWDAF assisting in user plane performance.

- Discussion

The UPF shall support reporting of data plane usage information to SMF. The UPF shall be capable to support reporting based on different triggers, including: i) Periodic reporting with period defined by the SMF. ii) Usage thresholds provided by the SMF. iii) Report on demand received from the SMF. Also, the SMF shall make sure that the multiple granularity levels required by the reporting keys in the Usage Reporting rules satisfy the following aggregation levels without requiring knowledge of the granularity levels by the UPF: i) PDU Session level reporting. ii) Traffic flow (for both charging and usage monitoring) level reporting iii) per UE, iv) per slice as defined by the reporting keys in the Usage Reporting Rule. Based on this mechanism of PFCP protocol between SMF and UPF via N4 interface, when the OAM or any other NF like AF request NWDAF to have an analysis report about data plane performance, the SMF should forwarding the usage information packages which received by UPF to the NWDAF.

2.	Proposal

It is proposed to include the added text into the TR 23.700-91 as a solution for Key Issue #21.

* * * * First Change * * * *

6.0	Mapping Solutions to Key Issues

Table 6.0-1: Mapping of Solutions to Key Issues

| Solutions | Key Issues |  |  |  |
| --- | --- | --- | --- | --- |
| 1 | 11 |  |  |  |
| 2 | 4 |  |  |  |
| 3 | 15 |  |  |  |
| 4 | 12 |  |  |  |
| X | 21 |  |  |  |

* * * * Second Change  * * * *

6.X	Solution X: NWDAF assisting in user plane performance

6.X.1	Description

This solution is for Key Issue #21: NWDAF assisting in user plane performance.

The objective of this solution is to let SMF forwarding the data plane usage information packages which contains the PDU session and traffic level reporting information elements to NWDAF for supporting the analysis report of data plane performance.

The below are the principles of the solution:

- The UPF shall support reporting of usage information to the SMF as describe in TS23.501. Then SMF should forward the Usage Information package to NWDAF after receiving the NWDAF subscribing data plane performance request.

- The NWDAF analysis the PDU session and traffic flow characteristics.

- The NWDAF reports the estimated top data volume users and their data usage ranking at the slice of interest

6.X.2	Input Data

NWDAF collects PDU session level and traffic level information elements from SMF for a specific UPF. The detailed data are described in Table 6.X.2-1.

Table 6.X.2-1 input data to detect new application from NFs

| Information | Source |  |  | Description |
| --- | --- | --- | --- | --- |
| S-NSSAI | SMF | Identifies the Network Slice for which analytics information is provided. |
| DNN | SMF | Identifies the data network name (e.g. internet) for which analytics information is provided |
| Start/end time | SMF | Start and end time of traffic detection |
| Data volume | SMF | Measured data traffic volume (per slice) |
| Data duration | SMF | Measured data traffic duration (per slice) |
| QoS flow Bit Rate | SMF | The observed bit rate (per slice) |
| Packet transmission | SMF | The observed number of packet transmission (per slice) |
| Payload packets | SMF | The user plane packets collected. |
| QoS flow Bit Rate | UPF | The observed bit rate for UL direction; and The observed bit rate for DL direction |
| QoS flow Packet Delay | UPF | The observed Packet delay for UL direction; and The observed Packet delay for the DL direction |
| Packet transmission | UPF | The observed number of packet transmission |
| Packet retransmission | UPF | The observed number of packet retransmission |
| N4 Session ID | UPF |  |  | Uniquely identifies a session.Uniquely identifies the Packet Detection Rule or Usage Reporting Rule within a session which triggered the report. |
| N4 Session ID | UPF | Identifies the N4 session associated to this URR |
| Rule ID | UPF | Unique identifier to identify this information. |
| Reporting triggers | UPF | One or multiple of the events can be activated for the generation and reporting of the usage report. |
| Periodic measurement threshold | UPF | Defines the point in time for sending a periodic report for this URR (e.g. timeofday). |
| Volume measurement threshold | UPF | Value in terms of uplink and/or downlink and/or total byte-count when the measurement report is to be generated. |
| Time measurement threshold | UPF | Value in terms of the time duration (e.g. in seconds) when the measurement report is to be generated. |
| Event measurement threshold | UPF | Number of events (identified according to a locally configured policy) after which the measurement report is to be generated. |
| Inactivity detection time | UPF | Defines the period of time after which the time measurement shall stop, if no packets are received. |
| Event based reporting | UPF | Points to a locally configured policy which is identifies event(s) trigger for generating usage report. |
| Linked URR ID(s) | UPF | Points to one or more other URR ID. |
| Measurement Method | UPF | Indicates the method for measuring the network resources usage, i.e. the data volume, duration, combined volume/duration, or event. |
| Measurement information | UPF | Indicates specific conditions to be applied for measurements |

6.X.3	Output Analytics

The output analytics of NWDAF is defined in Table 6.X.3-1.

- Table 6.X.3-1. Output for data plane performance analysis

| Information | Description |
| --- | --- |
| S-NSSAI | Identifies the Network Slice for which analytics information is provided. |
| DNN | Identifies the data network name (e.g. internet) for which analytics information is provided |
| List of anomaly events Analytics (1..max) |  |
| >Type | PDU session level or Traffic level |
| >Applicable Time Window | The time period that the analytics applies to |
| >Network Status Indication | Usage level |
| >Data Plane Usage | Report of user plane traffic in the UPF for the accumulated usage of network resources |

6.X.4	Procedures

In this procedure upon analysis info request of user plane performance in any slice of interest, either by OAM or other NFs like AF, the OAM requests the NWDAF to report the data plane characteristics analytics at the slice of interest.

- The OAM is requesting a user plane performance analysis to the NWDAF.

- The NWDAF send Event Exposure Subscribe to SMF to gathering the relative parameters.

- The UPF shall support reporting of usage information to the SMF as describe in TS23.501. Then SMF should forward the Usage Information package to NWDAF after receiving the NWDAF subscribing data plane performance request via Event Exposure Notify message.

Editor’s note: Whether SMF using Nsmf_EventExposure_Notify message to forward the Usage Information or not is FFS.

- The NWDAF analysis the PDU session and traffic flow characteristics.

- The NWDAF reports the data plane performance of intrest slice..

6.X.5	Impacts on services, entities and interfaces

NWDAF:

Need to provide a new analytic for data plane performance information.

SMF:

Need to forward Usage Information packets to NWDAF.

* * * * End of change * * * *