3GPP TSG-WG SA2 Meeting #140E e-meeting 	S2-2005391

Elbonia, August 19 – September 01, 2020	(revision of S2-200xxxx)

Source:	Huawei, HiSilicon

Title:	KI#1, Sol #1: Updates of Solution #1

Document for:	Approval

Agenda Item:	8.4

Work Item / Release:	FS_eNS_Ph2 / Rel-17

Abstract: This contribution proposes updates on Roaming support and evaluation to complete “Solution #1: PCF measurement based Network Slice SLA control for Maximum Number of UEs parameter” in TR 23.700-40.

1. Discussion

This contribution proposes to add an explicit description of roaming support in solution #1: ‘PCF measurement based Network Slice SLA control for Maximum Number of UEs parameter’ by providing a more clear information about roaming support for maximum number of UEs parameter.

This contribution also proposes to add an evaluation section to complete solution #1.

2. Text Proposal

It is proposed to capture the following changes to TR 23.700-40.

# * * * * First change * * * *

6.1	Solution #1: PCF measurement based Network Slice SLA control for Maximum Number of UEs parameter

6.1.1	Introduction

This is a solution to Key Issue #1, "Support of network slice related quota on the maximum number of UEs". This solution assumes the following:

-	It is assumed that Unified Data Repository (UDR) obtains information about network slice related global quota on the slice SLA attributes, e.g., maximum number of UEs from OAM to perform the enforcement of SLA of the number of UEs in a control plane procedure, and supports the functionalities of storage and retrieval of slice SLA data.

-	It is assumed that 5GC may have multiple enforcement points (e.g., PCF instances of a Network Slice) to perform the SLA enforcement of network slice related quota on the maximum number of UEs.

-	It is assumed that multiple enforcement points (i.e., PCF instances) of a network Slice are controlled by a central enforcement point (i.e., a primary PCF) of the network slice. The central enforcement point (i.e., a primary PCF for the S-NSSAI) obtains the global quota of the slice SLA attributes, maximum number of allowed UEs from UDR.

-	It is assumed the central enforcement point (i.e., the primary PCF of a given S-NSSAI) supports the functionalities of controlling distribution of the network slice related quota on maximum number of UEs parameter. The functionalities include the (re)-distribution of the received global quota of each slice SLA attribute from UDR to the associated multiple enforcement points (i.e., secondary PCF instances of the associated S-NSSAI). The central enforcement point (i.e., the primary PCF) can be found by its multiple distributed enforcement points (i.e., secondary PCFs ) based on configuration or the assistance of the UDR.

-	It is assumed that the support of Network Slice related quota on the maximum number of UEs applies to the associated Network Slice even when supported by multiple Network Slice instances.

6.1.2	High-level Description

A global slice SLA information is the global quota of the maximum number of UEs for a given S-NSSAI. A local slice SLA information is the local quota of the maximum number of UEs for a given S-NSSAI, which is stored at the enforcement points, e.g. PCF instances. The local quota of a slice SLA attribute is based on the global quota of a slice SLA attribute.

The proposed solution highlights controlling distribution of network slice related quota at the control plane performed by 5GC NFs: PCF and UDR. It considers a primary PCF of a given S-NSSAI for controlling distribution of the network slice related local quota to its secondary PCF instances. The secondary PCF instances, also known as AM-PCF for the access and mobility management, apply/enforce the associated network slice related local quotas in registration procedure.

The mechanisms of distributed management of a network slice attribute related quota by a central enforcement point (i.e. a primary PCF of a given S-NSSAI) provides a precise control of slice SLA for maximum number of UEs.

6.1.3	Procedures

6.1.3.1	General

The following figure represents a high-level principles and workflow procedure of the solution.

Figure 6.1.3-1: A high-level workflow of the solution

1.	UDR in 5GC obtains from OAM global slice SLA information including a global quota on the maximum number of allowed UEs per S-NSSAI and stores the global slice SLA information. A primary PCF of a given S-NSSAI obtains the global quota on the maximum number of allowed UEs from UDR to be used to perform the distribution of local quota of slice SLA control on the number of UEs. The UDR store the S-NSSAI and its associated primary PCF.

2.	A Primary PCF provides the local slice SLA information including local quotas of the maximum number of allowed UEs per S-NSSAI to its secondary PCF instances (which is for AM policy control) of the related Network Slice (enforcement points). A primary PCF may keep a shared quota of the maximum number of allowed UEs per S-NSSAI (e.g., to handle the delegation requests from its secondary PCFs). The sum of the local quotas to the associated secondary PCF instances of S-NSSAI shall be less or equal to the network slice related global quota of the maximum number of UEs per S-NSSAI.

NOTE:	This local quota distribution of a S-NSSAI can be performed before a UE registration request for the S-NSSAI arrives (e.g., pre-configuration) or it can be triggered by a primary PCF after the registration request(s) for the S-NSSAI by a secondary PCF. It is also possible that a secondary PCF has no local quota granted by a primary PCF, e.g., based on the operator's policy.

3.	When a UE registers for the network slice, the AMF instance of the network slice interacts with the proper PCF instance (enforcement point) for the quota enforcement by re-using existing Registration procedure, i.e. as part of the AM Policy handling. The allowed S-NSSAI(s) of UE are provided to the PCF when the AMF interacts with the PCF.

4.	Quota enforcement:

4a. For the S-NSSAI which needs quota control, each PCF instance (enforcement point) associated to the S-NSSAI maintains the local quota sent by its primary PCF for the associated number of allowed registered UEs. Each PCF instance generates a policy counter to track the local quota (i.e. per S-NSSAI the allowed number of registered UEs, received from primary PCF) and the local quota status (i.e., per S-NSSAI the actual number of registered UEs). Upon receiving UE registration request, the PCF instance decides, based on the local quota status and local quota of the S-NSSAI, the acceptance or rejection of the UE registration request for that S-NSSAI.

4b. In addition to step 4a, the PCF instance may send a delegation request of quota enforcement for the indicated S-NSSAI to its primary PCF, e.g., when the local quota of the S-NSSAI is consumed by the PCF instance or the secondary PCF has no local quota granted per operator's policy. Based on the global/shared quota status of the S-NSSAI the primary PCF (e.g. interaction with UDR) decides the acceptance or rejection of the delegation request for the UE registration on the indicated S-NSSAI and sends a response to the delegation request with the decision to the PCF.

The PCF based quota enforcement naturally supports the handling of slice SLA quota exceeding exceptions or exemptions by allowing special policies for the quota enforcement, e.g., no quota enforcement for the VIP customers or emergency services (e.g., MPS, MCX or PPDR).

The detail procedure of quota enforcement in registration procedure is described in clause 6.1.3.2.

5.	Upon PCF decision from step 4, PCF sends the response of the registration request for S-NSSAI(s) to the serving AMF.  If all requested S-NSSAI included in the UE registration request violates or exceeds the related  quotas of the requested S-NSSAIs the decision for the registration request can be decided based on, e.g., operator policy in PCF, The PCF sends an indication, e.g. rejection message, to the serving AMF. The Registration Reject message is sent to the UE by a serving AMF along with the back-off timer and a suitable cause value. If the registration request of one or more S-NSSAI is accepted, the AMF send the Registration Accept message to the UE, which includes the Allowed S-NSSAI(s), and Rejected S-NSSAI(s) and associated back-off timer if needed.

6.	It is possible at any time the UE initiates the registration procedure to update the allowed S-NSSAI(s), e.g. adding/removing one S-NSSAI to the Allowed NSSAI. In that case the AMF compares the updated Allowed NSSAI with the stored Allowed NSSAI and notifies the status to the PCF. The PCF decides the update the policy counter(s) of associated S-NSSAI(s).

7.	Independently of registration with quota enforcement procedure in step 1-6, local slice SLA quota re-distribution can be triggered by (slave) PCF instances or the primary PCF of the network slice at any time. The detail procedure of controlling (re)-distribution of local quota is described in clause 6.1.3.4.

7a.	PCF instances (enforcement points) may request their local quotas update for the indicated S-NSSAI (e.g., when the local quotas are (about to) consumed) to the primary PCF. The primary PCF based on the received requests may re-calculate and provide the updated local quota of the maximum number of UEs associated with the indicated S-NSSAI to one or more PCF instances. The primary PCF supports mechanisms for the (re)-distribution of local quota(s) to one or more PCF instances of associated S-NSSAI based on the global quota status of registered number of UEs of the indicated S-NSSAI.

7b.	Independently of step 7a, one or more PCF instances (enforcement points) shall report/notify the local quota statuses per S-NSSAI to the primary PCF (e.g., periodically or event based). Based on the reported local quotas the primary PCF can track the global status of number of registered UEs of indicated S-NSSAI and provides to one or more PCF instances an updated local quota (i.e., quota re-distribution) if required. It enables 5GS to know about the current number of UEs accessing the network slice, i.e. specific S-NSSAI.

8.	Independently of registration with quota enforcement and controlling local quota (re)-distribution, a de-registration for a S-NSSAI can be triggered by a UE (or network) any time.

8a.	A UE triggers a de-registration request to the serving AMF. The AMF triggers the update of policy counter to the associated PCF during AM policy association termination procedure.

8b. Upon receiving a deregistration request from the AMF, the secondary PCF of the S-NSSAI increases the remaining local quota of the S-NSSAI by one. If the secondary PCF has no local quota been granted per operator's policy, the secondary PCF may interact with the primary PCF for the quota update.

8c. The response of the deregistration of the S-NSSAI is sent to the UE by the serving AMF.

The detail procedure of deregistration of a S-NSSAI is described in clause 6.1.3.3.

In the roaming case, the quota control can be in the VPLMN or HPLMN. For more detail, it is described in clause 6.1.3.X.

6.1.3.2	General Registration with Quota Enforcement

Figure 6.1.3.2-1: General registration procedure with quota update

1.	Registration procedure (steps 1-15) according to clause 4.2.2.2.2 in TS 23.502 [6].

2.	The AMF performs an AM Policy Association Establishment/Modification. The allowed S-NSSAI(s) and its associated operation (i.e. increment or decrement of the associated policy counter) is also provided to the PCF.

3.	For each indicated allowed S-NSSAI, per the received allowed S-NSSAI(s) and its associated operation the PCF verifies the status of the associated local quota based on its policy counter of a slice parameter, the number of registered UEs. A policy counter per S-NSSAI indicates the local quota and its status, which comprises information of remaining number of allowed UEs and consumed number of UEs. If the number of UEs for the specific S-NSSAI is still within a valid bound, there is no negative affect for the registration request of that S-NSSAI.

4a. Upon identification of no quota left or the secondary PCF has no local quota granted, the PCF instance shall decide for the delegation of quota enforcement to the primary PCF. The delegation of quota enforcement message includes the S-NSSAI and its associated operation.

4b. The primary PCF upon receiving the delegation message, verifies the status of the shared quota of the requested S-NSSAI and decides the acceptance or rejection based on the status of a shared quota of each requested S-NSSAI. The primary PCF has its own policy counter per S-NSSAI to manages the status of a shared quota in the same way as the secondary PCF per S-NSSAI4c. Based on step 4b, the primary PCF sends a message to its requested PCF instance. The response message includes an indication to accept the registration or rejection of the indicated S-NSSAI.

5.	Based on step 3 or 4b, the local quota status is updated (e.g., increase the associated policy counter value for the indicated S-NSSAI by one).

6.	The PCF instance sends the response of quota enforcement (i.e., acceptance or rejection) of S-NSSSAI(s), based on step 3 or step 4c, to the AMF instance.

If more than one allowed S-NSSAI is provided to PCF, it is possible that part of S-NSSAIs is authorized and others are rejected per the action taken in step 3 or 4c.

Alt A: UE registration request includes at least one S-NSSAI, which does not violate or exceed the local quota without delegation of quota enforcement, or the shared/global quota with delegation of quota enforcement.

7.	Registration procedure (steps 17-22) according to clause 4.2.2.2.2 in TS 23.502 [6].

If one S-NSSAI is rejected, it is included in the rejected NSSAI and associated with cause value and back-off timer.

If the transaction is not completed successfully, e.g. the registration accept message is not received by the UE, the incorrect count at the PCF is corrected when the AMF deregister the UE due to the periodic registration timer expires.

Alt B: All the S-NSSAIs in the UE registration request violates or exceeds the local quota without delegation of quota enforcement, or the shared/global quota with delegation of quota enforcement.

8.	The Network cleans the related state, e.g. the AMF changes the UE registration state as deregistered.

9.	The AMF sends a registration reject message to the UE, which includes the Rejected NSSAI, associated cause value and back-off timer.

In the following when the UE initiates the registration procedure to update the registered S-NSSAI, e.g. adding one S-NSSAI to the Allowed NSSAI, the AMF compares the updated Allowed NSSAI with the stored Allowed NSSAI.

If one S-NSSAI is added into the Allowed NSSAI, the AMF checks with the PCF on whether this Allowed NSSAI is permitted per the status of quota information. If one S-NSSAI is removed from the Allowed NSSAI, the AMF notifies this status to the PCF.

6.1.3.3	Deregistration with Slice SLA Quota Update

Figure 6.1.3.3-1: Deregistration procedure with Slice SLA quota update.

UE is deregistered, which slice Quota management is required.

1.	UE or network initiates a deregistration procedure according to (steps 1-5) in clause 4.2.2.3.2 or (step 1-4) in clause 4.2.2.3.3 in TS 23.502 [6], accordingly.

2.	During the AM policy association termination procedure as defined in clause 4.16.3.2 in TS 23.502 [6], for each S-NSSAI the PCF instance of the deregistered UE decreases the policy counter value of a given S-NSSAI by one.

Optionally, If the PCF instance of the UE does not have local quota granted, e.g., as per operator's policy, the PCF instance delegates the request of policy counter update for the deregistration of the S-NSSAI to the primary PCF. The primary PCF decreases the policy counter value of the indicated S-NSSAI and sends the response to the secondary PCF.

NOTE:	If the PCF instance has local quota granted before, during the de-registration procedure, the involvement of the primary PCF is not required. The local quota reporting and re-distribution mechanism described in clause 6.1.3.4 can be used to recover the previously delegated requests to the primary PCF of the indicated S-NSSAI.

3.	UE or network initiates deregistration procedure according to (steps 6a-8) in clause 4.2.2.3.2 or (steps 5a-7) in clause 4.2.2.3.3 in TS 23.502 [6], accordingly.

6.1.3.4	Controlling (re)-distribution of local quota of Slice SLA attributes

Figure 6.1.3.4-1: Controlling (re)-distribution of local quotas of slice SLA attributes

The procedures of local quota distribution and re-distribution between a primary PCF and its secondary PCFs of a specific S-NSSAI are described in Figure 6.1.3.4-1.

1.	Local quota distribution.

Based on the received global quota of maximum number of allowed UEs from UDR, a primary PCF may proactively distributes the local quotas of maximum number of allowed registered UEs to the associated PCFs of the specific S-NSSAI. For example, a local quota is given to the secondary PCF to reduce signalling.

NOTE:	How much local quota shall be distributed on each PCF instance of S-NSSAI can be determined based on Operator policy (e.g., equal local quota distribution, with respect to the estimated load of PCFs, based on capabilities of PCF instances, etc.)

2.	For the specific S-NSSAI, Local quota re-distribution - Local quota status reporting.

2a.	The primary PCF subscribes the status of local quotas to its PCF instances, which may include time interval or an event trigger, e.g. when the remaining quota is less than 5% of the local quota or above a threshold.

2b.	Each PCF instance triggers the report of the status of the local quota to the primary PCF based on, e.g., events (as described in 2a) or periodically.

2c.	Optionally, the primary PCF requests the status of local quotas to the specific PCF instances.

2d.	Upon receiving the request, the requested PCF instances send the statuses of local quota report to the primary PCF.

3.	For the specific S-NSSAI, Local quota re-distribution - triggering.

3a.	The primary PCF may trigger the update of quota re-distribution to the other secondary PCF instances without the requests of quota update from these PCFs. The trigger is initiated based on the status of global quota or based on the local quota updates from other PCF instances of the same S-NSSAI, as described in 2a.

3b.	One or more PCF instance send request(s) for local quota(s) update to the primary PCF.

3c.	Upon receiving of one or more local quota update requests from secondary PCFs, the primary PCF provides re-distribution of local quotas to one or more PCF instances.

6.1.3.X	Controlling quota of Slice SLA attribute of Maximum Number of UEs at Roaming

Depending on the operator’s requirement, two options for quota control can be considered.

Quota control at the VPLMN: For the Roaming UEs, it is assumed that the local quota for the allowed number of Roaming UEs per Subscribed S-NSSAI per home PLMN is available at vPLMN, e.g., as per SLA or Roaming agreement. The AM-PCF and primary PCF, in vPLMN of the given S-NSSAI, enforces Roaming UEs’ requests for the registration or deregistration of the S-NSSAI and updates (i.e., increase or decrease) the policy counter of the S-NSSAI per home PLMN accordingly. The procedures of slice SLA quota enforcement and controlling for Roaming UEs are the same as per Clauses 6.1.3.2 - 6.1.3.4 with the additional HPLMN’s S-NSSAI information. That means for the same vPLMN only the indicated HPLMN’s S-NSSAI(s) need quota control.

Quota control at the HPLMN: If the primary PCF in hPLMN needs to perform controlling or enforcing the requests of its visiting UEs, the interaction between AM-PCFs in VPLMN and the primary PCF in hPLMN shall take place ( e.g., via N24). The further considerations of any necessary interaction between vPLMN and hPLMN for the roaming UEs will be left out for the normative work. The H-PLMN primary PCF may also distribute a local quota to the AM-PCF in VPLMN similar as the primary PCF distribute to the secondary PCF in the non-roaming case.

6.1.4	Impacts on services, entities and interfaces

UDR: obtaining, storing and allowing retrieval of global slice SLA information including a global quota on the maximum number of registered UEs.

PCF: A primary PCF of a given S-NSSAI for distributing/providing the network slice related shared/local quota on the maximum number of UEs; and its secondary PCF to apply/enforce the network slice related local quota on the number of UEs

AMF: handling of quota enforcement decision on the number of UEs

UE: handling of back-off timer and a (new) cause value as a rejection response

Editor's note:	Impacts on existing services and interfaces are FFS for the normative work.

6.1.X	Evaluation

The evaluation of the solution is described based on the following criteria.

- Scalability:

The solution allows a scalable solution to support a single instance or multiple instances of quota enforcement NFs without degradation in performance of the solution for slice SLA control and enforcement of the maximum number of UEs parameter.

- Reusability:

The solution reusability for other Key Issues is very high. The PCF measurement based slice SLA control and enforcement is a generic solution which can apply not only for KI#1 but also for KI#2 as described in solution #7: “Support of Network Slice SLA for Maximum Number of PDU sessions parameter”. In addition, the PCF based solution maintains the global quota status of a slice SLA attribute so that it can support the event notification of Network Slice quota of the slice SLA attribute to the consumer, by covering KI#4.

In addition, the PCF based solution reuses the existing NFs and existing control plane procedures effectively and efficiently.

- Accuracy of Quota Control and Enforcement:

By introducing global quota, local quota and shared quota concept, the PCF based solution enables to control the slice SLA quota accurately. It means that the secondary PCFs control and enforce accurately UEs’ requests for a S-NSSAI locally based on its local quota. In a case of a local quota update (e.g., about to be consumed) or a delegation request (e.g., no remaining local quota) by a secondary PCF, the primary PCF controls and enforces accurately based on the shared quota and global quota status of slice SLA attribute.

By providing quota control and management mechanisms by primary/secondary PCFs, 5GS can overcome inaccurate quota control and enforcement. An example of inaccurate quota control means a quota enforcement NF may accept the requests over the limit of the global quota as the quota enforcement NF do not receive the message of global quota exceeding before the UEs’ requests have been accepted.

- Rapid Response of Quota enforcement:

The PCF based solution allows instantaneous decision for an acceptance or rejection of the UEs’ requests based on the status of local quota or shared quota or global quota of a slice SLA attribute during the registration or deregistration procedures. Since the rapid response of quota enforcement is an important requirement of the solution, to support this feature, the enforcement NF shall be aware of the actual number of remaining quota of a specific slice SLA attribute in a timely manner.

- Handling of Quota exceeding Policy:

The PCF based solution naturally supports the handling of slice SLA quota exceeding situation, exceptions or exemptions by allowing special policies, e.g., for handling of VIP customers or special DNNs or emergency services (e.g., MPS, MCX or PPDR).

- Signalling:

The PCF based solution has very low signaling load by efficiently utilizing the existing procedures (AM policy association establishment/modification and AM policy association termination procedure as defined in in TS23.502).

If the local quotas are allocated to the secondary PCFs, the quota enforcement mechanisms by secondary PCFs do not require additional signaling for slice related quota management on maximum number of UEs for a given S-NSSAI. It is also notable that the signalling due to the controlling (re)-distribution of local quota of Slice SLA attributes and the two steps enforcement scheme proposed by Solution #1 are infrequent events and can be negligible in relation to the performance of the solution.

# * * * * End of changes * * * *