3GPP TSG-SA2 Meeting #140-e	S2-2005476

# Online, , 19st Aug 2020 – 2nd Sep 2020

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.502 | CR | draftCR | rev | - | Current version: | 16.5.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | N4 Node Level procedures for satellite backhaul connection |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Samsung |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GSAT_ARCH |  |  |  |  |  | Date: |  |  | 2020-08-13 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | The SA2 #139e approved contribution S2-2004705 with a comment below: Whether and when QoS limitation defined in Solution #11 needs to be considred for UPF selection may be discussed in the normative work N4 Node level procedure to inform QoS limitation per GTP-U peer is missing, so that SMF cannot collect all QoS limitation information of the satellite backhaul connections. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add N4 level procedures that UPF informs all QoS limitation due to satellite backhaul connected GTP-U peers in N4 Association Setp/Update procedures. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | QoS limitation awared UPF selection cannot be supported. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.xx (New) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

*************** First change (all text is new) **************

4.xx.x	N4 Node Level procedures

4.xx.x.1	N4 Association Setup Procedure

This procedure is based on the procedure described in clause 4.4.3.1 with the following differences and clarifications:

-	In Figure 4.4.3.1-1, when receiving an N4 Association Setup Request, the UPF shall provide QoS limitation Information (a list of Satellite Category, a list of time delay for each GTP-U peer) in the N4 Association Setup Response.

-	In Figure 4.4.3.1-2, the UPF provides QoS limitation Information (a list of Satellite Category, a list of time delay for each GTP-U peer) in the N4 Association Setup Request.

4.xx.x.2	N4 Association Update Procedure

This procedure is based on the procedure described in clause 4.4.3.2 with the following differences and clarifications:

-	In Figure 4.4.3.2-2, the UPF provides QoS limitation Information (a list of Satellite Category, a list of time delay for each GTP-U peer) in the N4 Association Update Request.

*************** End of changes ***************