SA WG2 Meeting #140-e	S2-2005632

Aug 19– Sept 02 2020, Online 	(Revision of S2-2004274)

Source:	NTT DOCOMO

Title:	KI#4, Sol#2: update to address the editor’s note

Document for:	Approval

Agenda Item:	8.1

Work Item / Release:	FS_eNA_ph2 / Rel-17

Abstract of the contribution: This contribution addresses the editor’s note

# Discussion

Editor's note:	How input data for slice load analytics is collected by NWDAF is FFS.

In the solution, the NWDAF provides the slice service experience and slice load information, but it is not explained how the NWDAF collect the data to determine the respective analytic ID.

In order the NWDAF determine the slice service and load, the NWDAF need to collect the data from the slice associated NFs (e.g. AMF, SMF, UPF, NRF, etc.,). However, the NWDAF lacks the knowledge of NFs associated to the slice. Therefore, it is proposed that NWDAF subscribed to the OAM to notification of a new slice configured in the network or NWDAF retrieves the NFs associated to a slice from the OAM. After, the NWDAF can collect the data from the slice associated NFs.

Up on slice information, the NWDAF can collect the data from NFs associated to slice. For example, to determine the slice load, the NWDAF should understand the NF associated to the slice, such that NWDAF determine which source can be used to collect the data. For instance, NWDAF can use the NRF as a source to collect the AMF and SMF load or OAM for the RAN slice information. Based on the collected data from different sources, the NWDAF can aggregate and determine the overall slice load information.

# Proposal

It is proposed to include the following update in the eNA TR.

********************************  START OF CHANGES **************************************

## 6.2	Solution #2: Remaining aspects on how to ensure that slice SLA is guaranteed

### 6.2.1	Description

This solution addresses Key Issue #4 by leveraging NWDAF slice level analytics to trigger a slice load distribution mechanism aimed at assisting with SLA guarantee. Relying mostly on observed service experience analytics for a Network Slice as defined in TS 23.288 [5], the proposed load distribution mechanism can act at Network Slice level, Network Slice instance level, or both, and the consumer NF of analytics that also triggers the mechanism can be NSSF, or in its absence, AMF.

#### 6.2.1.1	General

In this solution, NSSF or AMF subscribe to slice service experience analytics and NWDAF collects the corresponding data as detailed in TS 23.288 [5]. In addition, NSSF or AMF subscribe to slice load analytics to avoid selecting an overloaded slice for new UE registrations and/or PDU session establishments.

To assist with slice SLA, NSSF/AMF continuously analyse statistics and/or predictions for both service experience and load of the slice. The internal logic of NSSF/AMF determine when a load distribution decision is required to address an issue identified by processing the analytics. For example, when a slice is detected or forecasted to experience service experience degradation, new UE registrations or PDU sessions may not be assigned to that slice anymore by triggering a Network Slice load distribution mechanism. The proposed mechanism can act on both Network Slice and Network Slice instance levels, and s based on the simple idea of establishing a restriction for UE registrations and/or PDU sessions on a Network Slice or Network Slice instance whenever NSSF/AMF decide to do so based on the slice level NWDAF analytics. The restriction(s) may be enforced by a quota management NF when applied to a Network Slice, as described in TR 23.700-40 [10].

In addition to NSSF/AMF, OAM may also simultaneously subscribe to both slice service experience and slice load analytics from NWDAF. OAM may use these analytics in addition to other management plane data to make eventual management decisions on a slice (e.g. scaling of a slice) and, if required, it may inform 5GC of such decisions.

#### 6.2.1.2	Procedures

NOTE:	In all the procedures below, AMF* indicates the AMF instance involved in the call flow may refer to any AMF instance(s) within a same AMF Set.

##### 6.2.1.2.1	Slice information collection

Figure 6.2.1.2-1: Procedure for slice information collection.

- It is a generic subscription that NWDAF subscription for slice creation notification with the OAM.

- Upon creation or configuration of a new slice in the network, the OAM system notifies the NWDAF with the S-NSSAI, optionally NSI, and associated NFs, slice SLA to the NWDAF.

- The NWDAF (e.g. when a NF consumer requests the NWDAF for slice information (e.g. for slice load of a S-NSSAI#1)) explicitly requests the OAM system slice information. In this case, NWDAF provides the S-NSSAI (optionally NSI) to the OAM.

- OAM system provides the slice information along with the associated NFs list and SLA to the NWDAF.

##### 6.2.1.2.2	NSSF based solution

Figure 6.2.1.2-1: Procedure for NWDAF-assisted slice SLA guarantee solution based on NSSF

0a.-0b.	OAM creates a new slice and configures initial resources in RAN and in CN allocated to the slice. The slice is also configured to support a maximum number of UEs and/or PDU sessions. If a NWDAF subscribed for the slice creation notification, the OAM notifies the slice information as described in clause 6.2.1.2.1.

1a.-1b.	NSSF and optionally OAM subscribe to slice service experience analytics from NWDAF. One or multiple subscriptions to one or multiple S-NSSAI(s), NSI ID(s) are possible.

2.	NSSF and optionally OAM subscribe to slice load analytics from NWDAF.

3.	New UEs get continously registered in the slice.

NOTE 1:	Steps 0b and 3 should be performed according to the currently ongoing eNS_ph2 work on how to enforce UE and/or PDU session quotas on a Network Slice.

4. NWDAF collects input data required to derive slice service experience analytics or slice load analytics as described in TS 23.288 [5].

NOTE 2:	Figure 6.2.1.2.2-1 does not show the exhaustive list of NFs that may provide input data to NWDAF. Nevertheless, the NWDAF collects the slice load from one or more slice associated NFs i.e. collection either via directly with the slice associated NF(s) or via other sources (like OAM, NRF) e.g. as described in Table 6.5.2-1, TS 23.288 [5]. For the RAN slice information, NWDAF uses the OAM as a source to collect the RAN slice information.

5.	Slice service experience analytics are continuously delivered by NWDAF to NSSF and optionally to OAM.

6.	Slice load analytics are continuously delivered by NWDAF to NSSF and optionally to OAM.

NOTE 3:	How OAM may use NWDAF slice level analytics needs to be determined by SA5

7.	NSSF continuously analyzes statistics and predictions on slice load and service experience aiming to improve slice assignment decisions.

8.	[OPTIONAL] OAM continuously monitors slice SLA. For that purpose it may use as optional inputs NWDAF slice level analytics in addition to other management data.

9.	NSSF decides to trigger action based on the slice analytics provided by NWDAF. Hence, NSSF may trigger either Network Slice load distribution, or Network Slice instance load distribution, or both.

NOTE 4:	This solution assumes that more than one slice may have been previously created by OAM.

10a.	[OPTIONAL] NSSF triggers a Network Slice load distribution procedure (details below).

10b.	[OPTIONAL] NSSF triggers a Network Slice instance load distribution procedure (details below).

NOTE 5:	Step 10b can only be applied if the deployment choice of the operator allows Network Slice instance(s) in the 5GC, and those are identified via NSI ID(s)..

11.	[OPTIONAL] OAM may take management decisions based on the collected inputs including management data and NWDAF analytics. If required, OAM may inform 5GC of such management decisions.

(Figure 6.2.1.2.2-1, Step 10a) NSSF triggers a Network Slice load distribution procedure:

Figure 6.2.1.2.2-2: Detail of Step 10a in Figure 6.2.1.2.1-1

1.	Based on the slice level analytics provided by NWDAF, NSSF concludes a Network Slice restriction is required.

2a.	NSSF sends a Network Slice restriction to AMF.

2b.	The UE initiates registration procedure requesting registration on the restricted Network Slice (via e.g. S-NSSAI).

2c.	AMF/NSSF determine whether the Network Slice restriction can be satisfied for UE registration leveraging the Network Slice registration procedures defined in TS 23.501 [2].

2d.	The UE registration is completed or rejected. If completed, PDU sessions may be establised for the registered UE if/when required.

Editor's note:	An alignment with FS_eNS_Ph2 may be required for this solution based on the outcome of FS_eNS_Ph2.

(Figure 6.2.1.2.2-1 Step 10b) NSSF triggers a Network Slice instance load distribution procedure:

Figure 6.2.1.2.2-3: Detail of Step 10b in Figure 6.2.1.2.1-1

1.	Based on the slice level analytics provided by NWDAF, NSSF concludes a Network Slice instance restriction is required.

2.	[OPTIONAL] NSSF sends a Network Slice instance restriction to AMF.

3.	New UE initiates registration procedure requesting registration on the Network Slice (identified via S-NSSAI) containing the restricted Network Slice instance (identified via NSI ID).

4.	[OPTIONAL] If AMF handles NSI ID(s) information during UE registration, it may take into account the Network Slice instance restriction to assign NSI ID to the new UE registration.

5.	[OPTIONAL] AMF and NSSF may interact as described in TS 23.501 [2] to determine the NSI ID to be used for the new UE registration.

6.	UE registration is completed or rejected

7.	A UE already registered in the network requests a new PDU session establishment according to TS 23.502 [3].

8.	[OPTIONAL] If AMF handles NSI ID(s) information during PDU session establishment, it may take into account the Network Slice instance restriction to assign NSI ID to the new PDU session establishment.

9.	[OPTIONAL] SMF selection is performed according to TS 23.502 [3] accounting for the restricted NSI ID(s).

10.	PDU session establishment is completed.

##### 6.2.1.2.3	AMF based solution

Figure 6.2.1.2.3-1: Overall procedure for NWDAF-assisted slice SLA guarantee solution based on AMF

0a.-0b.	OAM creates a new slice and configures initial resources in RAN and in CN allocated to the slice. The slice is also configured to support a maximum number of UEs and/or PDU sessions. If a NWDAF subscribed for the slice creation notification, the OAM notifies the slice information as described in clause 6.2.1.2.1.

1a.-1b.	AMF and optionally OAM subscribe to slice service experience analytics from NWDAF. One or multiple subscriptions to one or multiple S-NSSAI(s), NSI ID(s) are possible.

2.	AMF and optionally OAM subscribe to slice load analytics from NWDAF.

3.	New UEs get continously registered in the Network Slice.

NOTE 1:	Steps 0b and 3 should be performed according to the currently ongoing eNS_ph2 work on how to enforce UE and/or PDU session quotas on a Network Slice.

4.	NWDAF collects input data required to derive slice service experience analytics as described in TS 23.288 [5]. Nevertheless, the NWDAF collects the slice load from one or more slice associated NFs i.e. collection either via directly with the slice associated NF(s) or via other sources (like OAM, NRF) e.g. as described in Table 6.5.2-1, TS 23.288 [5]. For the RAN slice information, NWDAF uses the OAM as a source to collect the RAN slice information.

NOTE 2:	Figure 6.2.1.2.3-1 does not show the exhaustive list of NFs that may provide input data to NWDAF.

5.	Slice service experience analytics are continuously delivered by NWDAF to AMF and optionally to OAM.

6.	Slice load analytics are continuously delivered by NWDAF to AMF and optionally to OAM.

NOTE 3:	How OAM may use NWDAF slice level analytics needs to be determined by SA5

7.	AMF continuously analyzes statistics and predictions on slice load and service experience.

8.	[OPTIONAL] OAM continuously monitors slice SLA. For that purpose it may use as optional inputs NWDAF slice level analytics in addition to other management data.

9.	AMF decides to trigger action based on the slice analytics provided by NWDAF. Hence, AMF may trigger either Network Slice load distribution, or Network Slice instance load distribution, or both.

NOTE 4:	This solution assumes that more than one slice may have been previously created by OAM.

10a.	[OPTIONAL] AMF triggers Network Slice load distribution (details below).

10b.	[OPTIONAL] AMF triggers Network Slice instance load distribution (details below).

NOTE 5:	Step 10b can only be applied if the deployment choice of the operator allows Network Slice instance(s) in the 5GC, and those are identified via NSI ID(s).

11.	[OPTIONAL] OAM may take management decisions based on the collected inputs including managament data and NWDAF analytics. If required, OAM may inform 5GC of such management decisions.

(Figure 6.2.1.2.3-1, Step 10a) AMF triggers a Network Slice load distribution procedure:

Figure 6.2.1.2.3-2: Detail of Step 10a in Figure 6.2.1.2.2-1

1.	Based on the slice level analytics provided by NWDAF, AMF concludes a Network Slice restriction is required.

2a.	The UE initiates registration procedure requesting registration on the restricted Network Slice (via e.g. S-NSSAI) .

2b.	AMF determines whether the Network Slice restriction can be satisfied for UE registration leveraging the Network Slice registration procedures defined in TS 23.501 [2].

2c.	The UE registration is completed or rejected. If completed, PDU sessions may be establised for the registered UE if/when required.

Editor's note:	An alignment with FS_eNS_Ph2 may be required for this solution based on the outcome of FS_eNS_Ph2.

(Figure 6.2.1.2.3-1, Step 10b) AMF triggers a Network Slice instance load distribution procedure:

Figure 6.2.1.2.3-3: Detail of Step 10b in Figure 6.2.1.2.2-1

1.	Based on the slice level analytics provided by NWDAF, AMF concludes a Network Slice instance restriction is required.

2.	New UE initiates registration procedure requesting registration on the Network Slice (identified via S-NSSAI) containing the restricted Network Slice instance (identified via NSI ID).

3.	[OPTIONAL] If AMF handles NSI ID(s) information during UE registration, it may take into account the Network Slice instance restriction to assign NSI ID to the new UE registration.

4.	UE registration is completed or rejected

5.	A UE already registered in the network requests a new PDU session establishment according to TS 23.502 [3].

6.	[OPTIONAL] If AMF handles NSI ID(s) information during PDU session establishment, it may take into account the Network Slice instance restriction to assign NSI ID to the new PDU session establishment.

7.	SMF selection is performed according to TS 23.502 [3] accounting for the restricted NSI ID(s).

8.	PDU session establishment is completed.

### 6.2.2	Impacts on services, entities and interfaces

NWDAF:

-	Needs to provide both slice service experience and slice load analytics to NSSF, AMF and OAM.

NSSF/AMF:

-	Subscribes to both slice service experience and slice load analytics from NWDAF;

-	Continuously analyses slice level analytics (i.e. service experience and load) provided by NWDAF;

-	Based on analytics, trigger Network Slice and/or Network Slice instance load distribution procedure.

-	Support Network Slice/Network Slice instance restriction.

OAM:

-	May take management decisions using NWDAF slice level analytics as inputs and inform 5GC if required.

- 	Provides the slice information notification service to the NWDAF.

******************************** END OF CHANGES **************************************