SA WG2 Meeting #140E	S2-2005860

August 19 - September 2, 2020, Elbonia	(revision of S2-200xxxx)

Source:	Apple

Title:	KI #5, Sol#12: solution evaluation

Document for:	            Approval

Agenda Item:	8.4

Work Item / Release:	FS_eNS_Ph2 / Rel-17

Abstract: This contribution adds solution evaluation to solution #12.

The following updates are proposed to be added to Solution #12 in TR 23.700-40 0.4.0.

* * * * Start of Changes * * * *

### 6.12.5	Evaluation

This solution builds on solution #2 / solution#10 that introduce the NSQ function that is a centralised quota management with PLMN granularity. NSQ introduces a minimal impact to the system as NSQ interacts with the SMF only. The solution has no UE impacts.

During SM procedures, NSQ monitors actual Slice-AMBR and when it exceeds maximum allowed Slice-AMBR, NSQ instructs SMF to dynamically adjust the session-AMBR and MFBR values of the existing PDU session by a certain threshold or reject the PDU session establishment. Then, SMF initiates PDU session modification procedure to reduce Session-AMBR/MFBR values in existing PDU sessions. If there are numerous existing PDU sessions, this can result in excessive NAS SM signalling.

* * * * End of Changes * * * *