SA WG2 Meeting #140E (e-meeting)	S2- 2005117r05

# August 19th – September 2nd, 2020, Elbonia		                             (revision of S2-20yyyyy)

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.502 | CR | 2346 | rev | - | Current version: | 16.5.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Time in trajectories of Expected UE Behaviour |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S2 |  |  |  |  |  |  |  |  |  |
| Work item code: | eNA |  |  |  |  |  | Date: |  |  | 2020-08-11 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | An AF that might have got Expected UE Behaviour analytics reports from NWDAF may want to provision such data into UDM for further use. This procedure is describedf in TS 23.502 clause 4.15.4.2. Although the Expected UE Behaviour data provisioned from AF to NEF and further to UDM contains expected trajectory of the UE, it does not include the time and date of the week when the UE is expected to be at each of the locations of the trajectory. Without that information the AMF cannot take any decision based on the trajectory, since it would not be able to distinguish simple cases, e.g., a UE who is on working days and working hours at a location (working location) and the rest of time at another location (home). This is in spite an AF might have got time and day of week in Analytic reports from NWDAF. In addition, this contribution aligns with stage 3 Clause 5.10.2.2.5, TS 29.122 that defines “UmtLocationArea5G” data type that represents the user location area describing the UE moving trajectory, which is sent from the AF, including time of the day of the stay. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add Expected Time and Day of Week in Trajectory to the list of parameters of Expected UE Behaviour. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Misalignment with stage-3, TS 29.122. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.15.6.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

*** BEGIN CHANGES ***

#### 4.15.6.3	Expected UE Behaviour parameters

These Expected UE Behaviour parameters characterise the foreseen behaviour of a UE or a group of UEs. Sets of these parameters may be provided via the NEF to be stored as part of the subscriber data. Each parameter within the Expected UE Behaviour shall have an associating validity time. The validity time indicates when the Expected UE Behaviour parameter expires and shall be deleted by the related NFs. The validity time may be set to indicate that the particular Expected UE Behaviour parameter has no expiration time. When the validity time expires, the related NFs delete their local copy of the associated Expected UE Behaviour parameter(s). The provision procedure of the Expected UE Behaviour is realized by external parameter provision procedure defined in clause 4.15.6.2.

The Expected UE Behaviour parameters stored as AMF-Associated Expected UE Behaviour parameters which is per UE level and SMF-Associated Expected UE Behaviour parameters which is per PDU session level in UDM. AMF retrives the AMF-Associated Expected UE Behaviour parameters from UDM which may related to both PDU session(s) and SMS transmission. SMF retrives the SMF-Associated Expected UE Behaviour parameters from UDM for the specific PDU session. AMF and SMF uses the Expected UE Behaviour parameters as described in clause 5.4.6.2 in TS 23.501 [2].

Table 4.15.6.3-1: Description of Expected UE Behaviour parameters

| Expected UE Behaviour parameter | Description |
| --- | --- |
| Expected UE Moving Trajectory | Identifies the UE's expected geographical movement Example: A planned path of movement |
| Stationary Indication | Identifies whether the UE is stationary or mobile [optional] |
| Communication Duration Time | Indicates for how long the UE will normally stay in CM-Connected for data transmission. Example: 5 minutes. [optional] |
| Periodic Time | Interval Time of periodic communication Example: every hour. [optional] |
| Scheduled Communication Time | Time and day of the week when the UE is available for communication. Example: Time: 13:00-20:00, Day: Monday. [optional] |
| Battery Indication | Identifies power consumption criticality for the UE: if the UE is battery powered with not rechargeable/not replaceable battery, battery powered with rechargeable/replaceable battery, or not battery powered. [optional] |
| Traffic Profile | Identifies the type of data transmission: single packet transmission (UL or DL), dual packet transmission (UL with subsequent DL or DL with subsequent UL), multiple packets transmission [optional] |
| Scheduled Communication Type | Indicates that the Scheduled Communication Type is Downlink only or Uplink only or Bi-directional [To be used together with Scheduled Communication Time] Example: <Scheduled Communication Time>, DL only. [optional] |
| Expected Time and Day of Week in Trajectory | Identifies the time and day of week when the UE is expected to be at each location included in the Expected UE Moving Trajectory. [optional] |

The Expected UE Moving Trajectory may be used by the AMF. All other parameters may be used by the AMF and by the SMF.

The Scheduled Communication Type and the Traffic Profile should not be used by the AMF to release the UE when NAS Release Assistance Information from the UE is available.

In the case of NB-IoT UEs, the parameters may be forwarded to the RAN to allow optimisation of Uu resource allocation for NB-IoT UE differentiation.

*** END OF CHANGES ***