SA WG2 Meeting #140-E	S2-2005166r01

Elbonia, 19 August – 02 September 2020

Source:	Samsung

Title:	KI#1, New Sol: ProSe group discovery

Document for:	Approval

Agenda Item:	8.8

Work Item / Release:	FS_5G_ProSe / Rel-17

Abstract of the contribution: This contribution proposes a new Solution for Key Issue #1 to support PC5 group discovery, management and identifiers provisioning in Rel-17.

# 1. Background

Solutions #4, #22 and #37 as agreed in the past 3GPP SA2 meetings address KI#1 (and KI#2) to support PC5 group communication for commercial services and public safety applications in Rel-17.

When it comes to commercial services (e.g. NCIS), more dynamic assignment of group identifiers is expected. Frequent provisioning by (UE Policy) PCF may result in control-plane level network congestion.

For commercial applications, Application Server (e.g. over PC1) may support group identifiers provisioning.

Based on the above, this paper proposes a new solution to use V2X-based group management as the baseline, similar to Solution #37. However, for commercial applications, it is proposed that application server to coordinate with 5G DDNMF to authorize Group Member Discovery parameters to trigger group communication. Furthermore, optionally, delegated provisioning of some group identifiers over PC5 (via a group lead UE) is proposed to be supported in line with solution #4.

# 2. Text Proposal

It is proposed to update the following text within the TR23.752.

*** Start of the change (all new texts) ***

## 6.x	Solution #X: ProSe group discovery

### 6.x.1	Description

This solution addresses Key Issue #1 (ProSe Direct discovery) to support group communication over PC5 reference point.

### 6.x.2	Procedures

#### 6.x.2.1 ProSe group discovery

Figure 6.x.2.1-1: ProSe group discovery

1. Group management is carried out at application layer.

For public safety applications, the group identifiers including an application group ID are pre-provisioned to the group member UEs (e.g. in ME from PCF or configured in UICC as also defined in TS 23.303 [9] clause 5.3.7).

For commercial applications in addition to the above, dynamic group identifier provisioning can be handled by the 5G ProSe Application Server. Application Server may also assign a group lead.

NOTE: 	Application Server may interact with 5G DDNMF (if supported) to authorize Group Member Discovery parameters.

2. ProSe Application layer may provide group identifiers received in step 1 to ProSe Layer.

3. Each UE may run a ProSe direct group discovery that is restricted only to users sharing similar group identifiers (e.g. an application group ID) based on step 2.

4. Each UE self-assigns a source Layer-2 ID and determines a destination Layer-2 ID based on group identifiers received in step 2 from ProSe Application Layer. Each member UE determines the PC5 QoS parameters for this groupcast. The ProSe layer of each member UE passes groupcast related parameters to Access Stratum (TS 23.287 [5]).

5. ProSe Service for groupcast is established.

#### 6.x.2.2 Delegated group information update

Figure 6.x.2.2-1: Delegated Group Information Update

Application Server may indicate to UEs to use delegated group information update. When delegated group information update is supported, the source Layer-2 ID is self-selected by each member UE intending to use delegated group information update. A specific Destination Layer-2 ID for sharing delegated group identifier information is determined based on a mapping provisioned by e.g. application server (when UE joins the group). This is different from the destination Layer-2 ID used for groupcast communication. A message type field within group information update message indicates that the payload is related to delegated group information update to distinguish it from other direct discovery messages. This is to be identified at the ProSe application layer of the UE. The message type field can be transparent to Access Stratum.

1. Group Communication is established and ongoing based on clause 6.x.2.1. UE-4 (not shown in the figure) plans to join the group. UE-1 is assigned as the group lead by the Application Server.

2. UE-3 is notified by Application Server to use delegated group information update. UE-3 may groupcast a discovery message to reach the group lead, i.e. UE-1. UE-3 may also periodically listen for delegated group information update over PC5 on specific Destination layer 2 ID (mapped based on parameters already provisioned by Application Server  when UE-3 joined the group).

3. Application Server may provide group identifier update for the new UE (UE-4) joining the group (e.g. over PC1).  It may also indicate to the group lead to share this as a Delegated Group information Update.

4. 	UE-1 as the group lead UE provides group information update over PC5 to be collected by UE-3.

### 6.x.3	Impacts on services, entities and interfaces

UE:

-	Performs direct discovery for group communication.

-	(a group lead UE), optionally provides group identifiers update to other member UEs by using PC5 discovery messages (based on Application Server request when delegated group information update is supported).

Application Server (Commercial Applications):

-	Performs group management.

-	Provides group identifiers to group members (e.g. over PC1 or optionally delegated via the group lead UE).

-  Optionally Assigns a group lead.

### *** End of the change ***