3GPP TSG SA WG2 Meeting #140-e		S2-200XXXX

Electronic, 19 Aug – 02 Sep, 2020

Title:		[Draft] LS on Use Cases for providing IMS services to SNPN

Reply to:	-

Release:	Rel-17

Work Item:	FS_eNPN

Source:	SA2

To:		SA1

Cc:		-

Contact person:	George Foti

George.Foti@ericsson.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	None

# 1	Overall description

As part of the study on enhanced support of non-public networks (FS_eNPN), SA2 is working on Key Issue #3 (Support of IMS voice and emergency services for SNPN) as defined in TR 23.700-07 clause 5.3.

SA2 has reached interim conclusion as defined in clause 8 for the specific key issue that is covering the scenario where IMS is provided by the SNPN and emergency calls are also provided by said SNPN either to subscribers of this SNPN or UEs in limited service state.  For this use case, there are still some variants based based on existing/deployed/specified IMS authentication schemes that have proposed solutions but not agreed to; some examples:

-	The IMS network relies on SNPN authentication.

-	The IMS network reuses SNPN identifiers (a la USIM).

There are additional use cases that have been proposed corresponding solutions where

the SNPN can have an SLA agreement with a third party (different Administrative Domain) IMS provider to provide IMS services. There are also several variants here; some examples:

-	All IMS nodes, including HSS is maintained in the IMS provider domain.

-	Some IMS nodes can be maintained by SNPN (e.g. HSS, P-CSCF, etc.).

In another discussed use case with proposed solution,

the SNPN acts as a serving network to subscribers of another SNPN (“home SNPN”) that also provides the IMS service. The serving SNPN in this case can have an SLA with the (“home SNPN”) so that when such an SNPN gets selected by subscribers of this (“home SNPN”) they can be authenticated and provided with connectivity to acquire IMS services.

Several solutions have been documented, so far, in the TR 23.700-07, addressing most of the above use cases.

There is no agreement in SA2 regarding which use cases should be supported in Rel-17.

SA2 would like SA1 to review these use cases and provide guidance to SA2 regarding the use cases and their applicability for Rel-17 and potentially in future releases.

# 2	Actions

To SA1

ACTION: 	SA2 kindly asks SA1 the following:

- 	Iindicate whether there are service requirement where the SNPN can have an SLA agreement with a third party (different Administrative Domain) IMS provider to provide IMS services.

-	 For the case where the SNPN provides the IMS network, if there is need to consider additional variants regarding the IMS authentication/IMS identifiers similar to what exists today.

# 3	Dates of next TSG SA WG2 meetings

SA2#141E		12-23 October				Electronic meeting

SA2#142E		16-20 November			Electronic meeting