SA WG2 Meeting #140e	S2-2004860

Electronic Meeting, August 19 – September 2, 2020

Source:	Convida Wireless LLC, AT&T, Samsung

Title:	KI #1, Evaluation: Partial Evaluation for Discovery of Edge Application Server

Document for:	Approval

Agenda Item:	8.5

Work Item / Release:	FS_enh_EC / Rel-17

Abstract of the contribution: This paper proposes a partial evaluation for Key Issue #1: Discovery of Edge Application Server.

# 1	Discussion

This p-CR adds text to the Key Issue #1 evaluation section. The evaluation only focuses on solutions that deal with scenarios where applications are aware of Edge Computing. Thus, the added text only provides evaluation comments for solution 16 and 28.

The authors of this paper also propose that SA2 send an LS to SA6 inviting them to comment on Solution 16. A proposed draft of the LS can be sound in S2-2004862

# 2	Proposal

This contribution proposes to implement the following updates to TR 23.748 v0.4.0.

* * * Start of Change * * *

## 7.x	Evaluation for Key Issue #1: Discovery of Edge Application Server

### Solution #16 deals with the case where the UE hosts an Edge Enabler Client and Application Clients as described by SA6 in TS 23.558 [12]. As discussed in Solution #16, SA6 has introduced functional entities called Edge Configuration Servers and Edge Enabler Servers (EES) which can be used by UE Applications to discover Edge Application Servers. SA6 has agreed that ECS address information can be provisioned by MNO through a 5GC procedure and that such 5GC procedure is in SA2 scope. Solution #16 describes how UE Applications can discover contact information for Edge Configuration Servers. Thus, when paired with the features specified by SA6 in TS 23.558 [12], Solution #16 provides a complete solution for how a UE can “discover a suitable Edge Application Server to serve the application/UE”.

Editor’s Note: Solution 16 should be sent to SA WG6 and SA WG6 should be invited to comment. SA WG2 should consider SA WG6 input during evaluation of the solution.

# 9	Conclusions

Editor's Note: Whether Solution #16 "Edge Configuration Server Based Discovery " will be proceeded in normative work will be re-evaluated after SA2 receives feedback from SA6.