SA WG2 Meeting #140E (e-meeting)	S2-2004823r22

Elbonia, 19 August-1 September 2020	(was S2-200xxxx)

Source:	Qualcomm Incorporated, Ericsson, Nokia, Nokia Shanghai Bell

Title:	KI#4, Interim Conclusions

Document for:	Discussion/Approval

Agenda Item:	8.2

Work Item / Release:	FS_eNPN/Rel.17

Abstract of the contribution: Analyses the similarities and difference identified in the solutions documented in TR 23.700-07 for Key Issue #4 and proposes interim conclusions.

- Conclusion of KI#4

This paper documents the principles for KI#4 conclusion.

Primary authentication using

Default UE credentials in O-SNPN

Primary authentication using

PLMN credentials in PLMN ON

No primary authentication

User plane protocols

Using control plane

procedure e.g.

UPU, UCU

UE Onboarding (component 1)

Remote Provisioning (component 2)

Primary authentication using

PLMN credentials

User plane protocols

Using Control

Plane procedure

e.g. UPU, UCU

UE Onboarding (component 1)



N/A

Remote Provisioning (component 2)

- Conclusion

It is proposed to document the following text in TR 23.700-07.

>>>>START CHANGES<<<<

Primary authentication using

Default UE credentials in O-SNPN

Primary authentication using

PLMN credentials

No primary authentication with O-SNPN

User plane protocols

Using control plane

procedure e.g.

UPU, UCU

Onboarding (component 1)

Provisioning (component 2)

8	Conclusions

8.X	Key Issue #4: UE onboarding and remote provisioning

Editor's note:	These are *INTERIM* conclusions for Key issue #4

SNPN UE onboarding (Component 1 of KI#4)

[…]

Remote provisioning for SNPN credentials (Component 2 of KI#4)

-	Usage of a PLMN as Onboarding Network for a UE equipped with a USIM shall be possible. The SNPN credentials can be transmitted to UE via UP connectivity that is established. The UE shall be configured with Default credentials in USIM to register with any PLMN where the UE can register with the Default credentials in order to communicate with the provisioning server.

-	When user plane is used for UE Onboarding (as in component 1), User plane remote provisioning protocol used for provisioning of SO-SNPN credentials. How the UE downloads the SNPN credential from the Provisioning Server (PS) after PDU session establishment in the O-SNPN is out of scope of SA2 and uses industry developed mechanisms;

-	Control plane remote provisioning based on UE Parameters Update Procedure as defined in TS 23.502 is used for provisioning of SO-SNPN credentials;

Editor’s note: SA3 feedback will need to be taken into account for including of the CP based provisioning.

Editor’s note: It is FFS whether the UE Parameters Update container can be delivered to the UE during the registration procedure.

-	For the provisioning of IMSI accompanied by AKA credentials, GSMA RSP is used, Provisioning Server (PS) can provision the credential to UE over User Plane (UP) connectivity;

-	For the provisioning of Non-3GPP credentials, the credentials can be provided to UE over UP connectivity;

-	It shall be possible to pre-configure the Provisioning Server (PS) address, neither or both  and DNN/NSSAI used to access PS on the UE and it shall also be possible that the O-SNPN provides the PS address and DNN/NSSAI used to access PS to the UE after successful authentication and authorization. The PS address and DNN/NSSAI used to access PS from the O-SNPN shall be integrity protected. The PS address and DNN/NSSAI used to access PS provided by the network is prioritized, if configured and overrides any PS address,and/or DNN/NSSAI used to access PS stored in the UE. Configuration of PS address and DNN/NSSAI used to access PS from the onboarding configuration data to the UE can be supported using one of the following methods:

  - SMF may deliver onboarding configuration data as part of extended Protocol Configuration Options (PCO) in PDU Session Establishment Response to UE. This is similar to use of PCO to configure Autoconfiguration server for UE in Wireless and Wireline Convergence (TR 23.716 [28] clause 6.10).

  - Alternatively, onboarding configuration data may be configured in the UE during Registration Procedure using UE Route Selection Policy (URSP) that may be subject UE capabilities. As part of UE initial registration (based on received UE capability information) AMF indicates to PCF that UE has requested restricted/provisioning registration. The PCF may initiate UE Policy delivery using UE Route Selection Policies (URSP), for instance to trigger UE after successful registration to request establishment of specific type of PDU Session limited to onboarding purposes only.

  - In addition, onboarding configuration data may be configured in the UE using service specific policies subject to UE capabilities similar to what is used for V2X communications as specified in TS 23.287 [29] clause 5.1.1 for ways how parameters may be made available to the UE and TS 23.287 [29] clause 6.2.5 for AF-based service parameter provisioning and TS 24.587 [30] clause 5.2.4 for configuration parameters such as validity timer, server address and geographical area.

Editor’s Note: Whether SO-SNPN identity is needed to access PS is FFS.

-	It is assumed that the UDM (or AAA) of SO-SNPN is is provisioned with UE credentials/subscription data when remote provisionging is successfully performed.

Editor’s Note: Whether the specific procedure for  provisioning of UDM (or AAA) with the UE credentials/subscription data is in scope of 3GPP is FFS.

-	Upon successful establishment of restricted access PDU session, if the UE still does not have a PS address, the device uses a well-known FQDN to perform PS discovery.

NOTE: SA3 may evaluate these mechanisms and provide guidance on appropriateness of use for SNPNs.

UE

Onboarding for PNI-NPN (Component 1 of KI#4)

[…]

Remote provisioning for PNI-NPN credentials (Component 2 of KI#4)

-	At least network initiated remote provisioning of credentials to allow access to PNI-NPN services should be supported in Rel-17;

-	Both procedures using Control Plane and using User Plane protocols after establishing PDU session shall be enabled for remote provisioning the PNI-NPN credentials used for NSSAA and/or PDU Session secondary authentication;

Editor’s Note: SA3 feedback for the suitability of the procedure will need to be taken into account

Editor's note:	whether an extra security layer for protection of credentials between PS and UE is needed should be decided by SA3.

-	For User Plane remote provisioning:

-	The protocol for provisioning of PNI-NPN credentials used for NSSAA and/or PDU Session secondary authentication, i.e. how the UE download the NPN credential from the PS after PDU session establishment in PNI-NPN, is out of scope of SA2;

-	The PS address and DNN/NSSAI used to access PSmay be provided to the UE during or after the Registration procedure.

Editor's Note: How the PS address is provided to the UE is FFS.

Editor’s note: The vertical may verify the UE before PNI-NPN credential is provisioned to UE, and howthis is done should be decided by SA3.

- Upon successful remote provisioning of the UE, the UE Subscription Data in the UDM/UDR may be updated to enable the access to the PNI-NPN.

Editor's note:	for PNI-NPN credentials remote provisioning, whether the 3GPP operator could decide to update the UE Subscription Data (e.g., S-NSSAI, DNN, CAG information) in the UDM/UDR used to access to the PNI-NPN based on the input from the vertical which may be outside 3GPP operator domain should be decided by SA3.

>>>>END OF CHANGES<<<<