SA WG2 Meeting #140e	S2-2004841

# Aug 19th – Sept 1st, 2020 ; Elbonia        		                					(revision of S2-2004841)

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 2385 | rev | - | Current version: | 16.5.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Support of the mapping from IP addressing information provided to an AF to the user identity |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S2 |  |  |  |  |  |  |  |  |  |
| Work item code: | EDGEAPP |  |  |  |  |  | Date: |  |  | 2020-08-04 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | SA6 LS (S2-2004784 = S6-200947) on IP address to GPSI translation asking: - SA6 Question: Does the core network provide an API to translate a UE's IP address (private and public) to its GPSI, and if not, would it be feasible to provide such functionality in Rel-17 in order to address the SA6 requirement for UE IP address translation? - SA6 Question: is it feasible to provide application-specific GPSIs, to ensure that a single GPSI can not be used to track an end user's activity across applications (EASs), to protect end user privacy? |  |  |  |  |  |  |  |  |
| Summary of change: |  | An AF may only be able to identify an UE target of an AF request for data provisioning or for Event exposure (information retrieval about a UE) by providing IP addressing information (IP address and port information) about this UE. In this case the 5GC first needs to retrieve the Permanent identifier of the UE based on such IP addressing information (IP address and port information) The IP addressing information (IP address and port information) of the UE that is provided by the AF may correspond to an UE IP address as allocated by 5GC or to an IP address that has been NATed (Network and Port Address Translation) by an entity controlled by the 5GC operator. In the latter case the 5GC needs to first get from the NATF (new 5GC NF that has carried out NAT at the user plane) the translation back from the IP addressing information provided by the AF to the IP address that the 5GC has allocated to the UE; The 5GC may provide a GPSI value to an AF. The GPSI value provided to the AF does not allow to track an end user's activity across applications (EASs). This is to protect end user privacy. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The SA6 requirements are not supported. Not possible to request data provisioning or data retrieval about an individual UE that the application can only identify by its IP addressing information |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.20 ; 6.2.X (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | X |  | Other core specifications |  |  |  | 23.502 CR2329 |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | This CR is only provided for technical endorsement. No creation of R17 version of the specification is meant |  |  |  |  |  |  |  |  |

FIRST CHANGE

## 5.20	External Exposure of Network Capability

The Network Exposure Function (NEF) supports external exposure of capabilities of network functions. External exposure can be categorized as Monitoring capability, Provisioning capability, Policy/Charging capability and Analytics reporting capability. The Monitoring capability is for monitoring of specific event for UE in 5G System and making such monitoring events information available for external exposure via the NEF. The Provisioning capability is for allowing external party to provision of information which can be used for the UE in 5G System. The Policy/Charging capability is for handling QoS and charging policy for the UE based on the request from external party. The Analytics reporting capability is for allowing an external party to fetch or subscribe/unsubscribe to analytics information generated by 5G System.

Monitoring capability is comprised of means that allow the identification of the 5G network function suitable for configuring the specific monitoring events, detect the monitoring event, and report the monitoring event to the authorised external party. Monitoring capability can be used for exposing UE's mobility management context such as UE location, reachability, roaming status, and loss of connectivity. AMF stores URRP-AMF information in the MM context to determine the NFs that are authorised to receive direct notifications from the AMF. UDM stores URRP-AMF information locally to determine authorised monitoring requests when forwarding indirect notifications.

Provisioning capability allows an external party to provision the Expected UE Behaviour or the 5GLAN group information or service specific information to 5G NF via the NEF. The provisioning comprises of the authorisation of the provisioning external third party, receiving the provisioned external information via the NEF, storing the information, and distributing that information among those NFs that use it. The externally provisioned data can be consumed by different NFs, depending on the data. In the case of provisioning the Expected UE Behaviour, the externally provisioned information which is defined as the Expected UE Behaviour parameters in TS 23.502 [3] clause 4.15.6.3 or Network Control parameter TS 23.502 [3] clause 4.15.6.3a consists of information on expected UE movement, Expected UE Behaviour parameters or expected Network Configuration parameter. The provisioned Expected UE Behaviour parameters may be used for the setting of mobility management or session management parameters of the UE. In the case of provisioning the 5GLAN group information the externally provisioned information is defined as the 5GLAN group parameters in TS 23.502 [3] clause 4.15.6.7, and it consists of some information on the 5GLAN group. The affected NFs are informed via the subscriber data update as specified in TS 23.502 [3] clause 4.15.6.2. The externally provisioned information which is defined as the Service Parameters in clause 4.15.6.7 of TS 23.502 [3] consists of service specific information used for supporting the specific service in 5G system. The provisioned Service Parameters may be delivered to the UEs. The affected NFs are informed of the data update.

Policy/Charging capability is comprised of means that allow the request for session and charging policy, enforce QoS policy, and apply accounting functionality. It can be used for specific QoS/priority handling for the session of the UE, and for setting applicable charging party or charging rate.

Analytics reporting capability is comprised of means that allow discovery of type of analytics that can be consumed by external party, the request for consumption of analytics information generated by NWDAF.

An NEF may support CAPIF functions for external exposure as specified in clause 6.2.5.1.

An NEF may support exposure of NWDAF analytics as specified in TS 23.288 [86].

An AF may only be able to identify the UE target of an AF request for Data Provisioning or for Event Exposure about a UE by providing IP addressing information (IP address and port information) about this UE. In this case the 5GC first needs to retrieve the Permanent identifier of the UE before trying to fulfil the AF request.  The 5GC may to retrieve the Permanent identifier of the UE based on:

- the IP addressing information (IP address and port information) of the UE as provided by the AF: this may correspond to an UE IP address as allocated by 5GC or to an IP address (and port) that has been NATed (Network and Port Address Translation) by an entity controlled by the 5GC operator. In the latter case the 5GC (NEF) needs to first get from the NATF (5GC NF that has carried out NAT at the user plane) the translation back from the IP addressing information provided by the AF to the IP address that the 5GC has allocated to the UE.

- the corresponding DNN and/or S-NSSAI information: this may have been provided by the AF or determined by the NEF using the identity of the AF;

NOTE:	When the AF has been triggered by an application running on an Edge Application Server that only identifies an UE by its IP addressing information, the AF is only be able to identify the UE by providing IP addressing information for this UE;

The 5GC (NEF) may provide a GPSI value to an AF

- that has explicitly requested a translation from IP addressing information (IP address and port information) of the UE to a GPSI or

- that has implicitly requested a translation from IP addressing information of the UE to a GPSI by requesting data provisioning or information retrieval about an individual UE identified by IP addressing information

NEXT CHANGE (2)

# 6	Network Functions

### 6.2.X	NATF

The NATF NF supports:

- NA(P)T (Network Address and Port Translation) in the user plane,

NOTE:	How to steer NA(P)T functionality is not specified in this release of the specification. See 5.6.10.1.

- A service providing the reverse translation from the UE IP address and port provided as input in the service request into the corresponding IP address as allocated by the 5GC to the UE

The NATF registers in the NRF the (public) IP address ranges it owns and its reverse translation service may be discovered by using a NRF lookup about a public IP address.

NEXT CHANGE (3)

### 5.6.10	Specific aspects of different PDU Session types

#### 5.6.10.1	Support of IP PDU Session type

The IP address allocation is defined in clause 5.8.1

The UE may acquire following configuration information from the SMF, during the lifetime of a PDU Session:

-	Address(es) of P-CSCF(s);

-	Address(es) of DNS server(s).

NOTE 1:	When DNS over (D)TLS is supported by the network, the configuration information sent by the SMF may also include the corresponding DNS server security information as specified in TS 33.501 [29].

-	the GPSI of the UE.

The UE may acquire from the SMF, at PDU Session Establishment, the MTU that the UE shall consider, see clause 5.6.10.4.

The UE may provide following information to the SMF during the lifetime of a PDU Session:

-	an indication of the support of P-CSCF re-selection based on procedures specified in TS 24.229 [62] (clauses B.2.2.1C and L.2.2.1C).

-	PS data off status of the UE.

NOTE 2:	An operator can deploy NA(P)T functionality in the network; how to steer NA(P)T functionality is not specified in this release of the specification.

NEXT CHANGE (4)

NEXT CHANGE (5)

END OF CHANGES