SA WG2 Meeting #140e (e-meeting)	S2-2004854r02

(Revision of S2-200xxxx)

19th August - 2nd September 2020

Source:	China Telecom, NEC

Title:	Solution update for Solution#3: User consent for UE data collection

Document for:	Approval

Agenda Item:	8.1

Work Item / Release:	FS_eNA_ph2 / Rel-17

Abstract of the contribution: This contribution proposes a solution update for solution#3 User consent for UE data collection.

# 1. Introduction

This solution update is proposed to address FFS in solution#3.

Before collecting UE data, a user consent may be required for UEs based analytics information. Solution#3 provides the User consent for the UE data collection; thus, AF could provide the user consent to UDM via NEF.

# 2. Discussion

Solution#3 uses Nnef_ParameterProvision_Update Request service to provide the information via the NEF in the user consent for UE data collection. Then, NEF checks AF authorization. Since the AF is authorized for user consent provision or update, it is concluded that the Service Provider (AF) is trusted. Because the AF is confidential, the “Editor's note: How the user consent is validated is FFS” could be removed.

Step 6 in procedures in the solution#3 shows that AMF needs to retrieve the user consent for UE data collection before triggering some UE data specific data collection. Considering that a separate retrieval of the user consent by AMF produces a larger signalling load, this step can be covered via regular AMF - UDM communication during UE registration. Therefore, the “Editor's note: Whether step 6 is needed or can be covered via regular AMF - UDM communication during UE registration is FFS” could be removed.

Editor's note: ‘The retrieval of user consent by the AMF as well as collection of data based on such consent is FFS and subject to KI#8 conclusion on methods for data collection from the UE’ - This solution is actually about UE related data collection by NWDAF and/or by the AMF from the UE, from the AF or from other NFs, i.e. not just directly from the UE. So, this solution should not depend from the KI#8 conclusion for data collection from the UE directly as the User consent may be needed for UE related data collection from the AF or from other NFs. That is it is proposed to remove this Editor’s Note.

# 3. Proposal

According to the above discussion, we proposed to:

- Proposal 1: remove the “Editor's note: How the user consent is validated is FFS”;

- Proposal 2: remove the “Editor's note: Whether step 6 is needed or can be covered via regular AMF - UDM communication during UE registration is FFS”.

- Proposal 3: remove the Editor's note: The retrieval of user consent by the AMF as well as collection of data based on such consent is FFS and subject to KI#8 conclusion on methods for data collection from the UE.’

First change

### 6.3.1	Description

#### 6.3.1.1	General

To satisfy the requirement in Key Issue #15: User consent for UE data collection/analytics for privacy of some of the UE data, a User consent may be required for some or all the UE information. It is network operator's responsibility to collect and manage the User consent, if required, before initiating UE data collection and reporting by the UE. The user consent information can be considered as part of the subscription data.

Editor's note:	Coordination with SA3 is required.

#### 6.3.1.2	Procedure

The provision of the User consent for the UE data collection is shown in Figure 6.3.1.2-1 below.

Figure 6.3.1.2-1: User consent provision for UE data collection

1).	Nnef_ParameterProvision_Update Request(UE_ext_Id, user consent for UE data collection) - The Service Provider (AF) may collect the user consent for UE data collection (if applicable) from its users and provide the information via the NEF in the user consent for UE data collection parameter along with the UE Id for which the user consent applies. The Service Provider may also update or revocate the user consent.

2).	AF authorisation - The NEF checks whether the AF is authorized for user consent provision or update.

3).	The NEF may inquire with the UDM to translate UE ext_Id to a 3GPP internal identity UE_Id. Then the NEF forwards the request from the AF to the UDM via Nudm_ParameterProvision_Update Request (UE_Id, user consent for UE data collection).

4).	The user consent for UE data collection parameter is stored in the UDM as a subscription information. The user consent for UE data collection parameter may define the user consent for UE level data collection from UE or application or network side. The user consent for UE data collection parameter may also be provisioned by the network operator.

Alternatively, the user consent for UE data collection parameter may be configured in the UDM/UDR as a user subscription information.

5).	Nudm_SDM_Get (UE_Id, user consent for UE data collection) - The NWDAF may retrieve the user consent for UE data collection before triggering some UE data specific analytics and data collection.

6).	Nudm_SDM_Get (UE_Id, user consent for UE data collection) - The AMF may retrieve the user consent for UE data collection before triggering some UE data specific data collection.

NOTE:	This step can be covered via regular AMF - UDM communication during UE registration.

### 6.3.2	Impacts on services, entities and interfaces

UDM:

-	Holds the user consent for UE data collection as a subscription data.

NEF:

-	AF authorisation and rejection if needed.

NWDAF/AMF:

-	Retrieves and checks the user consent for UE data collection before triggering some UE data specific analytics and data collection.

End of change