3GPP TSG-SA WG2 Meeting #140e	S2-2005039

# E-Meeting, Aug 19th – 02nd Sept 2020

Title:	LS Reply on Bulk operation of LCS-service

Response to:	LS (C4-203629) on Bulk operation of LCS-service

Release:	Release 16

Work Item:	5G_eLCS

Source:	SA2

To:	CT4

Cc:	CT3

Contact person:	Pallab Gupta

pallab.gupta@nokia.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	None

# 1	Overall description

SA2 thanks CT4 for the LS on bulk operation of LCS service.

SA2 would like to kindly give following responses to CT4 questions

- When using Service Based Interface (SBI), which node is expected to be the aggregation point of Bulk Operation of LCS Service, i.e. the node who resolves the group identifier to member UEs and aggregate the location requests and responses for each UE in the group, NEF or GMLC or both?

[Answer] GMLC is the aggregation point of Bulk Operation of LCS service request/response for LCS client and Nnef_EventExposure_Subscribe request/response. When the request arrives at GMLC, it may check with UDM if a mapping is available for the group ID. If a mapping is not available, the GMLC rejects the location request and include the appropriate error code in the response. If group ID mapping is available, the GMLC sends an acknowledgement in the response. After that, the UE location estimation event notification is sent on a per UE basis, and the GMLC is not responsible for aggregating the UE location estimation for the group of UE

- If the GMLC is expected to be the aggregation point of Bulk Operation of LCS Service, what should the GMLC respond for a request of bulk operation, especially for 5GC_MT_LR procedure?

NOTE:	CT4 observed that if NEF sends a 5GC-MT-LR request towards GMLC for group of UEs, there is no way for GMLC to relay the multiple responses (one per UE) to the NEF, using current service operations specified in TS 23.273 (Only one service response is allowed per service request in SBI and location for one UE is allowed in Ngmlc_Location_ProvideLocation response).

[Answer] See answer for question 1 above.

- If used as aggregation point, when does the GMLC / NEF acknowledge the request for Bulk Operation, i.e. does it respond the LCS client / AF after it received all positioning responses from network side for all UEs in the group or respond without AMF checking?

[Answer] See answer for question 1 above.

- If GMLC / NEF responds the LCS client / AF immediately, based on what criterion does the GMLC / NEF decide to respond success or failure or partial success? And How should GMLC / NEF to notify the LCS Client / AF if the requests for positioning for some UEs fail because of some different reasons (e.g. The UE is not online) after GMLC / NEF responded success to LCS client / AF?

[Answer] See answer for question 1 above.

# 2	Actions

To CT4

ACTION: 	SA2 kindly asks CT4 to take the above information into account and update their specification.

# 3	Dates of next TSG SA WG 2 meetings

3GPP TSG SA2 #141e	TBD	E-Meeting