3GPP TSG-WG SA2 Meeting #140E e-meeting 	S2-2005071r01

Elbonia, August 19 – September 1, 2020	(revision of S2-200xxxx)

Source:	Huawei, HiSilicon

Title:	Update to Solution 22 to resolve the EN

Document for:	Approval

Agenda Item:	8.4

Work Item / Release:	FS_eNS_Ph2 / Rel-17

Abstract: This contribution proposes to remove the EN of Solution 22.

# 1. Introduction/Discussion

There is an EN in Solution 22:

Editor's note:	Whether the interaction between AMF and PCF is necessary is FFS.

In the roaming case, the VPLMN may want to adjust the maximum bitrate based on roaming agreement, e.g. downgrade the maximum bitrate. Then, the PCF should authorize the subscribed maximum bitrate from HPLMN. As described in TS 23.503, UE AMBR is a parameter within AM Policy information:

Table 6.5-1: Access and mobility related policy control information

| Information name | Description | Category | PCF permitted to modify in a UE context in the AMF | Scope |
| --- | --- | --- | --- | --- |
| UE-AMBR | This defines the UE-AMBR value that applies for a UE | Conditional (NOTE 5) | Yes | UE context |
| Service Area Restrictions | This part defines the service area restrictions |  |  |  |
| List of allowed TAIs. | List of allowed TAIs (NOTE 3) (NOTE 4). | Conditional (NOTE 1) | Yes | UE context |
| List of non-allowed TAIs. | List of non-allowed TAIs (NOTE 3). | Conditional (NOTE 1) | Yes | UE context |
| Maximum number of allowed TAIs | The maximum number of allowed TAIs. (NOTE 4) | Conditional (NOTE 1) | Yes | UE context |
| RFSP Index | This part defines the RFSP index |  |  |  |
| RFSP Index | Defines the RFSP Index that applies for a UE | Conditional (NOTE 2) | Yes | UE context |
| SMF selection management | This part defines the SMF selection management instructions |  |  |  |
| DNN replacement of unsupported DNNs | Defines if a UE requested unsupported DNN is requested for replacement by PCF | Conditional (NOTE 6) | Yes | UE context |
| List of S-NSSAIs | Defines the list of S-NSSAIs containing DNN candidates for replacement by PCF | Conditional (NOTE 6) (NOTE 7) | Yes | UE context |
| Per S-NSSAI: List of DNNs | Defines UE requested DNN candidates for replacement by PCF | Conditional (NOTE 6) | Yes | UE context |
| NOTE 1: If service area restrictions is enabled. NOTE 2: If RFSP index is enabled. NOTE 3: Either the list of allowed TAIs or the list of non-allowed TAIs are provided by the PCF. NOTE 4: Both the maximum number of allowed TAIs and the list of allowed TAIs may be sent by PCF. NOTE 5: If UE-AMBR is enabled. NOTE 6: If SMF selection management by PCF is enabled. NOTE 7: The List of S-NSSAIs contains S-NSSAIs, valid in the serving network, of the Allowed NSSAI. |  |  |  |  |

There is similar requirement for SMBR. For example, the VPLMN may downgrade the SMBR for roaming users based on roaming agreement. Then, the VPLMN should also be able to update the Subscribed SMBR following the above method.

# 2. Text Proposal

It is proposed to capture the following changes vs. TR 23.700-40.

# * * * * First change * * * *

#### 6.22.3.1	Registration

UE sends Initial Registration Request as described in step 1-3 of clause 4.2.2.2.2 of TS 23.502 [6].

The detail flows are shown in Figure 6.22.3-1.

Figure 6.22.3-1: Procedures for Slice-AMBR during registration

Step 1 and 2. The AMF retrieves subscribed SMBR per S-NSSAI from UDM.

Step 3. Tthe AMF sends the Subscribed SMBR per S-NSSAI obtained from the UDM to (R)AN.

# * * * * Second change * * * *

### 6.22.4	Impacts on services, entities and interfaces

AMF:

-	Support Subscribed SMBR transfer during initial registration via Nudm_SDM Service.

-	Support SMBR transfer via N2 interface and SMBR enforcement.

UDM:

-	Support Subscribed SMBR transfer during initial registration via Nudm_SDM Service.

RAN:

-	Support SMBR transfer via N2 interface and SMBR enforcement.

# * * * * End of changes * * * *