3GPP TSG-SA WG2 Meeting #140E (e-meeting)	S2-2005091r01

Aug 19 – Sep 01, 2020, Elbonia

Source:	NEC

Title:	KI#7 Sol#31: Updates to Solution #31 Steering the UE to a network slice in a different frequency band

Document for:	Approval

Agenda Item:	8.4

Work Item / Release:	FS_eNS_Ph2 / Rel-17

Abstract: This contribution proposes updates and a new evaluation section to Solution #31 Steering the UE to a network slice in a different frequency band.

# 1. Background

This contribution proposes updates to resolve Editor’s Notes and also adds a new solution evaluation section. The following updates in TR23.700-40 v0.4.0

******* Start of the change ********

### 6.31	Solution #31: Steering the UE to a network slice in a different frequency band

### 6.31.1	Introduction

This solution addresses the below requirements from Key Issue #7: Support of 5GC assisted cell selection to access network slice.

-	How does 5GS steer UEs to a 5G-AN (e.g. a specific frequency band) that can support the network slices that the UE can use.

-	What information does 5GS need to take a decision to steer UE to a proper 5G-AN.

### 6.31.2	High Level Description

The solution is based on the following high level assumptions and principles:

-	The UE is allocated Allowed NSSAI which can contain S-NSSAIs supported in different frequency bands, however all S-NSSAIs are supported in all Tracking Areas or the Registration Area.

### 6.31.3	Procedures

Figure 6.31.3.1-1 - Steering the UE to a network slice in different frequency band

1)	The UE is in idle mode, registered via RAN-1 for S-NSSAI-1, which operates only in frequency band 1 (FB-1) and S-NSSAI-2, which operates only in frequency band 2 (FB-2).

2)	An application in the UE needs to establish service on S-NSSAI-2 in FB-2.

3)	The UE establishes RRC connection with RAN-1.

4)	The UE triggers PDU Session Establishment Request on S-NSSAI-2 via RAN-1.

5)	AMF is aware that RAN-1, via which the UE is connected, does not support S-NSSAI-2 and the AMF is aware that S-NSSAI-2 is supported by another RAN Node.

6) AMF requests RAN-1 to steer the UE to a RAN Node supporting S-NSSAI-2.

7)	RAN-1 triggers inter-frequency cell change to RAN-2 which supports S-NSSAI-2 and is in UE's location.

If the UE has other active PDU Session(s) that can not be transferred to the target RAN-2, the UE stays on RAN-1 and RAN-1 triggers the Dual Connectivity configuration with RAN-2 in order to establish the new PDU Session associated with S-NSSAI-2 on RAN-2 while the PDU Session(s) associated with S-NSSAI-1 stay on RAN-1.

NOTE:	It is for RAN Working Groups to define the inter-frequency cell change in connected mode e.g. Handover, Cell Change Order, RRC release with re-direction or RRC Connection reconfiguration.

8)	AMF continues with the PDU Session establishment procedure on S-NSSAI-2 via RAN-2.

9)	After PDU session on S-NSSAI-2 is released, the UE falls-back (e.g. re-selects back) to a cell in FB-1. This step is optional. If not implemented, the UE will stay on FB-2 until it is steered back to FB-1 due to a new service request on FB-2.

### 6.31.4	Impacts on services, entities and interfaces

-	UE: cell fall-back to a cell on the initial frequency band (optional).

-	AMF: decision and request for handover to a cell from another frequency band.

-	RAN: deploys Dual Connectivity for the new PDU Session when there are existing PDU Sessions that can not be transferred to the cell from another frequency band.

### 6.31.5	Evaluation

This solution is very simple and efficient when the PDU Session to be transferred to a cell from another frequency band is the only PDU Session for the UE.

The solution would also work for when the UE has multiple PDU Sessions and all of them can be transferred to a cell from another frequency band (e.g. the cell from the other frequency band supports the network slices on which the UE has active PDU Session(s).

If not all PDU Sessions can be transferred to a cell from another frequency band, the solution still works via RAN switching to Dual Connectivity configuration.

******* End of the change *******