SA WG2 Meeting #S2-140E	S2-2005124

19th August – 2nd September, 2020, Elbonia	(Revision of S2-2003608)

Source: 	Ericsson, NTT DOCOMO

Title: 	KI#2, new solution, policy based control

Document for: 	Approval

Agenda Item: 	8.2

Work Item / Release:	FS_eNPN / Rel-17

Abstract of the contribution: This paper proposes a solution to address the Key Issue #2: NPN support for Video, Imaging and Audio for Professional Applications (VIAPA).

# Discussion

During the discussion at the eNPN conference calls, in relation with KI#2, the case where VIAPA applications are delivered via SNPN, these applications are very demanding in terms of QoS, typically require high 5QI value in a small area for simultaneously access for a large number of users, since the capacity of the SNPN is limited, it will be beneficial to be able to instruct the UE on the subscription to use when the VIAPA application traffic is detected, this allows the UE to select the Uu interface that provides the best performance for the VIAPA application.

In addition, VIAPA ASP have SLA with the PLMN to provide QoS for those UE registered to the concert. In order to do that the PLMN has an SLA with the SNPN to serve e.g. X number of PLMN UEs PDU sessions for VIAPA application. At the time the VIAPA ASP communicates with the PLMN that a number of UEs are registered for the concert, the PLMN determines which UEs, of those attending the concert, will be served by SNPN, this is determined based on  local policies that consider the UE capabilities for SNPN mode support, UE preferences, Analytics on QoS sustainability in that area.

Another option is that the VIAPA ASP have SLA with the SNPN to provide the best QoS performance to VIAPA users, at the time the SLA is agreed, the VIAPA ASP requests enhanced QoS for UEs e.g. attending a music festival for a specific time period (e.g. 2 hours). The SNPN can check if it can provide the expected performance or not, using analytics on QoS sustainability and if not, then check if any of the PLMNs that has an agreement with the SNPN can provide it. The result can be both: to inform the VIAPA ASP that SNPN network took into account the QoS requirements to deliver the application data to the users, and in case the SNPN decides to use an alternative PLMN to route VIAPA traffic, the configuration of the alternative PLMN to use and the Access Type to use.

The following issues are addressed in this revision of this document as raised in the SA2#139E meeting:

- The terminology is clarified, this proposal is applicable for a single radio UE with both a SNPN subscription and at least a PLMN subscription-

- The UE receives an indication, that may be part of the URSP rule or a separate indicator, on the subscription to use when the VIAPA application traffic is detected. The proposal does not impact the PLMN selection procedure that remains unchanged.

- For a UE that has multiple subscriptions and multiple, possibly conflicting, URSP rules for the same application, the UE should use the precedence of the URSP that is already defined in TS 23.503.

# Proposal

Add the following solution to TR 23.700-07.

*** BEGIN CHANGES ***

## 6.0	Mapping Solutions to Key Issues

Table 6.0-1: Mapping of Solutions to Key Issues

|  | Key Issues |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Solutions | 1 | 2 | 3 | 4 | 5 | 6 |
| 1 | X | X |  |  |  |  |
| 2 | X | X |  |  |  |  |
| 3 | X |  |  |  |  |  |
| 4 | X |  |  |  |  |  |
| 5 |  |  |  | X |  |  |
| 6 |  |  |  | X |  |  |
| 7 |  |  |  | X |  |  |
| 8 | X |  |  |  |  |  |
| 9 | X |  |  |  |  |  |
| 10 | X |  |  |  |  |  |
| 11 | X |  |  |  |  |  |
| 12 | X |  |  |  |  |  |
| 13 |  | X |  |  |  |  |
| 14 |  | X |  |  |  |  |
| 15 |  | X |  |  |  |  |
| 16 |  | X |  |  |  |  |
| 17 |  | X |  |  |  |  |
| 18 |  | X |  |  |  |  |
| 19 |  |  | X |  |  |  |
| 20 |  |  | X |  |  |  |
| 21 |  |  | X |  |  |  |
| 22 |  |  | X |  |  |  |
| 23 |  |  | X |  |  |  |
| 24 |  |  | X |  |  |  |
| 25 |  |  | X |  |  |  |
| 26 |  |  | X |  |  |  |
| 27 |  |  |  | X |  |  |
| 28 |  |  |  | X |  |  |
| 29 |  |  |  | X |  |  |
| 30 |  |  |  | X |  |  |
| 31 |  |  |  | X |  |  |
| 32 |  |  |  | X |  |  |
| 33 |  |  |  | X |  |  |
| 34 |  |  |  | X |  |  |
| 35 |  |  |  | X |  |  |
| 36 |  |  |  | X |  |  |
| 37 |  |  |  | X |  |  |
| 38 |  |  |  | X |  |  |
| 39 |  |  |  | X |  |  |
| 40 |  |  |  | X |  |  |
| X |  | x |  |  |  |  |

*** NEXT CHANGES – All new Text***

## 6.X	Solution #X:  Steering a single access UE to use Uu interface of the SNPN or the PLMN

### 6.X.1	Introduction

The solution addresses key issue #2 " NPN support for Video, Imaging and Audio for Professional Applications (VIAPA)". It applies for a single radio UE with both a SNPN subscription and at least a PLMN subscription as exemplify in figure 6.x.1-1:

Figure 6.x.1-1: Example of a single radio capable UE with both a SNPN and multiple PLMN subscriptions

This solution proposes a mechanism to allow the network to steer a single access UE to use a certain Uu interface for VIAPA traffic, either via PLMN or SNPN and then use IPSec tunnel via N3IWF in the other network to send or receive other type of data from the other network.

The selection of the Uu interface to use for VIAPA services is based on operator policies that take into account:

- the SLA between the PLMN and the SNPN indicating that UEs acceding VIAPA application or VIAPA DNN from other NWs can be served.

- User preferences, that may be either stored in the UDR or received from the UE in NAS signalling.

- UE capabilities to support SNPN mode and

- QoS sustainability analytics

Two possible scenarios are shown below:

Figure 6.x.1-1. s VIAPA traffic delivered over PLMN. The UE is connected to PLMN via Uu interface

Figure 6.x.1-2. VIAPA traffic delivered over SNPN. The UE is connected to SNPN via Uu interface

### 6.X.2	Functional Description

#### 6.x.2.1	General

Some assumptions are listed below:

-	The UE is the area which is covered by the NG-RAN of both PLMN(s) and SNPN.

-	Both networks (PLMN and SNPN) can be operated by different operators.

-		The user has separated subscriptions to both PLMN(s) and SNPN. Alternatively, as SNPN is less regulated and for more local service, the UE does not have subscription with the SNPN, but SLA between the SNPN and the PLMN exist to serve X number of PLMN UEs PDU session for VIAPA applications or for VIAPA DNN.

Editor's note: How this solution can work if the UE does not have a subscription with the SNPN is FFS.

-	The SNPN operator can have a business agreement with each of the PLMN operators serving the area where the UE is located.

-	The Data services being provided in this VIAPA KI are very demanding in terms of QoS.

#### 6.x.3.1	High-level Procedures

##### 6.x.3.1.x	General description

The high-level flow describes the scenario where the VIAPA ASP checks with the PLMN the QoS requirements to serve the UEs that bought tickets for the concert.  The PLMN has an SLA with an SNPN. The SNPN may be either less regulated and more a local services where the UE does not have a subscription with the SNPN or the UE may have a subscription with the SNPN. The procedure below aims to depict both options.

Figure 6.27.3.1-1: Checking QoS requirements for VIAPA applications - overview

1-	The following steps takes place at the time the VIAPA ASP, the SNPN and the PLMN prepare to deliver VIAPA services:

- The VIAPA Application Service Provider configures its portal with information on the SNPN to be used for delivering the VIAPA application to their consumers.

- The PLMN configures its network with the list of SNPNs that has an agreement with. For each SNPN, the SLA indicates the number of PLMN UE PDU sessions for VIAPA applications (or for VIAPA DNN) that can be served.

2-	A number of users buy tickets to the concert.

3-	The VIAPA ASP portal informs the PLMN about the number of UEs (and its identities) that bought tickets, the area where the concert will take place, the type of Application that will be delivered and the time of the day.

4-	The PLMN checks QoS requirements then estimates using analytics whether QoS degradation may happen. If PLMN estimates that QoS requirements can be fulfilled for all the VIAPA sessions, then step 6 follows.

5-	If the PLMN estimates that QoS requirements can be fulfilled for a subset of all the VIAPA sessions, then checks the SLA with the PLMN(s) that serve the area where the concert will take place and the users that bought a tickets .  The PLMN determines, based on UE capabilities to support SNPN mode, QoS sustainability analytics, user preferences, which UE PDU sessions for VIAPA applications should be served by the SNPN.

6-	 As result of the above process, the PLMN knows the list of UEs that use PLMN Uu access for VIAPA service and the list of UEs that use SNPN Uu access for VIAPA service. Then, at the time the UE registers to the PLMN where it has a subscription, the network provides UE Policies to indicate whether the SNPN Uu or PLMN Uu interface should be used to transfer the VIAPA traffic, including both the VIAPA traffic identifier (such as IP 3- tuple or DNN) and the indication of the subscription to use for NW selection (if the SNPN does not require to have subscriptions, then the SNPN id is provided to the UE instead).  When the VIAPA application traffic starts matching a the UE Policy, the UE checks  the indication of the NW subscription to be used for NW selection or the SNPN id, then registers to the SNPN.

Editor's note: Using the start of the VIAPA application traffic as a trigger to change the active subscription and to perform network selection will interrupt the VIAPA application traffic. Whether this is acceptable for VIAPA applications is FFS.

As result to the registration to the SNPN, URSP rules are downloaded to indicate that 3GPP Access is used for VIAPA traffic (including the IP 4-tuple or the DNN) . There may be another type of traffic that use IPSec tunnel via N3IWF in the other network to send or receive other type of data from the other network

NOTE: The indication of the subscription to use for PLMN selection may be part of the URSP rule or a separate indication in a new UE Policy.

Editor's note: How to handle scenarios that additional applications are started on the UE that instead require use of the PLMN instead of the SNPN is FFS.

Editor's note: How the UE determines to select the PLMN subscription again is FFS.

Editor's note: Whether this solution requires the UE to always evaluate all URSP rules of all subscriptions (and if this is feasible) or only the URSP rules of the currently active subscription is FFS.

#### 6.x.4.1	How to indicate to the UE which Uu interface to use for VIAPA traffic.

#### Method 1: Extending URSP rules

The RSD in the URSP rule is extended to indicate the subscription to use for PLMN selection as described below:

Table 6.6.2.1-1: Route Selection Descriptor extended with “indication on subscription to use for NW selection”

| Information name | Description | Category | PCF permitted to modify in URSP | Scope |
| --- | --- | --- | --- | --- |
| Route selection components | This part defines the route selection components | Mandatory |  |  |
| Access Type preference | Indicates the preferred Access Type (3GPP or non-3GPP or Multi-Access) when the UE establishes a PDU Session for the matching application. The preferred access type may include a reference to the NW subscription to be used for the matching application. | Optional | Yes | UE context |

Table 6.6.2.1-2:  SNPN URSP rules to route VIAPA App1 using the PLMN_1 subscription

| Example SNPN URSP rules | Comments |  |
| --- | --- | --- |
| Traffic descriptor: VIAPA App1 Route Selection Descriptor Precedence=1 Access Type preference: 3GPP access, subscription PLMN id_1 |  | This SNPN URSP rule associates the traffic of application "VIAPA App1" with 3GPP access. The subscription of the PLMN_id_1 should be used for network selection. |  |

Editor's note: Using PLMN ID 1 subscription does not ensure that the UE will register on that PLMN; instead the UE may register on an equivalent PLMN or other PLMNs. How QoS can be guaranteed in thise case is FFS.

Table 6.6.2.1-3:  PLMN URSP rules to route VIAPA App1 though 3GPP access of PLMN_1

| Example PLMN URSP rules (PLMN_URSP_1) | Comments |  |
| --- | --- | --- |
| Traffic descriptor: VIAPA App1 Route Selection Descriptor Precedence=1 Network Slice Selection: S-NSSAI-b SSC Mode Selection: SSC Mode 3 DNN Selection: internet Access Type preference: 3GPP access |  | This PLMN URSP rule associates the traffic of application "VIAPA App1" with S-NSSAI-a, SSC Mode 3, the "internet" DNN and 3GPP Access. |  |

Method 2: A new UE Policy

A UE Policy to indicate which subscription to use for NW selection for VIAPA applications or VIAPA DNNs is described below:

Table 6.6.2.1-4: Route Selection Descriptor extended with “indication on subscription to use for NW selection”

| Information name | Description | Category |
| --- | --- | --- |
| Traffic descriptor | Application descriptor or IP descriptor or DNN | Mandatory |
| Indication on subscription to use for NW selection | Indicates the subscription to use for NW selection when the UE detects traffic matching the traffic descriptor. If a match is found, the UE uses the indication to select a subscription to trigger NW selection. | Mandatory |

### 6.X.3	Procedures

#### 6.X.3.1	Initial provisioning of QoS requirements from the ASP to the PLMN

Prior to transfer of VIAPA applications, both the ASP provides QoS requirements for an initial check at the  PLMN, the ASP provides the number of UEs that are registered for the event, their location and the time of the day. The PLMN requests predictions on QoS Sustainability to know the number of QoS flows (for VIAPA services) that will be released in the area and time where the UEs will be located. If the number reaches a threshold, then the PLMN considers that there will be QoS degradation and will determine with UE should use Uu interface in the SNPN to access VIAPA application.

Figure 6.x.3.2. VIAPA ASP negotiates a Transfer Policy for users of the VIAPA application

- Web portal at the ASP offers tickets to VIAPA application to end users. At some point in time, before the VIAPA application starts, the ASP informs the SNPN, using Nnef_Negotiate_Transfer_Policy Request service operation that a number of users, list of users, will use a VIAPA application, at certain time and location.

- The PLMN NEF authorizes the ASP request, select then send Npcf_Negotiate_Transfer_Policy Request to the PLMN PCF to handle the ASP request, this can be any PCF supporting this capability or service. The PLMN PCF checks the QoS requirements for the VIAPA application, the number of users and their location/time and determines using local policies and analytics on the ”QoS sustainability” in the location (i.e. gNB) where UEs would be located, and the 5QI of the VIAPA application. If the SNPN PCF estimates that the users of the VIAPA application would not experience QoS degradation, since the QoS sustainability threshold is not met then step 6 takes place.

- The PLMN PCF may decide that QoS sustainability threshold will not be met and then determine which UE should use Uu interface in one of SNPN that has an agreement with to access VIAPA application. The PCF determines which UEs should use the Uu interface of the SNPN based on the SLA that the PLMN has with the SNPN, UE capabilities, user preferences, and operator policies.

- The PLMN PCF stores the PLMN_Transfer_Policy Id and its context into PLMN UDR using Nudr_DM_Create_Request for Data Set Application data.

- The SNPN UDR sends Nudr_DM_Create_Response.

- The SNPN PCF sends Npcf_Negotiate_Transfer_Policy Response to SNPN NEF including the SNPN_Transfer_Policy_Id.

- The SNPN NEF sends Nnef_Negotiate_Transfer_Policy Response including the SNPN_Transfer_Policy_id that may indicate whether some users may experience QoS degradation based on predictions.

Example:

- AF informs that VIAPA_App_1 is delivered to 100 users, (GPSI_1, GPSI_100), geographical area and time of day

- NEF translate GPSI. Geographical area to SUPI and area of interest

- PLMN PCF calculates 5QI, PER and PDB for VIAPA_App_1. PCF subscribes to NWDAF to report predictions on QoS retainability when threshold is reached, threshold is set to 10, area of interest (gnB) and time of day.

- PLMN PCF is informed that QoS retainability threshold is reached, so that 10 QoS flows are predicted to be released.

- PLMN PCF checks which SNPN has a SLA with and determines which UEs should use Uu interface of the SNPN.

#### 6.X.3.1	Provisioning of NW preferences on the Uu interface to use for VIAPA applications to the UE

The procedure for UE Policy Association Establishment defined in TS 23.502 clause 4.16.11 is used, and then the PCF provides a new UE Policy (method 2) or a URSP Rule (method 1) to route VIAPA traffic via the 3GPP access of a different Network.

Figure 6.3.x.1-1. input for selection of subscription in the UE to use for Network selection

Editor's note: Whether the decision which PLMN subscription to use can be determined by an SNPN is expected to be confirmed by SA1.

### 6.X.4	Impacts on existing entities and interfaces

NEF:

-	New service to request negotiation of transfer policy for VIAPA applications.

PCF:

-	New service to request negotiation of transfer policy for VIAPA applications.

-	Subscription to NWDAF Analytics on QoS Sustainability for GBR QoS flows for a 5QI, PDB, PER         corresponding to a VIAPA application.

-	Store Transfer Policy and PLMN URSP rule id for UEs that will transfer VIAPA applications via a different PLMN.

-	Provide URSP rule for VIAPA applications to be transferred over a different PLMN.

UE:

-	Determine the Uu interface to be used to send VIAPA traffic based on the URSP rule extended (method 1) or w new UE Policy (method 2) with the PLMN subscription.

*** END CHANGES ***