SA WG2 Meeting #140	S2-2005213

eMeeting, 19 Aug – 2 Sep, 2020

Source:	China Telecom

Title:	Solution update for KI#3 on limitation of data rate per network slice in UL and DL per UE

Document for:	Approval

Agenda Item:	8.4

Work Item / Release:	FS_eNS_Ph2 / Rel-17

Abstract of the contribution: This contribution proposes a solution for KI#3.

1. Introduction

This solution addresses the below requirements from Key Issue #3: limitation of data rate per network slice in UL and DL per UE, including:

- Whether and how to limit the data rate of UE for a Network Slice, ensuring that the aggregate of the PDU sessions that use the slice are rate limited to the rate defined for the Network Slice in DL and UL, including:

- How to signal the rate limits.

- Impact on subscription data.

Based on the incoming LS response from GSMA, the new attributes for Guarantee and Maximum Uplink/Downlink Throughput per network slice per UE should apply to both GBR and non-GBR traffic of Uplink and Downlink, respectively.

According to that requirement, this solution suggests introducing two separate new attributes, slice-UL-AMBR and slice-DL-AMBR, which are provided by OAM/UDR to PCF in policy control procedures. For Non-GBR traffic, if UE supports rate limitation function, the limitation enforced by UE for UL traffic and UPF for DL traffic, otherwise the limitation enforced by UPF for U/DL traffic ; while the (R)AN is responsible for the enforcement for GBR traffic.

3. Proposal

It is proposed to adopt the following solutions for Key issue#3 into 23.700-40.

6.13	Solution #13: Limitation of data rate per network slice in UL and DL per UE

6.13.1 Introduction

Each slice is associated with the following aggregate rate limit QoS parameters for each UE:

- Slice-UL-AMBR, per Slice UpLink Aggregate Maximum Bit Rate per UE

- Slice-DL-AMBR, per Slice DownLink Aggregate Maximum Bit Rate per UE

These parameters limit the aggregate bit rate that can be expected to be provided across non-GBR QoS flows of a UE by a specific network slice and the existence of GBR flows within the same S-NSSAI.

Suppose when a UE initiates a PDU session establishment request, it carries a capability identifier indicating whether the UE supports rate limitation function for non-GBR data traffic, and SMF stores this capability identifier for UE. SMF will prefer UE as the enforcement node for UpLink non-GBR data rate limitation, and if UE does not support this feature, UPF will be selected instead.

6.13.2 High-level Description

The proposed Slice-UL-AMBR and Slice-DL-AMBR should be part of user’s subscription data and could be configured by OAM and retrieved by PCF/UDR flexibly. The subscribed Slice-UL-AMBR and Slice-DL-AMBR should firstly guarantee the sum of GFBR of all active GBR flows for the UE within the S-NSSAI, then the rest quota is for all Non-GBR flows on best-effort services as authorized Slice-UL-AMBR and Slice-DL-ABMR.

Slice-UL-AMBR and Slice-DL-AMBR is calculated as follows:

- When there is no active GBR flows for a specific network slice of a UE, i.e., all traffic is of Non-GBR flow, the authorized Slice-UL-AMBR and Slice-DL-AMBR are set to the value of subscribed Slice-UL-AMBR and Slice-DL-AMBR respectively.

- When a UE initiates a new PDU session, a Non-GBR flow is established by default. It should not affects authorized Slice-UL-AMBR and Slice-DL-AMBR, i.e., no need to update the authorized value.

- When a Non-GBR flow is to be established or terminated in existing sessions for a specific S-NSSAI, for best-effort service, the procedure should be as current defined in TS23.502 and the authorized Slice-UL-AMBR/Slice-DL-AMBR values stay the same.

- Only when a GBR flow is to be established or terminated in existing sessions, the authorized Slice-UL-AMBR and Slice-DL-AMBR should be re-calculated by PCF and delivered to enforcement nodes. For example:

When a UE modifies a PDU session with new GBR flow request within an S-NSSAI, PCF calculates the sum of GFBR of the target GBR flow and all active GBR flows of that S-NSSAI for UL and DL separately.

- If the sum of UL is no greater than the subscribed Slice-UL-AMBR, and the sum of DL is no greater than the subscribed Slice-DL-AMBR, the new GBR flow is OK to be established. Then the authorized Slice-UL-AMBR and Slice-DL-AMBR of the UE will be set to the remaining of subscribed slice-AMBR excepting the sum of GBR flows.

- If the sum of UL is greater than the subscribed slice-UL-AMBR, or the sum of DL is greater than the subscribed slice-DL-AMBR, PCF will decide whether to pre-empt existing GBR flow for the new one based on ARP as defined in Clause 5.7.2.2 of TS23501. If old flow is pre-empty for new flow, the authorized Slice-UL-AMBR and Slice-DL-AMBR of the S-NSSAI of the UE will be set to the remaining of subscribed slice-AMBR regarding the new GBR flow. If not, the establishment request will be rejected and the limit for Non-GBR flows stays the same.

The authorized Slice-UL-AMBR of the S-NSSAI of the UE will be sent to UE for enforcement. The authorized Slice-DL-AMBR of the S-NSSAI of the UE will be sent to UPF for enforcement. (R)AN responsible for the enforcement for GBR traffic, i.e. reserving resource for new flows while terminating old flows if indicated by PCF.

To assure the accuracy of data limitation control, the proposed method is based on the assumption that, the network selects the same SMF/UPF/PCF for all the sessions of the same S-NSSAI per UE. It is recommended to add some subscribed information (e.g. slice data rate limitation requirement identification) in UDM for certain UE that means selecting the same SMF for all PDU sessions with certain S-NSSAI. When such UE initiate a PDU session establishment request, AMF selects the same SMF for the UE based on the subscribed information of the UE in UDM and passes the relevant information to SMF，thus SMF selects the same PCF and UPF for all the PDU sessions of the same S-NSSAI per UE.

NOTE: The way for AMF to choose the same SMF for all PDU sessions per UE by the subscription information in UDM has been specified in TS23.501.This solution has taken such way, and extend to the all PDU sessions per UE per Slice.

6.13.3 Procedures

The following takes a PDU session establishment/modification with new GBR flow to illustrate the procedure of Slice-UL-AMBR and Slice-DL-AMBR enforcement.

- Suppose when a UE initiates a PDU session establishment request, it carries a capability identifier indicating that the UE supports rate limitation function for non-GBR data traffic, and SMF stores this capability identifier for UE.

Figure 1 Slice-UL-AMBR and Slice-DL-AMBR enforcement at PDU Session Establishment/Modification (When UE indicates that it can support rate limitation function for non-GBR data traffic)

Step 1,2,3,4. Same as specified in TS 23.502.

Step 5. The PCF makes policy decision for the request of new GBR flow as described in 6.x.2. If the new flow is accepted, PCF updates the authorized Slice-UL-AMBR and Slice-DL-AMBR and selects the existing flow(s) with lower ARP value to terminate in case of insufficient available resource.

Step6. The PCF replies with the updated authorized Slice-UL-AMBR and Slice-DL-AMBR to the SMF, as well as indication of which GBR flow(s) with lower ARP value should be terminated for the new one if needed

Step 7. The SMF provides the updated authorized Slice-DL-AMBR to the same UPF for download data limitation enforcement per UE per slice for non-GBR flow.

Step 8. The SMF sends to AMF with the updated authorized Slice-UL-AMBR, and the indication of flows to be terminated if needed.

Step 9 and 10. The Session establishment/modification result with updated authorized Slice-UL-AMBR is delivered to (R)AN and UE. The upload data limitation per UE per slice is enforced by UE for non-GBR flow. While the GBR flow resource is controlled by (R)AN as current procedures defined in TS23.501 [2].

- Suppose when a UE initiates a PDU session establishment request, it carries a capability identifier indicating that the UE cannot support rate limitation function for non-GBR data traffic, and SMF stores this capability identifier for UE.

Figure 2 Slice-UL-AMBR and Slice-DL-AMBR enforcement at PDU Session Establishment/Modification (When UE indicates that it cannot support rate limitation function for non-GBR data traffic)

Step 1,2,3,4. Same as specified in TS 23.502.

Step 5. The PCF makes policy decision for the request of new GBR flow as described in 6.x.2. If the new flow is accepted, PCF updates the authorized Slice-UL-AMBR and Slice-DL-AMBR and selects the existing flow(s) with lower ARP value to terminate in case of insufficient available resource.

Step 6. The PCF replies with the updated authorized Slice-UL-AMBR and Slice-DL-AMBR to the SMF, as well as indication of which GBR flow(s) with lower ARP value should be terminated for the new one if needed

Step 7. The SMF provides the updated authorized Slice-UL/DL-AMBR to the same UPF for download data limitation enforcement per UE per slice for non-GBR flow.

Step 8. The SMF sends to AMF with the indication of flows to be terminated if needed.

Step 9. The Session establishment/modification result is delivered to (R)AN including the indicator of existing GBR flow to be terminated if needed. While the GBR flow resource is controlled by (R)AN as current procedures defined in TS23.501 [2].

6.13.4 Impacts on existing services and interfaces

Impacts on UDM

- Responsible for adding some subscribed information for certain UE (e.g. slice data rate limitation requirement identification) that means selecting the same SMF for all PDU sessions with certain S-NSSAI.

Impacts on UDR:

- Responsible for the storage of subscribed Slice-UL-AMBR and Slice-DL-AMBR

Impacts on PCF:

- Responsible for calculating the authorized Slice-UL-AMBR and Slice-DL-AMBR in PDU session per UE

Impacts on AMF:

- Select the same SMF for all the PDU Sessions within the same S-NSSAI.

Impacts on SMF:

- Responsible for storing the capability indicator of data rate limitation for UE, delivering the decision and authorized Slice-UL-AMBR and Slice-DL-AMBR to (R)AN, UE and UPF respectively.

- Select the same UPF for all the PDU Sessions within the same S-NSSAI.

Impacts on (R)AN:

- Enforcement of GBR flow control and deliver authorized Slice-UL-AMBR to UE.

Impacts on UPF:

- Enforcement of Slice-UL/DL-AMBR limitation per UE per S-NSSAI for non-GBR data traffic.

Impacts on UE:

- Adding a capability identifier indicating whether the UE support data rate limitation for non-GBR data traffic.

- Enforcement of Slice-UL-AMBR limitation per S-NSSAI for upload non-GBR traffic.