3GPP TSG-WG SA2 Meeting #140E e-meeting 	S2-2005409

Elbonia, August 19 – September 1, 2020	(revision of S2-200xxxx)

Source:	Huawei, HiSilicon, Nokia, Nokia Shanghai Bell, Qualcomm Incorporated, CATT

Title:	New architecture for MBS

Document for:	Approval

Agenda Item:	8.9

Work Item / Release:	FS_5MBS / Rel-17

Abstract: This contribution introduces a potential way forward for the architecture to be used with 5MBS.

1. Introduction

Currently there are two baseline architectures proposed for 5G MBS as depicted in TR 23.757 Annex A.1 and A.2 respectively, and based on each architecture, quite a few solutions have been proposed for different key issues. To facilitate the conclusion of the TR, it would be helpful if the two architectures could be consolidated into one.

An analysis of network functionalities in architecture 1 and 2 for supporting 5G MBS is shown in Table 1.1-1.

  - On the Functionalities of two architecture baselines

Table 1.1-1 Analyses of the functionalities of the architectural alternatives

|  | Baseline architecture 1 (as per TR description) | Baseline architecture 2 (as per TR description) | Converged architecture |
| --- | --- | --- | --- |
| UE | - Support of UE policy configuration extension to MBS. - Support of SM extension for MBS flows. - Signaling for joining MBS flow (via SM signaling or user plane IGMP Join). - MBS support at AS layer. | - Support 5G MBS services. | - Support of UE policy configuration extension to MBS. - Support of SM extension for MBS flows. - Signaling for joining MBS flow - MBS support at AS layer (RAN responsibility). |
| NG-RAN | - Receive MBS flows via N3 and delivery over-the-air. - Switch between multicast and unicast delivery of MBS flows. - Configure UE for MBS flow reception at AS layer. (TBD how UE AS layer configuration of 5G MBS works) | - Support Point-to-Multipoint (PTM) and Point-to-Point (PTP) delivery of MBS media. - Control switching between PTM and PTP for best service quality and resource efficiency independently. | - Support session management extensions for MBS flows via N3 (e.g., tunnel establishment during HO). - Configure UE for MBS flow reception at AS layer (RAN responsibility). - Deliver MBS data over PTM or PTP (RAN responsibility). - Control switching between PTM and PTP delivery for UEs independently (RAN responsibility). |
| (MB-)SMF | - Control of MBS transport, based on received MBS policies from PCF. - Configure UPF for MBS flows and for point to point or point-to-multipoint transfer. - Configure RAN for MBS flows and QoS information. - SM configuration at the UE for MBS flows. (An SMF may be used for both unicast and MBS) | - Control MB Sessions, signaling with AF (via NEF/MBSF), QoS control using PCF, - Provision MB Session information on request from AMF. - The PDU session(s) the UE(s) maintain for individual delivery of an MBS service may be associated with the MB Session managed by the MB-SMF. | - Control MBS Sessions (including MBS transport and QoS) based on received MBS policies from PCF. - Signaling with NG-RAN and AMF for MBS Session management. - Signaling with AF (via, e.g., NEF, MBSF) for, e.g., allocation of 5GC entry point. In addition, it may support one or more of the following based on the selected solution(s): - Handling of session join (including UE authorization) and management of MB session context. - Configure UPF for MBS flows and for individual or shared MBS traffic delivery (including switching). |
| (MB-)UPF | - Support of packet filtering of MBS flows, and delivery of MBS flows to RAN via point to point or point-to-multipoint N3. - Receive 5G MBS flow configuration from SMF. - Detect IGMP packets and notify SMF. (A UPF may receive both unicast and MBS flows) (I-UPF(s) may be used for the delivery of MBS flows from the UPF attached to N6 to NG-RAN; the N9 interface can be used for MBS traffic delivery) | - Support MBS UP function. | - Packet filtering for MBS flows. - Replication of MBS data packets to different RAN nodes. - Delivery of MBS flows to RAN via shared MBS traffic delivery over N3. - QoS enforcement (MFBR) and counting/reporting based on existing means. In addition, it may support one or more of the following based on the selected solution(s): - Detect IGMP packets and notify SMF. - Delivery of MBS flows to RAN via individual MBS traffic delivery over N9. |
| AMF | - No requirement | - Select MB-SMF and be part of the signaling distribution tree. | - Signaling with NG-RAN and (MB-)SMF for MBS Session management. In addition, it may support one or more of the following based on the selected solution(s): - Handling of session join and management of MB session context. - Selection of MB-SMF. |
| PCF | - Support policies for Multicast services, including QoS parameters like 5QI, MBR, GBR. - Provide policy information regarding the MBS session to SMF. - Receive MBS service information from AF, directly (operator owned) or indirectly via NEF. | - PCF is enhanced to handle QoS for MB Sessions, e.g. to authorize the QoS profiles for shared delivery. | - Support QoS handling for MBS Session, including QoS parameters like 5QI, MFBR, GFBR - Provide policy information regarding the MBS session to SMF for authorizing the related QoS profiles. - Receive MBS service information from AF, directly or indirectly (via NEF). |
| MBSF | MSF Control Plane (MSF-C) - Multicast service configuration. - MBS service level management. - xMB-C/MB2-C termination. - Codec configuration (if needed). | May be part of NEF or be deployed independently. - Support TMGI allocation or other MBS signaling for the service level management. - Provides an interface to the Application Function or content provider. - Interface to the MBSU. - Perform authorization of the UE to join the MB session. | - Interact with AF to control MBSU, if encoding at MBSU is needed. In addition, it may support based on the selected solution(s): - Support TMGI allocation if this is not done by MB-SMF. |
| MBSU | MSF User Plane (MSF-U): - xMB-U/MB2-U termination. - Encoding of data at service layer. - Multicast service layer data packets delivery via N6. | Handle the payload part to cater for the service level functions and management. | - Handling of encoding of MBS data. NOTE: The MBSF and the MBSU may be collocated or separated physical nodes. |
| NEF | - 5G MBS service exposure. - Negotiation of 5G MBS service with AF, including QoS, 5G MBS service area | - NEF is an existing NF, which provides interface to the AF | - 5G MBS service exposure. - Negotiation of 5G MBS service with AF, including QoS, 5G MBS service area NOTE: If separated from the MBSU, the MBSF may be implemented in the NEF. |
| AF | - Support MBS service functions, negotiation with NEF for service exposure. | - Signalling with MB-SMF (via NEF/MBSF) | - Support MBS service functions, negotiation with NEF for service exposure. |

Based on the analytics, a merged architecture supporting 5G MBS is proposed as follows:

UE

NG-RAN

(MB-)UPF

AMF

(MB-)SMF

NEF

MBSF

MBSU

Data Network

Application Server

PCF

N6

gMB-U

(xMB-U/

MB2-U)

Ny

N4

N3

Uu

N2

Namf

Nsmf

Npcf

Nnef

Naf

Nmbsf

SMF

Nsmf

UDR

Nudr

AF

gMB-C (xMB-C/

MB2-C)

N6

Figure A.1.1-1: 5GS Architecture supporting MBS

2. Text Proposal

It is proposed to capture the following changes to TR 23.757.

# * * * * First change * * * * (All Text New)

Annex A:
Architecture alternatives

A.X	Baseline architecture X: 5G MBS system architecture

A.X.1	General

Based on the two baseline architectures for 5G MBS as depicted in Annex A.1 and A.2, a consolidated 5GS architecture supporting MBS is proposed.

A.X.2	Reference Architecture

A.X.2.1	General

Figure A.X.2.1-1 illustrates the 5GS architecture supporting MBS.

UE

NG-RAN

MB-UPF

AMF

MB-SMF

NEF

MBSF

MBSU

Data Network

Application Server

PCF

N6

gMB-U

(xMB-U/

MB2-U)

Ny

N4

N3

Uu

N2

Namf

Nmbsmf

Npcf

Nnef

Naf

Nmbsf

SMF

Nsmf

UDR

Nudr

AF

gMB-C (xMB-C/

MB2-C)

N6

N1

Figure A.X.2.1-1: 5GS Architecture supporting MBS

In Figure A.X.2.1-1, the SMF and UPF which have the roles to support MBS Sessions are named "MB-SMF" and "MB-UPF" respectively.

NOTE 1:	In certain scenarios, a UPF can be placed between MB-UPF and NG-RAN.

NOTE 2:	The MB-SMF may be collocated with SMF, the MB-UPF may be collocated with UPF, i.e. SMF and UPF may be enhanced to support 5G MBS.

NOTE 3:	The architecture using the reference point representation may be specified during normative phase based on the conclusion.

NOTE 4:	gMB-C and gMB-U provide similar functionalities defined for xMB-C/MB2-C and xMB-U/MB2-U interfaces, respectively.

The 5G MBS System Architecture contains the following new service-based interfaces:

Nmbsmf:	Service-based interface exhibited by MB-SMF.

Nmbsf:	Service-based interface exhibited by MBSF.

A.X.2.2	Functional entities

NOTE:	The enhancements of network functions only covers commonalities of architecture alternative 1 and 2, and can be refined in normative work.

A.X.2.2.1	PCF

The PCF is used to provide policy rules for MBS services, and receive MBS service information from AF, directly or indirectly via NEF and UDR. The PCF performs the following enhancements to support MBS:

-	Support QoS handling for MBS Session, including QoS parameters like 5QI, MFBR, GFBR

-	Provide policy information regarding the MBS session to SMF for authorizing the related QoS profiles.

-	Receive MBS service information from AF, directly or indirectly (via NEF and UDR).

A.X.2.2.2	MB-SMF

The MB-SMF is used for MBS session management and control of MBS transport, including configuring the MB-UPF and RAN of NG UP tunnel and QoS control for MBS flows transport based on the policy rules for MBS services from PCF or local policy. The MB-SMF performs the following functions to support MBS:

-	Control MBS Sessions (including MBS transport and QoS) based on received MBS policies from PCF or local policy.

-	Signaling with NG-RAN, MB-UPF and AMF for MBS Session management.

-	Signaling with AF (via, e.g., NEF, MBSF) for, e.g., allocation of 5GC user plane entry point.

A.X.2.2.3	MB-UPF

The MB-UPF is used for delivery of MBS flows to RAN via NG UP tunnel and QoS enforcement for MBS services. The UPF performs the following functions to support MBS:

-	Packet filtering for MBS flows.

-	Distribution of MBS data packets to different RAN nodes.

-	Delivery of MBS flows to RAN via shared MBS traffic delivery over N3.

-	QoS enforcement (MFBR) and counting/reporting based on existing means.

A.X.2.2.4	AMF

The AMF performs the following functions to support MBS:

-	Signaling with NG-RAN and MB-SMF for MBS Session management.

A.X.2.2.5	NG-RAN

The NG-RAN is used to receive MBS flows via N3 tunnel and deliver MBS flows to the UEs. The NG-RAN performs the following functions:

-	Support shared tunnel establishment and reception of MBS flows via the shared tunnel.

-	Configure UE for MBS flow reception at AS layer.

-	Deliver MBS data using PTM or PTP method.

-	Control switching between PTM and PTP delivery method for UEs.

A.X.2.2.6	UE

The UE needs to support the MBS operations and receive MBS flows from NG-RAN. The UE performs the following functions:

-	Support MBS operations.

-	MBS support at AS layer.

A.X.2.2.7	AF

The AF supports MBS service to the 5GC, including:

-	provide MBS service information.

-	interact with PCF for MBS policy control.

-	negotiation with NEF for MBS related network exposure.

A.X.2.2.8	NEF

The NEF supports 5G MBS service exposure to AF for service provisioning and session operations, including QoS, 5G MBS service area, etc.

A.X.2.2.9	MBSF

The MBSF performs the following functions:

-	Interact with AF to control MBSU, if encoding at MBSU is needed.

-	Interact with legacy multicast application server if needed.

A.X.2.2.10	MBSU

The MBSU performs the following functions:

-	Handling of encoding of MBS data.

NOTE: The MBSF and the MBSU may be collocated or deployed separately.

# * * * * Second change * * * *

8	Conclusions

INTERIM CONCLUSION: Architectural option X "5G MBS system architecture" as described in Annex A.X is chosen as the baseline architecture for 5G MBS.

# * * * * End of changes * * * *