3GPP TSG-WG SA2 Meeting #140E e-meeting 	S2-2005409

Elbonia, August 19 – September 1, 2020	(revision of S2-200xxxx)

Source:	Huawei, HiSilicon, Nokia, Nokia Shanghai Bell, Qualcomm Incorporated, CATT

Title:	New architecture for MBS

Document for:	Approval

Agenda Item:	8.9

Work Item / Release:	FS_5MBS / Rel-17

Abstract: This contribution introduces a potential way forward for the architecture to be used with 5MBS.

# 1. Introduction

Currently there are two baseline architectures proposed for 5G MBS as depicted in TR 23.757 Annex A.1 and A.2 respectively, and based on each architecture, quite a few solutions have been proposed for different key issues. To facilitate the conclusion of the TR, it would be helpful if the two architectures could be consolidated into one.

An analysis of network functionalities in architecture 1 and 2 for supporting 5G MBS is shown in Table 1.1-1.

## On the Functionalities of two architecture baselines

Table 1.1-1 Analyses of the functionalities of the architectural alternatives

|  | Baseline architecture 1 (as per TR description) | Baseline architecture 2 (as per TR description) | Baseline functions |
| --- | --- | --- | --- |
| UE | - Support of UE policy configuration extension to MBS. - Support of SM extension for MBS flows. - Signaling for joining MBS flow (via SM signaling or user plane IGMP Join). - MBS support at AS layer. | - Support 5G MBS services. - Reception of multicast data using PTM /PTP in RRC Connected. - MBS control client which can exchange Session Join, and Session Leave messages - Switch of MBS reception from Source to Target NG-RAN when Xn and N2 Handover execution phase commences. | - Support of UE policy configuration extension to MBS. - Reception of multicast data using PTM /PTP in RRC Connected. - Support of management of MBS flows. - Signaling for joining MBS flow - Switch of MBS reception from Source to Target NG-RAN when Xn and N2 Handover execution phase commences. - MBS support at AS layer (RAN responsibility). |
| NG-RAN | - Receive MBS flows via N3 and delivery over-the-air. - Switch between multicast and unicast delivery of MBS flows. - Configure UE for MBS flow reception at AS layer. (TBD how UE AS layer configuration of 5G MBS works) | - Support Point-to-Multipoint (PTM) and Point-to-Point (PTP) delivery of MBS media. - Control switching between PTM and PTP for best service quality and resource efficiency independently. - Support for MB Sessions and MBS related signalling on N2 with AMF and on N3 with UPF. - Transmission of multicast data using PTM /PTP in RRC Connected. - Support for MB Session info in Xn Handover request. - MB Session resource setup during Xn Handover preparation phase. - MB Session resource setup during Xn Handover execution phase. - New parameters in Path Switch messages. | - Management of MBS flows via N2 and N3 - Configuration of UE for MBS flow reception at AS layer (RAN responsibility). - Delivery of MBS data over PTM or PTP (RAN responsibility). - Control switching between PTM and PTP delivery per UE (RAN responsibility). - Support for MB Sessions during Xn Handover preparation phase and execution phase and at N2 Handovers (shared S2 & RAN responsibility) |
| (MB-)SMF | - Control of MBS transport, based on received MBS policies from PCF. - Configure UPF for MBS flows and for point to point or point-to-multipoint transfer. - Configure RAN for MBS flows and QoS information. - SM configuration at the UE for MBS flows. (An SMF may be used for both unicast and MBS) | - Control MB Sessions, signaling with AF (via NEF/MBSF), QoS control using PCF, - Provision MB Session information on request from AMF. - The PDU session(s) the UE(s) maintain for individual delivery of an MBS service may be associated with the MB Session managed by the MB-SMF. - MBS control function which can maintain the MBS signaling tree and respond to or send session requests to/from the NEF/MBSF, PCF and AF including allocation of TMGIs. - Optionally support deployments with SMFs dedicated to MBS services. - For 5GC Individual MBS traffic delivery, the SMF (for PDU Session) handles the request from AMF to receive 5MBS data from MB-UPF. | - Control MBS Sessions (including MBS transport and QoS) based on received MBS policies from PCF. - Signaling with AMF and NG-RAN via AMF for MBS Session and signaling distribution tree management. - Signaling with AF (via, e.g., NEF, MBSF) for, e.g., allocation of 5GC entry point and allocation of TMGIs. - Optionally support deployments with SMFs dedicated to MBS services. - Signalling with the PCF. In addition, it may support one or more of the following based on the selected solution(s): - Handling of session join (including UE authorization) and management of MB session context. - Configure (MB-)UPF for MBS flows and for individual or shared MBS traffic delivery (including switching). - For 5GC Individual MBS traffic delivery, the SMF (for PDU Session) handles the request from AMF to receive 5MBS data from MB-UPF. |
| (MB-)UPF | - Support of packet filtering of MBS flows, and delivery of MBS flows to RAN via point to point or point-to-multipoint N3. - Receive 5G MBS flow configuration from SMF. - Detect IGMP packets and notify SMF. (A UPF may receive both unicast and MBS flows) (I-UPF(s) may be used for the delivery of MBS flows from the UPF attached to N6 to NG-RAN; the N9 interface can be used for MBS traffic delivery) | - Support MBS UP function. - MBS user plane function with southbound IP Multicast tunnelling/transport interface and northbound interface with N6 tunnelling of MBS DL media streams. - On N3 interface, IP Multicast routing protocol for receiving IGMP/MLD join/leave to the multicast transport IP address ("LL MC"). - Support MBS operations on the N4 interface for MB Session setup and release. - For 5GC Individual MBS traffic delivery, the (PSA-)UPF is requested by SMF to forward MBS data stream onto PDU Session and to join or leave the MC tree of the (MB-)UPF source host. | - Support MBS operations on the N4 interface for MB Session setup and release. - Packet filtering for MBS flows. - Replication of MBS data packets to different RAN nodes or different UPF nodes. - Delivery of MBS flows to RAN or UPF via shared MBS traffic delivery over N3/N9. - On N3 interface, IP Multicast routing protocol for receiving IGMP/MLD join/leave to the multicast transport IP address ("LL MC"). - QoS enforcement (MFBR) and counting/reporting based on existing means. In addition, it may support one or more of the following based on the selected solution(s): - Detect IGMP packets and notify SMF of the IPmc address. - Delivery of MBS flows to RAN via individual MBS traffic delivery over N9. |
| AMF | - No requirement | - Select MB-SMF and be part of the signaling distribution tree. - MBS control function which can maintain the MBS signaling tree and control MBS resources in NG-RAN. - Support selection of SMFs that have MBS capabilities using NRF. - For 5GC Individual MBS traffic delivery, the AMF interacts with SMF of PDU Session so that the PSA UPF can start/stop receiving 5MBS data from MB-UPF. | - Signaling with UE, NG-RAN and (MB-)SMF for MBS Session management and for maintaining the MBS signaling tree. - Support selection of (MB-)SMFs that have MBS capabilities. - Interaction with SMF for the PSA UPF to start/stop receiving 5MBS data from MB-UPF for 5GC Individual MBS traffic delivery. - Handling of session join and management of MB session context. |
| PCF | - Support policies for Multicast services, including QoS parameters like 5QI, MBR, GBR. - Provide policy information regarding the MBS session to SMF. - Receive MBS service information from AF, directly (operator owned) or indirectly via NEF. | - PCF is enhanced to handle QoS for MB Sessions, e.g. to keep 5G QoS Profiles for MB Sessions or authorize QoS profiles for shared delivery. - Receive MBS service requirements from AF, indirectly via (MB-)SMF and NEF. In addition, it may support one or more of the following based on the selected solution(s): - For AF specific priority of group member, the PCF recives from AF and provides towards RAN PDU session policy control information including the MBS Session ID and MBS priority using existing procedure for PDU Session QoS update. | - Support QoS handling for MBS Session, including QoS parameters like 5QI, MFBR, GFBR - Provide policy information regarding the MBS session to (MB-)SMF for authorizing the related QoS profiles. - Receive MBS service information from AF, directly or indirectly In addition, it may support one or more of the following based on the selected solution(s): - For AF specific priority of group member, the PCF recives from AF and provides towards RAN PDU session policy control information including the MBS Session ID and MBS priority using existing procedure for PDU Session QoS update. |
| MBSF | MSF Control Plane (MSF-C) - Multicast service configuration. - MBS service level management. - xMB-C/MB2-C termination. - Codec configuration (if needed). | May be part of NEF or be deployed independently. - Support TMGI allocation or other MBS signaling for maintaining the signaling distribution tree and the service level management. - Provides an interface to the Application Function or content provider. - Interface to the MBSU. - Perform authorization of the UE to join the MB session. | - Interact with AF and MB-SMF to control MBSU, e.g. if encoding at MBSU is needed. In addition, it may support based on the selected solution(s): - Support TMGI allocation if this is not done by MB-SMF. |
| MBSU | MSF User Plane (MSF-U): - xMB-U/MB2-U termination. - Encoding of data at service layer. - Multicast service layer data packets delivery via N6. | - Handle the payload part to cater for the service level functions and management. - Anchor and forward MBS data in case regionalized deployment with multiple MB-UPFs | - Handling of encoding of MBS data. - Anchor and forward MBS data in case regionalized deployment with multiple MB-UPFs NOTE: The MBSF and the MBSU may be collocated or separated physical nodes. |
| NEF | - 5G MBS service exposure. - Negotiation of 5G MBS service with AF, including QoS, 5G MBS service area | - NEF is an existing NF, which provides interface to the AF - Maintain the MBS signaling tree using MBSF. Managing session requests from the AF. - Support selection of SMFs that have MBS capabilities based on NRF registration. | - Generally being 5G MBS service exposure and specifically providing an interface for 5MBS procedures. - Maintain the MBS signaling tree using MBSF. Managing session requests from the AF. - Negotiation of 5G MBS service with AF, including QoS - Support selection of SMFs that have MBS capabilities NOTE: If separated from the MBSU, the MBSF may be implemented in the NEF. |
| AF | - Support MBS service functions, negotiation with NEF for service exposure. | - Signalling with MB-SMF (via NEF/MBSF) - The AF may provide UE specific priority to the PCF. | - Support MBS service functions, negotiation with NEF for service exposure. - The AF may provide UE specific priority to the PCF. |

# 2. Text Proposal

It is proposed to capture the following changes to TR 23.757.

# * * * * First change * * * * (All Text New)

Annex A:
Architecture alternatives

## A.X	Baseline Functions: 5G MBS system architecture

### A.X.1	General

Based on the two baseline architectures for 5G MBS as depicted in Annex A.1 and A.2, the commonalities and differences are captured here.

Editor’s Note: This is work in progress towards achieving a common 5G MBS architecture.

Table AX.1.1-1 Analyses of the functionalities of the architectural alternatives

|  | Baseline architecture 1 (as per TR description) | Baseline architecture 2 (as per TR description) | Baseline functions |
| --- | --- | --- | --- |
| UE | - Support of UE policy configuration extension to MBS. - Support of SM extension for MBS flows. - Signaling for joining MBS flow (via SM signaling or user plane IGMP Join). - MBS support at AS layer. | - Support 5G MBS services. - Reception of multicast data using PTM /PTP in RRC Connected. - MBS control client which can exchange Session Join, and Session Leave messages - Switch of MBS reception from Source to Target NG-RAN when Xn and N2 Handover execution phase commences. | - Support of UE policy configuration extension to MBS. - Reception of multicast data using PTM /PTP in RRC Connected. - Support of management of MBS flows. - Signaling for joining MBS flow - Switch of MBS reception from Source to Target NG-RAN when Xn and N2 Handover execution phase commences. - MBS support at AS layer (RAN responsibility). |
| NG-RAN | - Receive MBS flows via N3 and delivery over-the-air. - Switch between multicast and unicast delivery of MBS flows. - Configure UE for MBS flow reception at AS layer. (TBD how UE AS layer configuration of 5G MBS works) | - Support Point-to-Multipoint (PTM) and Point-to-Point (PTP) delivery of MBS media. - Control switching between PTM and PTP for best service quality and resource efficiency independently. - Support for MB Sessions and MBS related signalling on N2 with AMF and on N3 with UPF. - Transmission of multicast data using PTM /PTP in RRC Connected. - Support for MB Session info in Xn Handover request. - MB Session resource setup during Xn Handover preparation phase. - MB Session resource setup during Xn Handover execution phase. - New parameters in Path Switch messages. | - Management of MBS flows via N2 and N3 - Configuration of UE for MBS flow reception at AS layer (RAN responsibility). - Delivery of MBS data over PTM or PTP (RAN responsibility). - Control switching between PTM and PTP delivery per UE (RAN responsibility). - Support for MB Sessions during Xn Handover preparation phase and execution phase and at N2 Handovers (shared S2 & RAN responsibility) |
| (MB-)SMF | - Control of MBS transport, based on received MBS policies from PCF. - Configure UPF for MBS flows and for point to point or point-to-multipoint transfer. - Configure RAN for MBS flows and QoS information. - SM configuration at the UE for MBS flows. (An SMF may be used for both unicast and MBS) | - Control MB Sessions, signaling with AF (via NEF/MBSF), QoS control using PCF, - Provision MB Session information on request from AMF. - The PDU session(s) the UE(s) maintain for individual delivery of an MBS service may be associated with the MB Session managed by the MB-SMF. - MBS control function which can maintain the MBS signaling tree and respond to or send session requests to/from the NEF/MBSF, PCF and AF including allocation of TMGIs. - Optionally support deployments with SMFs dedicated to MBS services. - For 5GC Individual MBS traffic delivery, the SMF (for PDU Session) handles the request from AMF to receive 5MBS data from MB-UPF. | - Control MBS Sessions (including MBS transport and QoS) based on received MBS policies from PCF. - Signaling with AMF and NG-RAN via AMF for MBS Session and signaling distribution tree management. - Signaling with AF (via, e.g., NEF, MBSF) for, e.g., allocation of 5GC entry point and allocation of TMGIs. - Optionally support deployments with SMFs dedicated to MBS services. - Signalling with the PCF. In addition, it may support one or more of the following based on the selected solution(s): - Handling of session join (including UE authorization) and management of MB session context. - Configure (MB-)UPF for MBS flows and for individual or shared MBS traffic delivery (including switching). - For 5GC Individual MBS traffic delivery, the SMF (for PDU Session) handles the request from AMF to receive 5MBS data from MB-UPF. |
| (MB-)UPF | - Support of packet filtering of MBS flows, and delivery of MBS flows to RAN via point to point or point-to-multipoint N3. - Receive 5G MBS flow configuration from SMF. - Detect IGMP packets and notify SMF. (A UPF may receive both unicast and MBS flows) (I-UPF(s) may be used for the delivery of MBS flows from the UPF attached to N6 to NG-RAN; the N9 interface can be used for MBS traffic delivery) | - Support MBS UP function. - MBS user plane function with southbound IP Multicast tunnelling/transport interface and northbound interface with N6 tunnelling of MBS DL media streams. - On N3 interface, IP Multicast routing protocol for receiving IGMP/MLD join/leave to the multicast transport IP address ("LL MC"). - Support MBS operations on the N4 interface for MB Session setup and release. - For 5GC Individual MBS traffic delivery, the (PSA-)UPF is requested by SMF to forward MBS data stream onto PDU Session and to join or leave the MC tree of the (MB-)UPF source host. | - Support MBS operations on the N4 interface for MB Session setup and release. - Packet filtering for MBS flows. - Replication of MBS data packets to different RAN nodes or different UPF nodes. - Delivery of MBS flows to RAN or UPF via shared MBS traffic delivery over N3/N9. - On N3 interface, IP Multicast routing protocol for receiving IGMP/MLD join/leave to the multicast transport IP address ("LL MC"). - QoS enforcement (MFBR) and counting/reporting based on existing means. In addition, it may support one or more of the following based on the selected solution(s): - Detect IGMP packets and notify SMF of the IPmc address. - Delivery of MBS flows to RAN via individual MBS traffic delivery over N9. |
| AMF | - No requirement | - Select MB-SMF and be part of the signaling distribution tree. - MBS control function which can maintain the MBS signaling tree and control MBS resources in NG-RAN. - Support selection of SMFs that have MBS capabilities using NRF. - For 5GC Individual MBS traffic delivery, the AMF interacts with SMF of PDU Session so that the PSA UPF can start/stop receiving 5MBS data from MB-UPF. | - Signaling with UE, NG-RAN and (MB-)SMF for MBS Session management and for maintaining the MBS signaling tree. - Support selection of (MB-)SMFs that have MBS capabilities. - Interaction with SMF for the PSA UPF to start/stop receiving 5MBS data from MB-UPF for 5GC Individual MBS traffic delivery. - Handling of session join and management of MB session context. |
| PCF | - Support policies for Multicast services, including QoS parameters like 5QI, MBR, GBR. - Provide policy information regarding the MBS session to SMF. - Receive MBS service information from AF, directly (operator owned) or indirectly via NEF. | - PCF is enhanced to handle QoS for MB Sessions, e.g. to keep 5G QoS Profiles for MB Sessions or authorize QoS profiles for shared delivery. - Receive MBS service requirements from AF, indirectly via (MB-)SMF and NEF. In addition, it may support one or more of the following based on the selected solution(s): - For AF specific priority of group member, the PCF recives from AF and provides towards RAN PDU session policy control information including the MBS Session ID and MBS priority using existing procedure for PDU Session QoS update. | - Support QoS handling for MBS Session, including QoS parameters like 5QI, MFBR, GFBR - Provide policy information regarding the MBS session to (MB-)SMF for authorizing the related QoS profiles. - Receive MBS service information from AF, directly or indirectly In addition, it may support one or more of the following based on the selected solution(s): - For AF specific priority of group member, the PCF recives from AF and provides towards RAN PDU session policy control information including the MBS Session ID and MBS priority using existing procedure for PDU Session QoS update. |
| MBSF | MSF Control Plane (MSF-C) - Multicast service configuration. - MBS service level management. - xMB-C/MB2-C termination. - Codec configuration (if needed). | May be part of NEF or be deployed independently. - Support TMGI allocation or other MBS signaling for maintaining the signaling distribution tree and the service level management. - Provides an interface to the Application Function or content provider. - Interface to the MBSU. - Perform authorization of the UE to join the MB session. | - Interact with AF and MB-SMF to control MBSU, e.g. if encoding at MBSU is needed. In addition, it may support based on the selected solution(s): - Support TMGI allocation if this is not done by MB-SMF. |
| MBSU | MSF User Plane (MSF-U): - xMB-U/MB2-U termination. - Encoding of data at service layer. - Multicast service layer data packets delivery via N6. | - Handle the payload part to cater for the service level functions and management. - Anchor and forward MBS data in case regionalized deployment with multiple MB-UPFs | - Handling of encoding of MBS data. - Anchor and forward MBS data in case regionalized deployment with multiple MB-UPFs NOTE: The MBSF and the MBSU may be collocated or separated physical nodes. |
| NEF | - 5G MBS service exposure. - Negotiation of 5G MBS service with AF, including QoS, 5G MBS service area | - NEF is an existing NF, which provides interface to the AF - Maintain the MBS signaling tree using MBSF. Managing session requests from the AF. - Support selection of SMFs that have MBS capabilities based on NRF registration. | - Generally being 5G MBS service exposure and specifically providing an interface for 5MBS procedures. - Maintain the MBS signaling tree using MBSF. Managing session requests from the AF. - Negotiation of 5G MBS service with AF, including QoS - Support selection of SMFs that have MBS capabilities NOTE: If separated from the MBSU, the MBSF may be implemented in the NEF. |
| AF | - Support MBS service functions, negotiation with NEF for service exposure. | - Signalling with MB-SMF (via NEF/MBSF) - The AF may provide UE specific priority to the PCF. | - Support MBS service functions, negotiation with NEF for service exposure. - The AF may provide UE specific priority to the PCF. |

### A.X.2	Functional entities

NOTE:	The enhancements of network functions covers commonalities of architecture alternative 1 and 2, and should be further refined based on selected solutions.

#### A.X.2.1	PCF

The PCF is used to provide policy rules for MBS services, and receive MBS service information from AF, directly or indirectly. The PCF performs the following functions to support MBS:

-	Support QoS handling for MBS Session, including QoS parameters like 5QI, MBR, GBR

-	Support AF specific priority for 5MBS group member.

-	Provide policy information regarding the MBS session to MB-SMF for authorizing the related QoS profiles.

-	Receive MBS service information.

#### A.X.2.2	MB-SMF

The MB-SMF is used for managing MBS sessions (including QoS control) and the MBS signalling distribution tree, and for control of MBS transport including configuring the MB-UPF.

#### A.X.2.3	MB-UPF

The MB-UPF is used for delivery of MBS flows to RAN (or UPF) and QoS enforcement for MBS services. The MB-UPF performs the following functions to support MBS:

-	Packet filtering for MBS flows.

-	Distribution of MBS data packets to different RAN nodes (or UPF nodes).

-	Delivery of MBS data packets to RAN via 5GC shared MBS traffic delivery over N3.

-	QoS enforcement (MFBR) and counting/reporting based on existing means.

#### A.X.2.4	AMF

The AMF performs the following functions to support MBS:

-	Selection of MB-SMFs that have MBS capabilities.

-	Signalling with NG-RAN and MB-SMF for managing MBS Sessions and the MBS signalling distribution tree.

#### A.X.2.5	NG-RAN

The NG-RAN is used to receive MBS data packets via shared N3 tunnel and deliver MBS data packets to the UEs using PTP or PTM delivery method. The NG-RAN performs the following functions:

-	Management via N2 of MBS QoS flows via N3

-	Reception of MBS data via shared N3 tunnels.

-	Configuration of UE for MBS QoS flow reception at AS layer.

-	Delivery of MBS data using PTM and PTP.

-	Control switching between PTM and PTP delivery per UE.

-	Support for MB Sessions during Xn Handover and N2 Handovers.

#### A.X.2.6	UE

The UE needs to support the MBS operations and receive MBS data packets from NG-RAN. The UE performs the following functions:

-	Reception of multicast data using PTM /PTP.

-	NAS Signalling for joining and leaving MBS sessions.

-	MBS support at AS layer.

#### A.X.2.7	AF

The AF requests MBS service from the 5GC. The AF does so by

-	providing MBS service information.

-	communicating with NEF for MBS related service exposure.

-	managing MBS session operations towards 5GC.

-	may provide UE specific priority to the PCF.

#### A.X.2.8	NEF

The NEF supports 5MBS service exposure by providing an interface to AFs for 5MBS procedures including service provisioning and MBS session and QoS management..

#### A.X.2.9	MBSF

The MBSF performs the following functions:

-	Interacting with AF and MB-SMF for MBS session operations, for managing signalling distribution tree and MBS transport.

-	Selection of MB-SMF for MBS Session

-	Controlling MBSU if the MBSU is used.

#### A.X.2.10	MBSU

The MBSU performs the following functions:

-	Modification of encoding of MBS data.

-  Media anchor for MBS data traffic if needed.

NOTE: The MBSF and the MBSU may be co-located or deployed separately.

# * * * * End of changes * * * *