SA WG2 Meeting #S2-140E	S2-2005465

19 August - 2 September, 2020, Electronic	(revision of S2-20xxxxx)

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 2422 | rev | - | Current version: | 16.5.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Clarification to RAN sharing options involving NPNs |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Intel |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | Vertical_LAN |  |  |  |  |  | Date: |  |  | 2020-08-13 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | NG-RAN sharing supports all possible scenarios involving NPNs, as indicated in the following sentence in clause 5.18.1: 5G MOCN also supports the following sharing scenarios involving non-public networks, i.e.NG-RAN can be shared by any combination of PLMNs, PNI-NPNs (with CAG), and SNPNs (each identified by PLMN ID and NID). However, the subsequent sentence is a bit misleading because it seems to imply that all combinations are not possible at cell level: In all non-public network sharing scenarios, each Cell Identity is associated with one of the following configuration options: - one or multiple SNPNs; - one or multiple PNI-NPNs (with CAG); or - one or multiple PLMNs only. In fact, this sentence is nevertheless correct because the SIB in a “physical” cell is structured into several parts associated with a “Cell Identity” (which is sort of a “locgical cell” identifier). The SIB information associated with a specific “Cell Identity” can contain a list of either SNPNs or PNI-NPNs or PLMNs. Here are some relevant excerpts from TS 38.331 showing that the top-level SIB IE (CellAccessRelatedInfo) is composed of one plmn-IdentityList IE and one or more npn-IdentityInfoList IEs #### – CellAccessRelatedInfo The IE CellAccessRelatedInfo indicates cell access related information for this cell. CellAccessRelatedInfo information element -- ASN1START -- TAG-CELLACCESSRELATEDINFO-START CellAccessRelatedInfo ::= SEQUENCE { plmn-IdentityList PLMN-IdentityInfoList, cellReservedForOtherUse ENUMERATED {true} OPTIONAL, -- Need R ..., [[ cellReservedForFutureUse-r16 ENUMERATED {true} OPTIONAL, -- Need R npn-IdentityInfoList-r16 NPN-IdentityInfoList-r16 OPTIONAL -- Need R ]] } -- TAG-CELLACCESSRELATEDINFO-STOP -- ASN1STOP This is on the logical cell level for NPN: #### – NPN-IdentityInfoList The IE NPN-IdentityInfoList includes a list of NPN identity information. NPN-IdentityInfoList information element -- ASN1START -- TAG-NPN-IDENTITYINFOLIST-START NPN-IdentityInfoList-r16 ::= SEQUENCE (SIZE (1..maxNPN-r16)) OF NPN-IdentityInfo-r16 NPN-IdentityInfo-r16 ::= SEQUENCE { npn-IdentityList-r16 SEQUENCE (SIZE (1..maxNPN-r16)) OF NPN-Identity-r16, trackingAreaCode-r16 TrackingAreaCode, ranac-r16 RAN-AreaCode OPTIONAL, -- Need R cellIdentity-r16 CellIdentity, cellReservedForOperatorUse-r16 ENUMERATED {reserved, notReserved}, iab-Support-r16 ENUMERATED {true} OPTIONAL, -- Need R ... } -- TAG-NPN-IDENTITYINFOLIST-STOP -- ASN1STOP NPN-IdentityInfo The NPN-IdentityInfo contains one or more NPN identities and additional information associated with those NPNs. Only the same type of NPNs (either SNPNs or PNI-NPNs) can be listed in a NPN-IdentityInfo element. This is on the logical cell level for NPN: NPN-IdentityInfo information element NPN-IdentityInfo-r16 ::= SEQUENCE { npn-IdentityList-r16 SEQUENCE (SIZE (1..maxNPN-r16)) OF NPN-Identity-r16, trackingAreaCode-r16 TrackingAreaCode, ranac-r16 RAN-AreaCode OPTIONAL, -- Need R cellIdentity-r16 CellIdentity, cellReservedForOperatorUse-r16 ENUMERATED {reserved, notReserved}, iab-Support-r16 ENUMERATED {true} OPTIONAL, -- Need R ... } NPN-Identity information element -- ASN1START -- TAG-NPN-IDENTITY-START NPN-Identity-r16 ::= CHOICE { pni-npn-r16 SEQUENCE { plmn-Identity-r16 PLMN-Identity, cag-IdentityList-r16 SEQUENCE (SIZE (1..maxNPN-r16)) OF CAG-IdentityInfo-r16 }, snpn-r16 SEQUENCE { plmn-Identity PLMN-Identity, nid-List-r16 SEQUENCE (SIZE (1..maxNPN-r16)) OF NID-r16 } } CAG-IdentityInfo-r16 ::= SEQUENCE { cag-Identity-r16 BIT STRING (SIZE (32)), manualCAGselectionAllowed-r16 ENUMERATED {true} OPTIONAL -- Need R } NID-r16 ::= BIT STRING (SIZE (44)) -- TAG-NPN-IDENTITY-STOP -- ASN1STOP #### – PLMN-IdentityInfoList The IE PLMN-IdentityInfoList includes a list of PLMN identity information. PLMN-IdentityInfoList information element -- ASN1START -- TAG-PLMN-IDENTITYINFOLIST-START PLMN-IdentityInfoList ::= SEQUENCE (SIZE (1..maxPLMN)) OF PLMN-IdentityInfo PLMN-IdentityInfo ::= SEQUENCE { plmn-IdentityList SEQUENCE (SIZE (1..maxPLMN)) OF PLMN-Identity, trackingAreaCode TrackingAreaCode OPTIONAL, -- Need R ranac RAN-AreaCode OPTIONAL, -- Need R cellIdentity CellIdentity, cellReservedForOperatorUse ENUMERATED {reserved, notReserved}, ..., [[ iab-Support-r16 ENUMERATED {true} OPTIONAL -- Need S ]] } -- TAG-PLMN-IDENTITYINFOLIST-STOP -- ASN1STOP |  |  |  |  |  |  |  |  |
| Summary of change: |  | It is proposed to clarify that when the cell is shared by multiple network types (SNPN, PNI-NPN, PLMN) the system information broadcast needs to be configured with at least as many Cell Identities as there are network types. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Strictly speaking the existing text is not incorrect, but can be very confusing for a reader who is not familiar with the specifics of SIB signalling. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.18.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * * Start of Change * * * *

## 5.18	Network Sharing

### 5.18.1	General concepts

A network sharing architecture shall allow multiple participating operators to share resources of a single shared network according to agreed allocation schemes. The shared network includes a radio access network. The shared resources include radio resources.

The shared network operator allocates shared resources to the participating operators based on their planned and current needs and according to service level agreements.

In this Release of the specification, only the 5G Multi-Operator Core Network (5G MOCN) network sharing architecture, in which only the RAN is shared in 5G System, is supported. 5G MOCN for 5G System, including UE, RAN and AMF, shall support operators' ability to use more than one PLMN ID (i.e. with same or different country code (MCC) some of which is specified in TS 23.122 [17] and different network codes (MNC)) or combinations of PLMN ID and NID. 5G MOCN supports NG-RAN Sharing with or without multiple Cell Identity broadcast as described in TS 38.300 [27].

5G MOCN also supports the following sharing scenarios involving non-public networks, i.e.NG-RAN can be shared by any combination of PLMNs, PNI-NPNs (with CAG), and SNPNs (each identified by PLMN ID and NID).

NOTE 1:	PNI-NPNs (without CAG) are not explicitly listed above as it does not require additional NG-RAN sharing functionality compared to sharing by one or multiple PLMNs.

In all non-public network sharing scenarios, each Cell Identity as specified in TS 38.331[28] is associated with one of the following configuration options:

-	one or multiple SNPNs;

-	one or multiple PNI-NPNs (with CAG); or

-	one or multiple PLMNs only.

NOTE 1a:	One NR Cell Global Identity (NCGI) can correspond to several Cell Identities as specified in TS 38.331[28], which means that any combination of the options listed above can be supported in the cell identified by the same NCGI.

NOTE 2:	Different PLMN IDs (or combinations of PLMN ID and NID) can also point to the same 5GC. When same 5GC supports multiple SNPNs (identified by PLMN ID and NID), then they are not used as equivalent SNPNs for a UE.

NOTE 3:	There is no standardized mechanism to avoid paging collisions if the same 5G-S-TMSI is allocated to different UEs by different PLMNs or SNPNs of the shared network, as the risk of paging collision is assumed to be very low. If such risk is to be eliminated then PLMNs and SNPNs of the shared network needs to coordinate the value space of the 5G-S-TMSI to differentiate the PLMNs and SNPNs of the shared network.

Figure 5.18.1-1: A 5G Multi-Operator Core Network (5G MOCN) in which multiple CNs are 
connected to the same NG-RAN

* * * * End of Change * * * *