SA WG2 Meeting #140E	S2-2005554

Aug 19 – Sep 01, 2020, Elbonia

Source:	OPPO

Title:	KI#2, Sol#24: Update to clarify Federated Learning in 5GC

Document for:	Approval

Agenda Item:	8.1

Work Item / Release:	FS_eNA_Ph2 / Rel-17

Abstract of the contribution:  It is to update solution#24 to clarify how NWDAF architecture applies to Federated Learning.

# 1. Discussion

A practical federated learning is described as Fig.1 below

Fig.1

Step1: Each participant downloads the latest model from server A;

Step2: Each participant uses local data to train the model, encrypts the gradient and uploads it to server A, and server A aggregates the gradient of each user to update the model parameters;

Step3: Server A returns the updated model to each participant;

Step4: Each participant updates its model.

Repeating from step-2 to 4 until reach certain conditions.

To fulfill the federated learning within 5GS, we assume an Analytics Id is corresponding to a Federated learning. Then, participants registers to NRF with the Analytics Id. And the requester can discover the participants when sending to NRF discover_request with the Analytics Id. As shown in Fig.2 below.

Fig.2

Proposal: it is proposed to update solution#24 based on the principle illustrated in the discussion.

# 2. Proposal

The following changes are proposed for solution#24 in TR 23.700-91.

* * * * Start of Change* * * *

## 6.24	Solution #24: Federated Learning among Multiple NWDAF Instances

### 6.24.1	Description

This is a solution for the Key Issue#2: Multiple NWDAF instances and Key Issue #19: Trained data model sharing between multiple NWDAF instances.

#### 6.24.1.1	General

As shown in Figure 6.24.1.1-1, multiple NWDAF will be deployed in a big PLMN, so maybe it is difficult for NWDAF to centralize all the raw data that are distributed in different Areas. However, it is desired or reasonable for the NWDAF distributed in an Area to share its model or data analytics with others NWDAFs.

Figure 6.24.1.1-1: Hierarchical NWDAF deployment in a PLMN

Federated Learning (also called Federated Machine Learning) could be a possible solution to handle the issues such as data privacy and security, model training efficiency, and so on, in which there is no need for raw data transferring (e.g. centralized into single NWDAF) but only need for model sharing. For example, with multiple level NWDAF architecture, NWDAFs may be co-located with an 5GC NF (e.g. UPF, SMF), and the raw data cannot be exposed due to privacy concerns and performance reasons. In such case, the federated learning will be a good way to let a Server NWDAF coordinate with multiple localized NWDAFs to complete a machine learning.

The main idea of Federated Learning is to build machine-learning models based on data sets that are distributed in different network functions. A Client NWDAF (e.g. deployed in a domain or network function) locally trains the local ML model with its own data and share it to the server NWDAF. With local ML models from different Client NWDAFs, the Server NWDAF could aggregate them into a global or optimal ML model or ML model parameters and send them back to the Client NWDAFs for inference.

This solution tries to involve the idea of Federated Learning into the NWDAF-based architecture, which aims to investigate the following aspects:

-	Registration and discovery of multiple NWDAF instances that support Federated Learning;

-	How to share the ML models or ML model parameters during the Federated Learning training procedure among multiple NWDAF instances.

#### 6.24.1.2	General procedure for Federated Learning among Multiple NWDAF Instances

Figure 6.24.1.2-1: General procedure for Federated Learning among Multiple NWDAF Instances

1~3. Client NWDAF registers its NF profile into NRF.

In the Register request, Client NWDAF reports the Analytics Ids it supports. The analytics Id can be corresponding to a kind of federated learning, which may include forward and backward propagation for gradient.

4~6. Server NWDAF discovers one or multiple Client NWDAF instances which could be used for Federated Learning via the NRF to get IP addresses of Client NWDAF instances.

In the Register request, the Server NWDAF should report the requested Analytics Id, which is for Federated learning.

It is assumed an Analytics Id is preconfigured for a certain kind of Federated Learning. Thus, the NRF can realize the Server NWDAF is requesting to perform federated learning based on the pre-configuration. And the NRF responds to the central NWDAF the IP address of multiple NWDAF instances which support the Analytics Id.

NOTE: it is based on pre-configuration for how many NWDAF instances NRF can notify to central NWDAF to perform federated learning.

7a: Each client NWDAF communicates licensing conditions for its data and training infrastructure to participate in the federated learning task. These conditions can be based on policies set-up based on how sensitive the data is, how much compute is expected to be needed to perform local training, who will get the use the trained model, how revenues obtained through the use of the model will be shared, etc.

7b: Based on the response from NRF, Server NWDAF selects which NWDAF clients will participate based on its desired license model and.

7c. Server NWDAF sends analytics request to the selected Client NWDAFs that participate in the Federated learning according to steps 7a and 7b including some parameters (such as data type list, maximum response time window, etc.) to help the local model training for Federated Learning.

Editor's note: 	How to realize the interface and parameters between Server NWDAF and Client NWDAF is FFS.

8. The client NWDAF, e.g. co-located with 5GC NF, can use the local data to perform the model training (e.g. forward propagation). Each Client NWDAF collects its local data by using the current mechanism in clause 6.2, TS 23.288 [5].

9. During Federated Learning training procedure, each Client NWDAF trains local ML model based on its own data and the parameters (AI model) received from Server NWDAF, and reports the analytics result to Server NWDAF, such as the gradient calculated by Backward propagation.

10. The Server NWDAF aggregates all the local ML model information such as the gradient to update the AI model.

11. The Server NWDAF sends the aggregated ML model information (updated AI model) to each Client NWDAF for next ground model training.

12. Each Client NWDAF then updates the local ML model based on the aggregated model information (updated AI model) distributed by the Server NWDAF.

NOTE: The step 9~12 should be repeated until the training termination condition (e.g. maximum number of iterations, or the result of loss function is lower than a threshold) is reached.

13. After the training procedure is finished, the globally optimal ML model or ML model parameters could be distributed to the Client NWDAFs for the inference.

#### 6.24.1.3	Procedure for usage of Federated Learning in Abnormal Behaviour

Figure 6.24.1.3-1: Procedure for usage of Federated Learning in Abnormal Behaviour

1. A Server NWDAF determines to train an Abnormal Classifier using the input data defined in Table 6.7.5.2-1, TS 23.288 [5]). The Server NWDAF discovers two Client NWDAFs i.e. Client NWDAF 1 in Area A, Client NWDAF 2 in Area B that are participate in the license model as defined in steps 7 in figure 6.24.1.2-1.

2. Client NWDAF 1 and Client NWDAF 2 collect raw data from SMF and AMF, respectively. The data collection procedures from AMF and SMF are defined in step 2~3, clause 6.7.5.4, TS 23.288 [5].

3~4. Client NWDAF 1 and Client NWDAF 2 collect Exception information from AF, respectively. The Exception information is defined in clause 6.7.5.2, TS 23.288 [5].

Repeat from step 5-8 until the training termination condition (e.g. maximum number of iterations) is fulfilled:

5. During Federated Learning training procedure, based on the local raw data, Client NWDAF 1 and Client NWDAF 2 locally train local Abnormal Classifier, respectively. Or, Client NWDAF 1 and Client NWDAF 2 updates the local Abnormal Classifier based on the aggregated Abnormal Classifier information distributed by the Server NWDAF in step 7.

6. Client NWDAF 1 and Client NWDAF 2 reports the local Abnormal Classifier information (e.g. volume of the local dataset, parameters of the Abnormal Classifier) to the Server NWDAF.

7. Server NWDAF aggregates all the local Abnormal Classifier information to derive the aggregated Abnormal Classifier information.

8. Server NWDAF sends the aggregated Abnormal Classifier information to Client NWDAF 1 and Client NWDAF 2.

9. After the training procedure is finished, the globally optimal Abnormal Classifier or Abnormal Classifier parameters could be distributed to the Client NWDAF 1 and Client NWDAF 2 for the inference.

### 6.24.2	Impacts on Existing Nodes and Functionality

NWDAF:

-  Registration into NRF;

-  Pre-configuration of Analytics Id for a federated learning

As a server NWDAF,

-	Discover request to NRF with the analytics id corresponding to a federated learning

-       Client NWDAF instances discovery;

-	Initial Federated Learning parameters determination and distribution;

-  Collection of local ML model information from the Client NWDAF instances;

-	Aggregates local model information to update AI model and distribution to the Client NWDAF.

As a Client NWDAF,

-	register to NRF with the analytics id correponding to a federated learning

-       Local dataset collection;

-	Local model training, e.g. for gradient calculation, and reporting to the Server NWDAF;

-	Receive aggregated model information (updated AI model) from Server NWDAF and perform local ML model update.

- 	Report the licensing conditions.

NRF:

-	Server NWDAF and Client NWDAF registration and discovery.

* * * * End of Change * * * *