3GPP TSG-SA WG2 Meeting #140E 	S2-2005589

19 August – 2 September, 2020, Elbonia	 (revision of S2-200xxxx)

Source:	CATT

Title:	KI #3, Sol#22: Update the solution to add SMBR event notification procedure

Document for:	Approval

Agenda Item:	8.4

Work Item / Release:	FS_eNS_Ph2 / Rel-17

Abstract of the contribution: this paper proposes to update solution #22 in TR 23.700-40.

Discussion

This is proposed to update solution#22 with following aspects:

- Add a new procedure to support SMBR event notify.

- Update the impacts clause.

Proposal

It is proposed to update solution #22.

First change

6.22.3.x	SMBR event notification

Figure 6.22.3.x-1 SMBR event notification

0.	The AF sends Npcf_PolicyAuthorization_Subscribe to PCF to subscribe the event that the SMBR of UE reaches a threshold.

1.	The PCF sends Namf_EventExposure_Subscribe to AMF to subscribe the event that the SMBR of UE reaches a threshold.

2.	The AMF sends N2 message to RAN to subscribe the event that the SMBR of UE reaches a threshold.

3.	The RAN detects that the SMBR reaches the threshold.

4.	The RAN sends N2 message including S-NSSAI and indication of SMBR threshold reached to AMF.

5.	The AMF sends Namf_EventExposure_Notify (S-NSSAI, SUPI, SMBR threshold reached) to PCF.

6.	The PCF sends Npcf_PolicyAuthorization_Notify to AF to notify the event that the SMBR of UE reaches the threshold.

Second change

6.22.4	Impacts on services, entities and interfaces

AMF:

-	Support Subscribed SMBR transfer during initial registration via Nudm_SDM Service.

-	Support SMBR transfer via N2 interface and SMBR enforcement.

-	Support SMBR event notify.

UDM:

-	Support Subscribed SMBR transfer during initial registration via Nudm_SDM Service.

PCF:

-	FFS: Support authorized SMBR transfer via Npcf_AMPolicyControl Service.

-	Support SMBR event notify.

RAN:

-	Support SMBR transfer via N2 interface and SMBR enforcement.

-	Support SMBR event notify when the SMBR reaches a threshold.

End of changes