3GPP TSG-SA WG2 Meeting #140E 	S2-2005589

19 August – 2 September, 2020, Elbonia	 (revision of S2-200xxxx)

Source:	CATT

Title:	KI #3, Sol#22: Update the solution to add SMBR event notification procedure

Document for:	Approval

Agenda Item:	8.4

Work Item / Release:	FS_eNS_Ph2 / Rel-17

Abstract of the contribution: this paper proposes to update solution #22 in TR 23.700-40.

Discussion

This is proposed to update solution#22 with following aspects:

- Add a new procedure to support SMBR event notify.

- Update the impacts clause.

Proposal

It is proposed to update solution #22.

First change

6.22.3.x	SMBR event notification

Figure 6.22.3.x-1 SMBR event notification

1.	The AF or the PCF subscribes the event of the SMBR reached. The AF subscribes the event via NEF using step 1a. The PCF subscribes the event using step 1b.

2.	The AMF sends N2 message to RAN to subscribe the event that the SMBR of UE is reached.

3.	The RAN detects that the SMBR is reached.

4.	The RAN sends N2 message including S-NSSAI and indication of SMBR is reached to AMF.

5.	The AMF sends event notification to AF via NEF or to PCF. The PCF may decide to update UE policy to steer the UE traffic in the slice to N3GPP access.

Second change

6.22.4	Impacts on services, entities and interfaces

AMF:

-	Support Subscribed SMBR transfer during initial registration via Nudm_SDM Service.

-	Support SMBR transfer via N2 interface and SMBR enforcement.

-	Support SMBR event notify.

UDM:

-	Support Subscribed SMBR transfer during initial registration via Nudm_SDM Service.

PCF:

-	FFS: Support authorized SMBR transfer via Npcf_AMPolicyControl Service.

-	Support SMBR event notify.

RAN:

-	Support SMBR transfer via N2 interface and SMBR enforcement.

-	Support SMBR event notify when the SMBR is reached.

End of changes