3GPP TSG-SA WG2 Meeting #140E 	S2-2005589

19 August – 2 September, 2020, Elbonia	 (revision of S2-200xxxx)

Source:	CATT

Title:	KI #3, Sol#22: Update the solution to add SMBR event notification procedure

Document for:	Approval

Agenda Item:	8.4

Work Item / Release:	FS_eNS_Ph2 / Rel-17

Abstract of the contribution: this paper proposes to update solution #22 in TR 23.700-40.

Discussion

This is proposed to update solution#22 with following aspects:

- Add a new procedure to support SMBR event notify.

- Update the impacts clause.

Proposal

It is proposed to update solution #22.

First change

6.0	Mapping Solutions to Key Issues

Table 6.0-1: Mapping of Solutions to Key Issues

| Solution#'s | Solution Titles | Key Issue#'s |
| --- | --- | --- |
| 1 | PCF measurement based Network Slice SLA control for Maximum Number of UEs parameter | 1 |
| 2 | Max number of UEs per Network Slice control at registration | 1 |
| 3 | AMF/NSSF based counting of UEs in a Network Slice | 1 |
| 4 | NWDAF enhancements for supporting of network slice quota on the maximum number of UEs | 1 |
| 5 | NWDAF enhancements for supporting of network slice quota on the maximum number of PDU Sessions | 2 |
| 6 | PCF-based counting of PDU Sessions in a Network Slice | 2 |
| 7 | Support of Network Slice SLA for Maximum Number of PDU sessions parameter | 2 |
| 8 | AMF and O&M based solution | 1, 2 & 4 |
| 9 | Monitoring multiple quotas of number of UEs/PDU Sessions per S-NSSAI at NWDAF | 1, 2 & 4 |
| 10 | Max number of PDU Sessions per Network Slice control via NSQ function | 2 |
| 11 | Handling maximum number of sessions using NF status | 2 |
| 12 | NSQ assisted dynamic adjustment of data rate per slice via NAS signaling | 5 |
| 13 | Limitation of data rate per network slice in UL and DL per UE | 3 |
| 14 | UE-Slice-AMBR adjustment to meet the limitation of data rate per Network Slice | 5 |
| 15 | Using Back-off timer | 1 |
| 16 | Slice data rate enforcement and dynamic adjustment | 5 |
| 17 | Support of radio spectrum attribute by CN assisted RAN control | 7 |
| 18 | Proactive Slice Quota Management in AMF | 1, 2, 4, 5 |
| 19 | Support of network slice quota control and enforcement | 1, 2 & 5 |
| 20 | Reusing existing QoS model to ensure that to limit the Maximum throughput UL/DL in a Network slice is not exceeded | 3 & 5 |
| 21 | Limitation of data rate per network slice in UL and DL per UE without RAN involvement | 3 |
| 22 | Solution on limitation of data rate per Network Slice in UL and DL per UE | 3 |
| 23 | Network slice quota event notification | 4 |
| 24 | NSQ assisted dynamic adjustment of data rate per slice via user plane adjustment | 5 |
| 25 | Enforcement of MBR UL/DL per S-NSSAI | 5 |
| 26 | Network controlled enforcement of simultaneous usage of network slices based on user preference | 6 |
| 27 | Network slices simultaneous usage incompatibility support | 6 |
| 28 | Constraints on simultaneous use of the network slice | 6 |
| 29 | Providing Operating Band Information in the Configured NSSAI | 7 |
| 30 | Preferred frequency bands in Configured NSSAI | 7 |
| 31 | Steering the UE to a network slice in a different frequency band | 7 |
| x | SMBR event notification | 4 |

Second change (all new text)

6.X	Solution #X: SMBR event notification

6.X.1	Introduction

This solution mainly addresses the key issue #4.

6.X.2	High-level Description

The Solution is to notify the SMBR related event.

6.X.3	Procedures

Figure 6.x.3-1 SMBR event notification

1.	The AF or the PCF subscribes the event of the SMBR reached. The AF subscribes the event via NEF using step 1a. The PCF subscribes the event using step 1b.

2.	The AMF sends N2 message to RAN to subscribe the event that the SMBR of UE is reached.

3.	The RAN detects that the SMBR is reached.

4.	The RAN sends N2 message including S-NSSAI and indication of SMBR is reached to AMF.

5.	The AMF sends event notification to AF via NEF or to PCF. The PCF may decide to update UE policy to steer

6.x.4	Impacts on services, entities and interfaces

AMF:

-	Support SMBR event notify.

PCF:

-	Support SMBR event notify.

RAN:

-	Support SMBR event notify when the SMBR is reached.

End of changes