3GPP TSG SA WG2 Meeting #140-e		S2-2005689

Electronic, 19 Aug – 02 Sep, 2020

Title:	LS on System support for Multi-USIM devices

Response to:

Release:	Rel-17

Work Item:	FS_MUSIM

Source:	SA2

To:	RAN2, RAN3

Cc:	-

Contact person:	Sašo Stojanovski

saso stojanovski intel com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	None

# 1	Overall description

SA2 would like to inform RAN2 and RAN3 that they have progressed the study on FS_MUSIM (TR 23.761) and have agreed several interim conclusions on all three key issues. The interim conclusions list a set of principles that SA2 will consider for further study.

SA2 point out that these interim conclusions are subject to further revision taking into account feedback received from other WGs.

The table below lists the principles that are considered for further study and have RAN impact or require RAN feedback. The table also contains the applicability to key issues in TR 23.761, as well as questions to RAN WGs.

| Solution principle for further study in SA2 | Applicability to Key Issues | Question to RAN WGs |
| --- | --- | --- |
| Paging Cause (as in Solution #1) | KI#1. | Q1: Please confirm feasibility of sending a Paging Cause in [Uu] Paging message for both EPS and 5GS. [RAN2, RAN3] |
| Busy indication (as in Solution #3) | KI#1. Nice to have. | Q2: Please confirm feasibility for sending an RRC Busy Indication when the UE determines that it cannot respond to the paging. Please provide any feedback about complexity (if applicable). [RAN2, RAN3] |
| RRC-based leaving.and returning (as in all KI#2 solutions) with the following assumptions: - the leaving is initially considered as “short absence duration” i.e. the UE does not need to indicate the time duration of the absence. If the UE does not return within a network-controlled time interval, the “short absence duration” is automatically converted into a “long absence duration”. The RRC states corresponding to “short” and “long” duration are to be determined by RAN WGs. - The UE can use the negotiated absence to perform a MO procedure (e.g. periodic mobility registration, keep-alive message, etc.) or a MT procedure (e.g. pick-up an SMS, inspect a MT service invite, respond to a network-initiated C-plane procedure, etc.) in the other network. - It is assumed that the RRC request for leaving is explicitly acknowledged by the network. However, in the absence of network acknowledgement the UE can still proceed with leaving. | KI#1, KI#2. | Q3: Please confirm feasibility of the RRC-based leaving and returning procedure in 5GS. [RAN2, RAN3] Q4: SA2 expects that RRC-based leaving would also apply to 5GS/E-UTRA (Option 5), but there were concerns that this may not be in scope of the RAN work item. Please confirm whether changes to 5GS/E-UTRA (Option 5) to support this functionality are in scope of the RAN work item. [RAN2, RAN3] |

SA2 would also like to point out that TR 23.761 also contains several solutions to KI#2 requiring  RAN’s feedback. The solution principles in these solutions can be categorized as follows:

-	UE requested 5G-GUTI reassignment using the Mobility Registration Update procedure (as in Solutions #14 and #20)

-    Changes related to the UE_ID that is used for calculation of PF/PO:

-    Calculation of PF/PO by using an Alternative UE ID (different from UE’s GUTI) (Sol#15)

-    Calculation of PF/PO by using the UE Identity Index Value offset value negotiated between UE and MME (Sol#16)

-    Calculation of PF/PO based on MUSIM Assistance Information (Sol#17)

-     Paging on consecutive POs as in Sol#18. For this solution SA2 was not clear whether the network requires an explicit MUSIM capability so that a different paging strategy can be applied to MUSIM- and non-MUSIM capable devices.

-	Implementation-based solution to address overlapping POs (Sol#19).

-	Access Stratum-based solution with scheduling gap (Sol#21).

Q5: SA2 would like to ask RAN2 whether Solution #14-#20 are feasible for effective avoidance of paging collision between NR and NR/other RAT, or between EUTRA and EUTRA.

Q6:  SA2 would like to ask RAN2 whether Solution #15-#19 are compliant with the assumption “no EUTRA related impact”? And SA2 also would like RAN2 and RAN3 to clarify the exact meaning of “no EUTRA related impact”, e.g. whether it include the impact on S1 interface and UE behaviour defined in 36.304.

Q7: SA2 would like to ask RAN2 and RAN3 to take these solution into consideration and provide any other feedback or recommendation on the applicability of these solutions, or any other Access Stratum-based solutions for KI#2.

# 2	Actions

To RAN2, RAN3

ACTION: 	SA2 kindly asks RAN2 and RAN3 to take into consideration the information above and provide answers to questions Q1, Q2Q3, Q4, Q6and Q7.

To RAN2

ACTION: 	In addition, SA2 kindly asks RAN2 to provide answers to questions Q5.

# 3	Dates of next TSG SA WG 2 meetings

SA2#141E		12-23 October				Electronic meeting

SA2#142E		16-20 November			Electronic meeting