SA WG2 Meeting #S2-140e	S2-2004914r02

19 August - 2 September, 2020, Electronic

Source:	Ericsson

Title:	KI #2, sol 10: Updates on static forwarding solution

Document for:	Agreement

Agenda Item:	8.5

Work Item / Release:	FS_IIoT/ Rel-17

Abstract of the contribution: The paper addresses the outstanding editor’s notes for the solution.

# Discussion

We update the solution to address the Editor’s notes. The paper also renames the solution since it now focuses on one aspect of the UE to UE communication, namely the CNC provided static forwarding rules in all directions. The updated name reflects that aspect. Note that the solution could be combined with other solutions.

The solution in itself does not address other aspects related to the delay model, the determination of ports for UE to UE traffic or QoS aspects. It is clarified that those aspects are out of scope of the solution, and the related editor’s note is removed.

Other notes are clarified as discussed below.

Editor's note:	The co-existence with 5G VN mechanism is FFS.

The solution can co-exist with some aspects of the 5G VN mechanism, such as group management, and the setup of N19 tunnels.

Centrally managed TSN networks require the capability of the CNC to set static forwarding rules in the bridges of the network. This allows the CNC to set the path for the TSN streams, as well as control the forwarding for non-TSN traffic. Since CNC can centrally set up forwarding rules in the UPFs, there is no need for the SMF to also control the forwarding. In fact, SMF controlled forwarding rules in the PDRs could conflict with CNC provided forwarding rules. There needs to be only one central controller in a given network, and in our case CNC is the central controller. That is why there is no need for SMF provided forwarding rules in the form of PDRs.

Editor's note:	It is FFS how to support the logical port, the related N4 Session binding mechanism and the coexistence with CUPS mechanism. It is FFS how to differ the logical port and physical Ethernet port.

Editor's note:	It is FFS how to enable binding between logical port and PDU Session.

The solution does not restrict how the UPF binds the PDU Sessions to bridge ports in the implementation. One possible implementation could be to assign an interface identifier to the PDU Sessions as well as to the bridge ports and tag the packets with the interface identifier to realize the binding. Other implementation options are also possible. The implementation of how this binding is done is not visible outside of the UPF, hence it does not impact the N4 sessions. By binding the PDU sessions to bridge ports, the PDU Sessions act as logical ports, which act the same way as physical ports from the point of view of bridge forwarding.

# Proposal

It is proposed to update the solution in 23.700-20 as follows.

* * * * Start change * * * *

## 6.X	Solution #X Static forwarding rules to all directions

### 6.X.1	Introduction

This solution addresses the traffic forwarding aspect of Key Issue #2: UE-UE TSC communication. From a 3GPP system perspective, UE-UE communication is a special case as that requires the UPF to be able to forward traffic between UEs without going through a NW-TT. From the TSN network perspective, however, the UE-UE communication is not special; as the 5GS models a TSN bridge, it just corresponds to communication between two ports. Currently, static forwarding rules can only be configured by the CNC for the uplink direction in the UPF/NW-TT to select between the NW-TT ports. However, centrally managed TSN networks require the capability of configuring static forwarding rules in any direction so that the CNC can set up the TSN streams according to the delay requirements. Note also that centrally configured static forwarding rules are applicable also to non-TSN traffic.

This solution generalizes the forwarding capability 5GS bridge model, so that is becomes capable of TSC communication between any two ports. That includes communication between a DS-TT port and a NW-TT port, as already supported, but also extended to include communication between two DS-TT ports (i.e., UE to UE communication), as well as communication between two NW-TT ports. The solution makes the 5GS model comply to IEEE bridging functionality by including the Bridging forwarding function as a single entity which can then realizes the needed forwarding functions. In this way the 5GS provides a general framework that can apply to all cases, while vendors are free to implement the bridge forwarding function however they want.

The solution is illustrated in the figure 6.x.1-1 below. The bridge forwarding inside the NW-TT realizes Ethernet bridging functionality and makes use of the static forwarding rules to any port as provided by the CNC. The PDU Sessions are bound to the bridge forwarding function as logical ports, which act in the same way as physical ports from the point of view of forwarding. In case of downlink traffic, the bridge forwarding functionality determines which PDU Session to use. N4 rules configured by the SMF are still possible to use according to the current specification, e.g. for QoS enforcement or usage reporting; i.e., the CUPS mechanism remains as specified today. But as the bridge forwarding mechanism already determines the PDU Session to use, there is no need to set up N4 rules for selecting the downlink PDU Session.

NOTE:	for the impact on PDU session lookup is to be evaluated.

Figure 6.x.1-1 Static forwarding rule setup

This solution focuses only on the forwarding aspect, and hence it does not address questions related to the delay model and QoS.

### 6.X.2	Functional Description

- The CNC has the possibility to configure static forwarding rules into the bridging function within the NW-TT. Static forwarding rules may be configured for forwarding between any two ports in any direction, including the possibility of static forwarding between two DS-TT ports (UE to UE) or between two NW-TT ports. The possibility of static forwarding rules makes the 5GS logical bridge act similarly as TSN Ethernet bridge where centrally controlled forwarding rules are expected to be supported. This allows the CNC to explicitly establish the traffic forwarding path using the destination MAC address and VLAN combination of the TSN stream, even that MAC address/VLAN combination is different from what is used for non-TSN traffic from the given host which is observed by MAC learning. 
NOTE:	Configuring/updating static forwarding rules between two NW-TT ports in a UPF using the Rel-16 BMIC mechanism, which is based on PDU-session related signaling, is only supported as long as at least one PDU session for TSN is established towards the related UPF. This solution focuses on the forwarding aspect, and the mechanism used to convey the forwarding rules to the UPF is out of scope of this solution.

- For the purposes of static forwarding in the downlink in the NW-TT, the PDU Sessions correspond to bridge ports. When static forwarding dictates that a frame is to be forwarded on a given port, the NW-TT binds the port to the given PDU Session. How this binding is realized is implementation specific. The UPF does not need to install additional filtering in the packet detection rules (PDRs) to determine which PDU Session to select for the downlink traffic once the port is already determined by the static forwarding rules. Therefore, the PDRs may contain e.g., match-all filters or filters that correspond to the appropriate QoS or other rules; the downlink traffic that is offered to the PDR in the given N4 session corresponding to the PDU Session does not include the traffic that goes to another port as determined by the bridging function in the NW-TT.

Note that the solution does not restrict how the UPF binds the PDU Sessions to bridge ports in the implementation. One possible implementation could be to assign an interface identifier to the PDU Sessions as well as to the bridge ports and tag the packets with the interface identifier to realize the binding. The N4 rules are extended within the UPF automatically to map packets marked with the interface identifier to the given PDU session. Other implementation options are also possible. The implementation of how this binding is done is not visible outside of the UPF, hence it does not impact the N4 sessions. By binding the PDU sessions to bridge ports, the PDU Sessions act as logical ports, which act the same way as physical ports from the point of view of bridge forwarding.

Editor's note:	It is FFS how to manage the interface identifier in 5GS and how to maintain the binding between PDU session or N19 tunnel, interface identifier and (logical/physical) bridge port.

The solution realizes the bridge forwarding in a single logical function. It is up to the implementation to what extent that logical function makes use of PDR/FAR rules, and how the bridge forwarding implementation is integrated with the existing N4 rules.

- The solution assumes any two DS-TT ports are allowed to communicate, as determined by the CNC/CUC.

- The solution does not rely on 5G VN mechanism. The solution can co-exist with some features of the 5G VN mechanism, such as group management and the setup of N19 tunneling. An N19 tunnel may be bound to a port of the bridge forwarding function similarly as PDU sessions are bound to a port. As the centrally managed TSN network use static forwarding rules provided by the CNC, there is no need for using SMF provided PDRs for mapping downlink traffic to PDU Sessions; instead the selection of the downlink PDU Session is performed in the bridge forwarding function as described above.

### 6.X.3	Procedures

The following new or adjusted procedures are needed for the solution.

- The scope of the possible static forwarding rules is extended as these are currently specified for uplink traffic from the UEs to the NW-TT ports. This solution allows static forwarding rules to be provided between any two ports of the 5GS Bridge. As already specified, the CNC may provide static forwarding rules to the TSN AF, which includes the static forwarding rules in the Bridge Management Information Container that is forwarded to the NW-TT that implements the forwarding accordingly. When implementing the forwarding rules, the bridging function in the NW-TT binds PDU Sessions to logical ports so that no additional filtering in the PDRs are needed to determine which PDU Session to select for the downlink once the port is already determined by the static forwarding rules that are part of the bridging function in the NW-TT.

### 6.X.4	Impacts on existing services and interfaces

NW-TT:

- The NW-TT implements static forwarding rules not only in the uplink direction from a UE to the NW-TT ports, but also between any two ports.

- PDU Sessions are bound to the logical ports in the NW-TT bridging function so that when the  bridge forwarding rules dictate that a frame is forwarded on a port corresponding to a PDU Session, no additional filtering is needed in the PDRs to send the frame on the given PDU Session. It is implementation specific how the bridge forwarding function is realized and how the binding between the bridge ports and the PDU Sessions are implemented within the UPF.

SMF:

- Do not provide PDR filter conditions for mapping downlink traffic to PDU sessions

* * * * End change * * * *