SA WG2 Meeting #140-E	S2-2005969

Aug 19 – Sep 01, 2020, Electronic	(revision of S2-2005249r05)

Source:	vivo, OPPO, Xiaomi

Title:	KI#1, Sol#16: Updates to add impact

Document for:	Approval

Agenda Item:	8.3

Work Item / Release:	FS_enh_EC / Rel-17

Abstract of the contribution: This paper proposes some impacts to solution #16.

1 Discussion

In solution #16 of TR 23.748, it proposes that the UE gets Edge Configuration Server Information carried in the PCO from the SMF during the PDU Session Establishment procedure.

By using NAS to carry ECS info, the impacts on UE OS kernel (Android) is inevitable, UE OS kernel has to design new standard interface (such API) for functional invocation and adaptation. At least for now, the information from NAS is not allowed to communicated with application client directly. These impacts need to be reflected.

2 Proposal

It is proposed to add the following solution in TR 23.748.

# * * * * Start change * * * *

6.16.2.1	Functional Description

Edge Configuration Server Information may be provided by the SMF to the UE during the PDU Session Establishment procedure in the PCO. During PDU Session Establishment, the UE may provide an indication of Requesting Edge Configuration Server Information to retrieve the Edge Configuration Server Information.

Editor's note:	The need for an indication of Requesting Edge Configuration Server Information is FFS.

The SMF may derive the Edge Configuration Server Information based on local configuration, the UE's location, and/or UE subscription information received from the UDM. (The UE's subscription information may include identities of Edge Configuration Servers that the UE may access). If the Edge Configuration Server Information changes (e.g. due to local configuration change, or a change of UE location), the SMF may use the PDU Session Modification procedure to send the updated Edge Configuration Server Information to the UE in the PCO.

Editor's note:	The need to get ECS information from UDM/UDR is FFS.

Editor's note:	The HR case needs further clarification.

The Edge Configuration Server Information sent to the UE in the PCO corresponds to the IP address (and possibly port) of the ECS and/or to the FQDN of the ECS.

# * * * * Next change * * * *

### 6.16.4	Impacts on services, entities and interfaces

UE:

-	Provides an indication of Requesting Edge Configuration Server Information to the AMF during Registration.

-	Receives Edge Configuration Server Information from the AMF during the (re-)Registration and UE Configuration Update procedures.

-	UE may be provided with the ECS IP address and/or ECS FQDN and requires enhancement to the UE (e.g. OS or OS and AT Commands) to support delivery the ECS IP address or FQDN received in NAS signalling to the EEC inside the UE.

UDM/UDR (if the alternative to have ECS address stored in UDM/UDR is pursued):

-	A UE's subscription information may include Edge Configuration Server Information (i.e. one or more FQDNs and/or IP Address(es) of Edge Configuration Servers).

SMF:

-	May receive an indication of Requesting Edge Configuration Server Information from the UE during PDU Session Establishment.

-	Sends Edge Configuration Server Information to the UE during the PDU Session Establishment procedures.

-	Receives Edge Configuration Server information from the UDM/UDR during PDU Session Establishment (when Nudm_SDM_Get is invoked) (if the alternative to have ECS address stored in UDM/UDR is pursued).

# * * * * End of changes * * * *