3GPP TSG-WG SA2 Meeting #140E	S2-2006303

Aug 19 – Sep 01, 2020, Elbonia	(revision of S2-205358r01)

Source:	China Mobile, Huawei, CATT, Academy of Broadcasting Science

Title:	KI #1, Sol #8: Clarification on MBS Solution

Document for:	Approval

Agenda Item:	8.9

Work Item / Release:	FS_5MBS/Rel-17

Abstract: This contribution proposes clarification on Solution #8.

1. Introduction

This contribution proposes clarification on Solution #8.

The AF may trigger to change delivery mode from unicast to multicast for a certain multicast group, and this contribution intends to clarify the related procedures for completion of the MBS session establishment, and most of steps in Solution #3 can be reused.

2. Text Proposal

It is proposed to capture the following clarification in TR 23.757.

* * * * Start of Changes * * * *

6.8	Solution #8: Dynamic multicast/broadcast transmission

6.8.1	Functional Description

6.8.1.1	General

This solution addresses KI#1, and compared with other solutions (e.g. Solution #3) to KI#1, this solution mainly addresses the following aspects:

-	When and how to trigger the establishment of an MBS session?

The trigger may be based on network analysis result. The AF subscribes or requests the network analytics information from the NWDAF as specified in TS 23.288 [9]. Based on the analytics information, e.g. Observed Service Experience analytics defined in clause 6.4 of TS 23.288 [9], the AF decides/updates multicast related service parameters. The AF may use service specific parameter provision procedure as specified in TS 23.502 [8] to provide multicast related service parameters, including the multicast indication for different services/applications, to the PCF.

Based on multicast related service parameters, the PCF provides the multicast service related policy in PCC rules to the SMF. Then the SMF decides when to establish an MBS session for the services and UEs based on the multicast service related policy. The following procedures for completion of the MBS session establishment can refer to Solution #3.

6.8.1.2	System Architecture

The architecture functional entities are described in clause A.1 "5G MBS system architecture based on unicast 5GC". The functionalities of SMF/UPF in Figure 6.8.2.1-1 are equal to the ones of SMF1/UPF1 respectively in Figure 6.3.2.1-1.

6.8.2	Procedures

6.8.2.1	Dynamic multicast session setup/modification

Figure 6.8.2.1-1: multicast session/tunnel setup/modification

The key steps of the procedure for this solution are as follows:

1.	The step 1-3 specified in clause 6.3.2.1 are reused.

2.	The AF decides to update service parameters, including the multicast indication for different services/ applications. For example, the AF may subscribe or request network analytics information to the NWDAF as specified in TS 23.288 [9], e.g., Observed Service Experience defined in clause 6.4 in TS 23.288 [9], and based on the analytics information, the AF may decide to update service parameters.

The AF provides update service parameters to the PCF (e.g. via the UDR as specified in TS 23.502 [8] clause 4.15.6.7). The AF may further request UDR to store the following information for the multicast service: UE number of requesting the service as a threshold, the list of UE IDs (i.e. GPSIs), or UE group ID for receiving multicast service, and other service information, such as IP multicast address.

The AF can decide to subscribe or request network analytics information based on local configuration, or triggered by e.g. event report from the NEF.

3.	The PCF provides to the SMF(s) the multicast service related policy based on the service parameters, including multicast indication for specific service (represented by Service Description).

4.	The step 6-34 specified in clause 6.3.2.1 is reused with the following difference:

-	The SMF(s) initiates a PDU Session Modification procedure to include the UE(s) listed in the list of UE IDs in multicast reception.

NOTE X:	This is not preventing other possible authorization methods.

-	The SMF(s) may configure the UPF(s) for not sending the data via unicast PDU session any longer. It might be used for the case e.g., the external network uses IP multicast.

NOTE Y:	In application layer, the AF may indicate the UE that delivery mode is changed from unicast to multicast.

6.8.3	Impacts on services, entities and interfaces

In addition to what is specified in clause 6.3.3, the impacts also include the followings:

AF:

-	The AF needs to support the processing of multicast. The AF determines to sends to 5GC the service parameters for multicast based on e.g., the network analytics information provided by 5GC.

UDR:

-	The UDR is enhanced to store the multicast information, e.g., UE number of requesting the service as a threshold, the list of UE IDs (i.e. GPSIs), or UE group ID for receiving multicast service, and other service information, such as IP multicast address.

PCF:

-	The PCF is enhanced to provide to the SMF(s) the multicast service related policy.

SMF:

-	The SMF(s) may configure the UPF(s) for not sending the data via unicast PDU session;

-	The SMF handling multicast transmission may authorize the UE's join request based on the information provided by AF.

* * * * End of Changes * * * *