SA WG2 Meeting #141-e 	S2-2006952

12 Oct – 23 Oct 2020, Online	(Revision of S2-200xxxx)

Source:	China Telecom

Title:		Solution Evaluation for KI#15 User Consent

Document for:	Approval

Agenda Item:	8.1

Work Item / Release:	FS_eNA_Ph2 / Rel-17

Abstract of the contribution: Solution Evaluation for 23.700-91-050 of eNA Key Issue #15

1 Proposal

The aim of key issue 15 is to discuss and specify:

-	the method(s) for providing user consent to the relevant 5GC entities;

-	kind(s) of network entities will be involved in enforcing the user consent.

It proposes to document the following texts into TR 23.700-91.

First change

7	Overall evaluation

7.X	Evaluation of solutions for Key Issue #15 - User consent for UE data collection/analysis

For Key Issue #15 - User consent for UE data collection/analysis, 2 Candidate Solutions have been proposed. Candidate solutions #3 and #76 both provide user consent when collecting/analysing UE data.

Both these 2 solutions are similar, considering user consent as part of the subscription data.

Candidate Solution #3 is simpler than #76 which considers more scenarios and has more steps. In this solution, AF may collect the user consent (if applicable) from its users and provide the information via the NEF in the user consent along with the UE Id. After authorization, UDM translates UE external ID to a 3GPP internal UE ID. Before initiating UE data collection and reporting by the UE, data consumers (e.g. NWDAF(s)) request for user consent from UDM, which stores user consent as a subscription information.

Candidate Solution #76 introduces a "User Consent Tracking Functionality" (URDCF) that is provided by the NWDAF. This solution shows 2 procedures: the user consent validating in Data Request procedure and the procedure related to user consent revocation. In this solution, NWDAF with URDCF keeps track of Data requests/subscriptions and notifies Data Consumers (e.g. NWDAF) about withdrawn user consent, and Data Consumer deletes user data if user consent validity time has expired or upon request from NWDAF with URDCF if user consent was withdrawn.

End of change