3GPP TSG-WG SA2 Meeting #141E	S2-2007024

Oct 12 – Oct 23, 2020, Elbonia	(revision of S2-2005399)

Source:	China Mobile, CATT

Title:	KI #1: Interim conclusions

Document for:	Approval

Agenda Item:	8.1

Work Item / Release:	FS_eNA_ph2 / Rel-17

Abstract: This contribution proposes interim conclusions on Key Issue #1.

1. Introduction

This contribution proposes interim conclusions on KI #1: Logical decomposition of NWDAF and possible interactions between logical functions.

2. Text Proposal

It is proposed to capture the following new text in TR 23.700-91.

* * * * Start of Changes * * * *

8	Conclusions

Editor's note:	This clause is intended to list conclusions that have been agreed during the course of the study item activities. This should also capture the guiding principles and documentation approach for creating CRs to normative specifications within the responsibility of SA WG2.

8.1	Key Issue #1: Logical decomposition of NWDAF and possible interactions between logical functions

Interim conclusion:

NWDAF can be composed of Training Function and Analytics (Inference) Function, which expose training service (e.g. providing trained model to the NWDAF) and analytics service (e.g. providing analytics information to the NF or OAM) respectively. A NWDAF can be decomposed into Model Training Function (MTF) or Analytics Function (AnF). These decomposed functions (i.e. MTF and AnF) can be:

- Standalone functions with a standardized interface(s) between these functions and NF consumers;

- Colocated with the NWDAF.

NOTE 1:	It can also be possible to identify other standalone NF(s) based on other KI results, e.g. Data Dollection function from KI#11.

NOTE 2: The definition of these decomposed functions will be defined in the normative phase.

Editor's note:	Functional description of these decomposed functions are FFS.

For the interaction between MTF and AnF, solution 7 is considered as a basis for the normative work.

The proposed Data Collection function for NWDAF decomposition is dependant on the evaluation and conclusion of Key Issue #11: Increasing efficiency of data collection.

* * * * End of Changes * * * *