# 3GPP TSG-WG SA2 Meeting #141E	S2-2007345

# Elbonia, October 12 – 23, 2020 	(revision of S2-200xxxx)

Source:	CATT

Title:	KI #1, sol #1: updates

Document for:	Discussion/Approval

Agenda Item:	8.5

Work Item / Release:	FS_IIoT / Rel-17

Abstract of the contribution: This contribution proposes clarification of NEF behavior on solution #1.

1		Discussion

Per the conclusion of KI#3B, the NEF can work as AF to decide the BMCA port role.

For Method 1 of BMCA procedure, it is not clear how to implement BMCA by NEF when the TSN AF is not deployed. This pCR intends to clarify the behaviour of NEF :

-	Receives the Announce information reported by DS-TT/NW-TT port via PMIC signalling;

-	Executes BMCA to determine the port role and configure port role to associated DS-TT/NW-TT port;

In addition, this pCR adds impacts for Method 1.

2		Proposal

The following change is proposed for TR 23.700-20.

<<< Start of 1st change >>>

6.1.3.3 BMCA procedure

One of the following alternative methods for the BMCA procedure may be implemented. The BMCA state machines required in IEEE 802.1AS [6] clause 5.4.1, may run in different parts of the 5G system.

Method 1:

When a NW-TT port or a DS-TT port receives an Announce message, if the Announce information is better than the current best master information it knows about (per clock domain), the port reports the Announce information to TSN AF using PMIC signalling. NW-TT and DS-TT also inform TSN AF if a previously reported best clock is not available anymore (determined based on lack of Announce or Time Sync messages from that clock). Based on the received Announce information and local information of 5GS, the TSN AF runs the PortStateSelection state machine and decides the BMCA port roles for the ports in the 5GS bridge as described in IEEE 802.1AS-Rev [6] and configures each port role (per clock domain) accordingly using PMIC signalling. The port that is configured to Slave role, receives Sync and Announce messages from a node outside 5GS and sends the Sync and Announce messages to other port(s) inside 5GS. The port that is configured to Master role, receives Announce messages from the port with Slave role inside the 5GS bridge, increases the stepsRemoved field of the Announce messages with one, replaces the value of sourcePortIdentity field of Sync/Announce messages with its portIdentity, and runs the PortAnnounceTransmit state machine to send Announce messages to a node outside the 5GS. When DS-TT port configured to Passive role receives a Sync message from outside of 5GS, it discards the message. Optionally, NW-TT and DS-TT with port(s) with Master role(s) can generate Announce messages based on information from TSN AF (e.g. DS-TT or NW-TT acts as a (g)PTP grandmaster). In this case, Announce messages are not exchanged inside 5GS. When the TSN AF is not deployed, the NEF could work as TSN AF to receive the Announce information reported by DS-TT/NW-TT port via PMIC signalling, and execute BMCA to determine the port role and configure the port roles to associated NW-TT port or DS-TT port . (Details of BMCA procedure implemented by TSN AF are described in Solution #18.)

Method 2:

In this alternative, NW-TT receives Announce messages from any NW-TT and DS-TT ingress port (DS-TT forwards all Announce messages to NW-TT) and NW-TT runs the PortStateSelection state machine and decides the BMCA port roles.

Editor's note:	Whether NW-TT and DS-TT can send Announce messages only when there are chages including time out events and if there is a need for this optimization is FFS.

Upon completion of BMCA, NW-TT reports the BMCA result to TSN AF using BMIC signaling. Based on this, TSN AF configures the port roles in DS-TT using PMIC signaling. If TSN AF is not deployed then NEF terminates PMIC and BMIC signaling, i.e. NEF receives the BMCA result from NW-TT and configures configures the port roles in DS-TT using PMIC signaling.

NOTE 2:	NEF terminating BMIC/PMIC signaling does not have any impact on PCF/SMF as existing Rel-16 functionality is reused for this.

Port roles of NW-TT ports are configured locally by NW-TT based on the BCMA result. When a DS-TT configured to Master role receives Announce messages from the NW-TT via 5GS user plane, it increases the stepsRemoved field of the Announce messages with one, replaces the value of sourcePortIdentity field of Sync/Announce messages with its portIdentity, and runs the PortAnnounceTransmit state machine to send Announce messages to a node outside the 5GS. When DS-TT port configured to Passive role receives a Sync message from outside of 5GS, it discards the message. If the NW-TT time outs to receive Announce message from the Slave port and has available Passive port(s), it selects a Passive as new Slave port and update port role(s).

Method 3:

Support BMCA via the interworking between the DS-TT and NW-TT:

-	When DS-TT received any announce messages from the port(s) of the DS-TT, if the message Priority Vector of the announce message is better than the Port Priority Vector of the received port, the DS-TT sends the received announce messages to NW-TT via UE.

Editor's note:	It is FFS how a Passive port would learn the Port Priority Vector.

-	The NW-TT/UPF learns the received port for the announce message based on the port number associated with the PDU session sending the announce message. The NW-TT determines the best grand master based on the received announce messages, the port number of the received port on the DS-TT(S) and NW-TT, and the System Priority Vector. If the determined grand master is updated, the NW-TT decides the port state and announce message for next hop accordingly.

-	If the port state of a NW-TT port is master port, the NW-TT forwards the determined announce message to the NW-TT port directly. For each DS-TT port which is a Master port, the NW-TT/UPF runs the PortAnnounceTransmit state machine to send Announce messages towards the PDU session associated with the DS-TT port. Upon receiving the announce message the DS-TT forwards it to the port associated the PDU session receiving the announce message and update the Port Priority Vector accordingly.

-	When DS-TT port configured to Passive role receives an Announce message or a Sync message from outside of 5GS, it forwards it to to NW-TT who discards the message.

Method 4:

Support BMCA process in NW-TT (Details are described in Solution #Y [S2-2005160]).

-	There is a BMCA function in the NW-TT. It run the BMCA state machine and keep all the ports state.

-	When the DS-TT port or NW-TT port receives the Announce message, it forward the message to BMCA function in NW-TT via U-plane.

-	The BMCA function run the BMCA state machine to qualify the message, determines the best grand master and determine the DS-TT port and NW-TT port state.

-	If the Announce message is reveived from the slave port, the BMCA function update the Auunoce message and send to all NW-TT ports which role is Master and all DS-TT ports which role is Master.

-	If the Announce message is reveived from a non-slave port, the BMCA discard this message.

<<< End of 1st change >>>

<<< Start of 2nd change >>>

6.1.4	Impacts on services, entities and interfaces

For synchronizing TSN end stations behind 5G System (NW-TT) with the TSN GM in the network attached to the device, the impacts on UE, SMF, PCF, TSN-AF and UPF are like the following.

-	The Ingress TT is DS-TT.

-	The Egress TT is NW-TT.

For synchronizing TSN end stations behind other UE(s) with the TSN GM in the network attached to the device side via 5G System, the impacts on UE, SMF, PCF, TSN-AF and UPF are like the following.

-	The Ingress TT is DS-TT of a UE.

-	The Egress TT is DS-TT of the other UE.

-

-	[Optional] The TSN AF needs to trigger QoS setup procedures in case the operator decides to follow the recommendation to limit the bridge residence time to 10 ms

-	The UPF needs to perform local switching for Sync messages without NW-TT interaction.

Method 1 of BMCA procedure:

NW-TT port/DS-TT port:

-	Reports Announce information to TSN AF or NEF

TSN AF/NEF:

-	Receives the Announce information reported by DS-TT/NW-TT port;

-	Executes BMCA to determine the port role and configure port role to associated DS-TT/NW-TT port.

<<< End of changes >>>