SA WG2 Meeting #S2-141E	S2-2007512

12 October - 23 October 2020, Electronic, Elbonia	(revision of S2-200xxxx)

Source:	ZTE

Title:	Architecture#: Discussion and proposal on the convergence of architecture

Document for:	Agreement

Agenda Item:	8.9

Work Item / Release:	FS_5MBS / Rel-17

Abstract of the contribution: This contribution proposes a converged architecture for 5G MBS.

1. Proposal

This paper depends on the r16 of S2-2005409 and make further modification with revision mark.

*****************************************START of CHANGE **********************************************

# A.X	Baseline architecture X: 5G MBS converged system architecture

# A.X.1	General

Based on the two baseline architectures for 5G MBS as depicted in Annex A.1 and A.2, the commonalities and differences are captured in this alternative.

NOTE:	Please note that architectures are typically frameworks with little or no functionality in themselves. Therefore, architectures cannot successfully be evaluated or compared without any solutions. Functionalities need to be added to an architecture e.g. by studying certain use cases and providing solutions that realize these use cases. The description in A.X.2 below therefore contains baseline architecture together with a set of basic solutions.

Editor´s note: The functionality described will need to be refined depending on the conclusion for the key issue. It is not meant as evaluation criteria that all solution need to fulfil during the evaluation phase.
It is not necessary to update the terminology of all solutions to comply with the present baseline architecture. It is anticipated that this will be done fduring the normative phase.

## A.X.2	Reference Architecture

### A.X.2.1	General

Figure A.X.2.1-1 illustrates the 5GS SBI architecture supporting MBS.

UE

NG-RAN

MB-UPF

AMF

MB-SMF

NEF

MBSF

MBSU

Data Network

Application Server

PCF

N6

gMB-U

(xMB-U/

MB2-U)

Ny

N4

N3

Uu

N2

Namf

Nmbsmf

Npcf

Nnef

Naf

Nmbsf

SMF

Nsmf

UDR

Nudr

AF

gMB-C (xMB-C/

MB2-C)

N6

N1

N4

UPF

Figure A.X.2.1-1: 5GS Architecture supporting MBS

In Figure A.X.2.1-1, the SMF and UPF which have the roles to support MBS Sessions are named "MB-SMF" and "MB-UPF" respectively.

NOTE 1:	If 5GC individual delivery method is used, a UPF may be placed between MB-UPF and NG-RAN.

NOTE 2:	MBSU may be required depending on deployment.

Editor's note: It is FFS whether it is mandatoryto deploy MBSF.

NOTE 3:	The MB-SMF is an SMF extended with MBS functionality. An MB-SMF may be dedicated to MBS services. The MB-UPF is an UPF extended with MBS functionality. A MB-UPF may be dedicated to MBS services.

NOTE 4:	The architecture using the reference point representation may be specified based on the selected solutions.

NOTE 5:	gMB-C and gMB-U provide similar functionalities as defined for xMB-C/MB2-C and xMB-U/MB2-U interfaces, respectively.

The 5G MBS System Architecture contains the following new service-based interfaces:

Nmbsmf:	Service-based interface provided by MB-SMF, could be part of Nsmf.

Nmbsf:	Service-based interface provided by MBSF, could be part of Nnef.

Figure A.X.2.1-2 illustrates the 5GS reference point architecture supporting MBS.

Figure A.X.2.1-2: 5GS reference point Architecture supporting MBS

### A.X.2.2	Deployment configuration

To support different service or scenario (e.g. whether support legacy AF, whether interworking with eMBMS, whether the AF is in the trust domain or not), there may be different configuration on top of figure A.X.2.1-2.

The configuration support legacy AF (i.e. MB2/xMB capable AF):

The figure A.X.2.2-1 show how the 5MBS architecture support the legacy AF. The NEF and MBSF may be co-allocated. This configuration also support interworking with eMBMS.

Figure A.X.2.2-1: 5G MBS support legacy AF configuration

The configuration support 5GS enable AF (i.e. N33):

The figure A.X.2.2-2 show how the 5MBS architecture support the 5G MBS enabled AF. The NEF and MBSF may be co-allocated. If the interworking with eMBMS si supported, the MBSF is required.

Figure A.X.2.2-2: 5G MBS support 5G MBS enabled AF configuration

A.X.2.3	Functional entities

NOTE:	The enhancements of network functions covers commonalities of architecture alternative 1 and 2, and can be refined based on the selected solution.

A.X.2.3.1	PCF

The PCF is used to provide policy rules for MBS services, and receive MBS service information from AF, directly or indirectly. The PCF performs the following functions to support MBS:

-	Support QoS handling for MBS Session, including QoS parameters like 5QI, MBR, GBR

-	Provide policy information regarding the MBS session to SMF for authorizing the related QoS profiles.

-	Receive MBS service information.

A.X.2.3.2	MB-SMF

The MB-SMF is used for MBS session management (including QoS control), and control of MBS transport, including configuring the MB-UPF and RAN (via AMF) for MBS flows transport based on the policy rules for MBS services from PCF or local policy.

A.X.2.3.3	MB-UPF

The MB-UPF is used for delivery of MBS flows to RAN (or UPF for individual delivery) and QoS enforcement for MBS services. The UPF performs the following functions to support MBS:

-	Packet filtering for MBS flows.

-	Distribution of MBS data packets to different RAN nodes (or UPF nodes).

-	QoS enforcement (MFBR) and counting/reporting based on existing means.

A.X.2.3.4	AMF

The AMF performs the following functions to support MBS:

-	Selection of MB-SMFs that have MBS capabilities.

-	Signalling with NG-RAN and MB-SMF for MBS Session management.

A.X.2.3.5	NG-RAN

The NG-RAN is used to receive MBS data packets via shared N3 tunnel and deliver MBS data packets to the UEs using PTP or PTM delivery method. The NG-RAN performs the following functions:

-	Management of MBS QoS flows via N2

-	Reception of MBS data via shared N3 tunnel.

-	Configuration of UE for MBS QoS flow reception at AS layer.

-	Delivery of MBS data using PTM or PTP.

-	Control switching between PTM and PTP delivery per UE.

-	Support for MB Sessions during Xn Handover and N2 Handovers.

A.X.2.3.6	UE

The UE needs to support the MBS operations and receive MBS data packets from NG-RAN. The UE performs the following functions:

-	Reception of multicast data using PTM /PTP.

-	Reception of broadcast data.

-	Management of MBS QoS flows.

-	joining and leaving MBS session.

-	MBS support at AS layer.

A.X.2.3.7	AF

The AF requests MBS service from the 5GC. The AF does so by

-	providing MBS service information.

-	negotiating with NEF for MBS related service exposure.

-	instructing MBS session operation towards 5GC if needed.

A.X.2.3.8	NEF

The NEF supports 5MBS service exposure by providing an interface to AFs for 5MBS procedures including service provisioning and MBS session and QoS management..

A.X.2.3.9	MBSF

The MBSF performs the following functions:

-	Interacting with AF and MB-SMF for MBS session operations and transport.

-	Selection of MB-SMF for MBS Session

-	Controlling MBSU if the MBSU is used.

A.X.2.3.10	MBSU

The MBSU performs the following functions:

-	Modification of encoding of MBS data.

-  Media anchor for MBS data traffic if needed.

NOTE: The MBSF and the MBSU may be collocated or deployed separately.

********************************************Next CHANGE **********************************************

8	Conclusions

8.Y	Conclusions for architecture

Interim conclusion for Architectural: architecture option in Annex A.X "5G MBS converged system architecture" is chosen as the baseline architecture for 5G MBS.

*******************************************End of CHANGE **********************************************