3GPP TSG-WG SA2 Meeting #141E e-meeting 	S2-2007819

October 12 – 23, 2020, Electronic

# Title:	Reply LS to Clarification on the assumption of only one AMF region to a tracking area

# Release:	Rel-16

# Work Item:	5GS_Ph1

Source:	3GPP SA2

To:	3GPP SA3-LI

Cc:

Contact Person:

Name:	Devaki Chandramouli

Tel. Number:

E-mail Address:	Devaki.chandramouli@nokia.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

# Attachments:	-

1. Overall Description:

3GPP SA WG2 thanks SA3-LI for their LS on Clarification on the assumption of only one AMF region to a tracking area.

Question: SA2 is kindly asked to clarify whether SA3 LI can assume that an AMF Region ID can uniquely be determined from the TAI.

SA2 Answer:

Stage 2 allows one TAI to map to AMFs in more than one AMF Region. In order to avoid high signalling load at AMF Region boundaries, some networks may choose to allow AMF Regions to overlap.

Thus, generally, it is not possible to uniquely determine an AMF Region ID from TAI.

While stage 2 allows the AMF Set ID and AMF Pointer to be reused in different AMF Regions, (snippet from TS 23.501) this cannot be done safely within the area of AMF region overlap.

NOTE 1:	The AMF Region ID addresses the case that there are more AMFs in the network than the number of AMFs that can be supported by AMF Set ID and AMF Pointer by enabling operators to re-use the same AMF Set IDs and AMF Pointers in different regions.

In order for the Service Request and Paging procedures to work properly in tracking areas where the AMF region overlaps, it is up to the deployments to ensure 5G-S-TMSI is unique within a Tracking Area (no explicit mechanism is currently defined in the spec).”

Some associated information:

Multiple AMFs can be connected to one gNB.

Tracking Area boundaries do not cross AMF boundaries. The AMF is assumed to be connected to all the gNBs that are part of the TAI(s) in the TAI list allocated to a given UE by that AMF.

2. Actions:

To 3GPP SA3-LI

ACTION:

SA WG2 requests SA3-LI to take the above answer into account.

3. Date of Next TSG SA WG2 Meetings:

TSG-SA2 Meeting #142E   		16 - 20 November 2020