3GPP TSG-SA/WG2 Meeting #141e	S2-2007876

# Elbonia, 12 – 23 Oct. 2020, Electronic meeting

Source:	ZTE

Title:	KI#3 solution#49 update

Document for:	Agreement

Agenda Item: 	8.3

Work Item / Release:	FS_enh_EC / Rel-17

Abstract of the contribution: This contribution proposes update for solution 49

- Proposal

It is proposed to agree the changes:

/********************Start of Change***************/

6.49	Solution #49: Network Information Provisioning to EAS with low latency based on User Plane

6.49.1	Description

In this solution, the network information is sent via user plane in the core network to the Local PSA and then further sent to the AF via a Local NEF.

The solution supports only providing information about UE defined in Rel-16 as part of QoS monitoring

Figure 6.49.1-1: Local NEF-based exposure

In Figure 6.49.1-1, a Local NEF is deployed in the Edge. The Local NEF can be co-located with the Local PSA or standalone, or as part of Edge Hosting Environment. The interface between local PSA and local NEF (i.e. Nx interface) could reuse N4 protocol.

The solution targets reporting QoS monitoring results as defined in Rel-16 TS 23.502 [3] towards the EAS/application.

1.	Event subscription

The AF responsible for the EAS subscribes the event by invoking Nnef_AFsessionWithQoS service to the local NEF and then the local NEF subscribes the event via the central NEF or PCF with the notification endpoint to the local NEF. The notification endpoint can be used to identify the associated AF subscription event in the local NEF. The local NEF needs to indicate the PCF that the local event notification is needed. Based on the indication of local event notification and operator's policy, the PCF can determine whether only local event notification or both local and central event notification are required. The PCF shall include an indication of local event notification or an indication of duplicated event notification and the notification endpoint of the local NEF within the PCC rule and provide to the SMF.

2.	Supported Events and notification procedure

For the QoS monitoring, when the SMF receives such kind of PCC rule, the SMF sends the QoS monitoring request to the RAN and N4 rules with notification endpoint of the local NEF to the Local PSA. Upon the detection of the QoS monitoring event, the Local PSA sends the notification to the notification endpoint of the Local NEF. The Local PSA may also send the notification to the SMF if the duplicate notification is needed.

3.	Mobility supporting

During UE mobility, the UPF and DNAI may be re-selected. To ensure the low latency exposure, the Local NEF should be also re-selected. The AF may consider one or more of following parameters to reselect a local NEF: UE location, AF Identifier, NEF Service Area, and DNAI, in additional to other parameters listed in TS 23.501 [2], clause 6.3.14. The new AF initiates a new AF session with required QoS and QoS monitoring is established and the old AF revokes the AF session and QoS monitoring is removed.

The new L-NEF can be determined with the assistance of 5GC (e.g. SMF, NEF) and the new EAS will trigger QoS monitoring to new L-NEF.

6.49.2	Procedures

Figure 6.49.2-1: Network Information Provisioning to Local Applications with low latency based on User Plane

1.	The UE establishes a PDU session with Local PSA as defined in clause 4.3.2.2.1 of TS 23.502 [3].

2.	The AF initiates an AF session with required QoS procedure as defined in clause 4.15.6.6 of TS 23.502 [3].

In the request, the AF may also subscribe to local notification of QoS monitoring and provide the associated notification endpoint of the AF to the local NEF. For the QoS monitoring, the AF shall include the corresponding QoS monitoring parameters.

In the request, the Local NEF indicates the PCF that local notification is required and the notification endpoint of the Local NEF is provided to the PCF. If the NEF is deployed, the subscription is performed via the NEF.

The PCF makes the policy decision. The PCF generates the PCC rule based on the service information.

If the local notification of QoS monitoring is subscribed, the PCF includes the QoS monitoring policy, notification endpoint of the Local NEF and the indication of local event notification within the PCC rule.

Based on the operator's policy, the PCF may determine that the duplicated notification is required, i.e. both local notification to the AF and central notification to the PCF is required. In this case, the PCF includes the indication of duplicated event notification within the PCC rule instead.

3.	PCF initiates the PDU session modification procedure as defined in clause 4.3.3.2 of TS 23.502 [3], step 1b, 3b, 4-8b.

In the case of local notification of QoS monitoring, the SMF includes the QoS Monitoring indication in the request to the UPF. The SMF may also include the indication of local event notification or duplicated event notification in the request.

4.	If the QoS monitoring is enabled, the Local PSA calculates the UL, DL or round trip packet delay of the QoS Flow as defined in clause 5.33.3 of TS 23.501 [2]. When the reporting trigger(s) is satisfied, e.g. the measured packet delay value exceeds the reporting threshold, or the reporting period expires, or the PDU Session is released, the event is detected.

5.	The Local PSA sends the event notification to the notification endpoint of the Local NEF carrying UL, DL or round trip packet delay measurement report.

If the duplicated event notification indication is received, the Local PSA sends the notification to the SMF as well as defined in TS 23.501 [2].

6.	The Local NEF sends the event notification to the notification endpoint of the AF.

7.	If due to mobility, UE moves to another area where DNAI changes. The PSA relocation and EAS relocation are performed.

8.	The new AF initiates a new AF session with required QoS to the new local NEF or the one indicated by 5GC (e.g. SMF or NEF). QoS monitoring is established by executing steps 2-4.

9.	The old AF revokes the AF session with the old local NEF and QoS monitoring is removed for the old QoS flow.

6.49.3	Impacts on services, entities and interfaces

AF:

-	Subscribe local notification of QoS Monitoring.

-	Re-select Local NEF based on UE location, AF Identifier, NEF Service Area, and DNAI.

PCF:

-	Subscribe local notification of QoS Monitoring.

SMF:

-	Subscribe local notification of QoS Monitoring (from UPF to Local NEF).

-	Support Local NEF selection.

UPF:

-	report local notification of QoS Monitoring to local NEF.

NEF:

-	Support Local NEF selection.

Local NEF:

-	Receive local notification of QoS Monitoring and transfer it to the AF.

/*******************End of Change****************/