3GPP TSG-WG SA2 Meeting #141E e-meeting  	S2-2007084

# Elbonia, 12 – 23 October 2020	(revision of S2-200xxxx)

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 2471 | rev | - | Current version: | 16.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Clarification on S-NSSAI use in 5G MOCN single 5GC |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_Ph1, TEI16 |  |  |  |  |  | Date: |  |  | 2020-10-02 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Based on the discussion in SA2#139e and SA2#140e on Vertical_LAN, there is clearly a confusion regarding the use of network slicing in 5G MOCN shared RAN pointing to the same 5GC for two PLMN IDs (or PLMN ID+NID). In Rel-15 and Rel-16, it is not possible within one single 5GC to have multiple namespaces for network slices (S-NSSAIs) depending on the PLMN ID (or PLMN ID + NID) used by the UE to access the 5GC. See S2-2007083 for more details on what is possible and what is not possible with the current 5GS design. Given the impact of supporting separate namespaces and the loss of the "shared network slice" capability, it is questionable whether we should change the current design in future releases. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Adding clarification that PLMNs and SNPNs using the same 5GC with 5G MOCN cannot use the same S-NSSAI to identify different network slices. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The ongoing confusion that S-NSSAI values are PLMN ID dependent within one 5GC might result in wrong implementation and specification work. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.18.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

### 5.18.5	Network Sharing and Network Slicing

As defined in clause 5.15.1, a Network Slice is defined within a PLMN or SNPN. Network sharing is performed among different PLMNs and/or SNPNs. In the case of network sharing, each PLMN or SNPN sharing the NG-RAN defines and supports its PLMN- or SNPN- specific set of slices that are supported by the common NG-RAN.

NOTE:	Different PLMNs/SNPNs supported by the same 5GC cannot use the same S-NSSAI to point to distinct network slices.

# * * * * End of changes * * * *