SA WG2 Meeting SA2-141e	S2-2007324

12th October – 23rd October, 2020, E-meeting	(revision of S2-201xxxx)

Source:	Intel

Title:	KI #4, Conclusion Update – Resolve ENs

Document for:	Discussion / Approval

Agenda Item:	8.2

Work Item / Release:	FS_eNPN / Rel-17

Abstract of the contribution:

# 1	Discussion

The discussion in this pCR is focused on the following two interim conclusions agreed in SA2#140e meeting for remote provisioning of SNPN credentials:

- Need to pre-configure the NSSAI/DNN in the UE and the associated EN.

- Reference to industry-based mechanisms for user plane provisioning of SNPN credentials.

- Need to pre-configure the NSSAI/DNN in the UE and the associated ENs

Editor's note:	the need to pre-configure the S-NSSAI and DNN in the UE and to send it to the UE is FFS as default DNN and S-NSSAI could be used for provisioning to an SNPN. Also, if needed, whether to progress one or more options into normative phase is FFS.

There can be two types of UE in the context of this KI for Onboarding i.e.,

- an off the shelf UE provisioned with default credentials and a unique UE identifier or a UE may be pre-configured

- provisioned with initial default configuration including PLMN ID and NID of the SNPN, S-NSSAI, DNN needed to access the provisioning server.

The Editor’s Note mentioned above is applicable for case b. If the UE is pre-configured with the S-NSSAI for the purpose of Onboarding only, then a new SST value needs to be standardized for the purpose of Onboarding. In our view, it is upto the network to decide selecting the AMF and the DNN for the purpose of Onboarding purpose based on an indication provided by the UE. Similar to RLOS, there should be one RRC indication that allows the NG-RAN to select a dedicated AMF for onboarding, and there should be one NAS indication that allows the AMF to e.g. select a dedicated SMF for onboarding and perform other onboarindg-related configuration. Given that the solution needs to work for off-the-shelf UE, we do not see the need to use a different mechanism for UEs with additional pre-configuration and propose deleting the corresponding EN on the use of S-NSSAI and DNN. Moreover, we propose to close the Editor’s note on the need for Onboarding indication.

- Reference to industry-based mechanisms for user plane provisioning of SNPN credentials.

In the following highlighted agreed interim conclusion, for the case of UP provisioning of SO-SNPN credentials the UP remote provisioning protocol used is out of scope of SA2. However, the sentence also includes reference to industry developed mechanisms without any clarification on what type of provisioning will be supported as part of the industry developed mechanisms. For example, in TR 23.700-07, an example user plane protocol in Solution 35 refers to reusing CMPv2 protocol. Though certificate-based provisioning is one of the possible UP provisioning mechanisms that may be used, it is unclear if the industry developed mechanisms can support other EAP-compatible authentication methods (e.g. pre-shared secret, identity-based cryptography, etc.). So, we propose to remove uses industry developed mechanisms from the interim conclusion without specific reference to the industry protocol and leave that discussion to SA3.

When User Plane is used for provisioning of SO-SNPN credentials, the User plane remote provisioning protocol used and how the UE downloads the SNPN credential from the Provisioning Server (PS) after PDU session establishment in the O-SNPN is out of scope of SA2 and uses industry developed mechanisms.

# 2	Proposal

It is proposed to update TR 23.700-07 as indicated below.

* * * * Start of Change (All text is new)* * * *

## 8.4	Key Issue #4: UE onboarding and remote provisioning

Editor's note:	These are *INTERIM* conclusions for Key issue #4.

UE onboarding for SNPN (Component 1 of KI#4)

-	It should be possible to support a registration procedure that enables support for UE onboarding using Default UE credentials and with an O-SNPN as the Onboarding Network (ON).

Editor's note:	In order to support UE onboarding using Default UE credentials and O-SNPN as the Onboarding Network (ON) the distribution of security functions when primary authentication is used should be decided by SA WG3, e.g. whether and how to support the primary authentication based on default credential in case DCS is deployed or not.

-	It should be possible that one SNPN can take the role of both Onboarding Network (ON) and SO (Subscription Owner), and it should be possible that the ON and SO are different SNPNs i.e. O-SNPN and SO-SNPN.

Editor's note:	DCS is potentially introduced to authenticate a UE with default UE credentials or provide means to another entity to do it. There are two potential mechanisms for DCS to authenticate the UE. 1) DCS interacts with O-SNPN and Network Function in SO-SNPN (Subscription Owner SNPN) is not involved in the authentication procedure. As a result, the SO-SNPN is not aware of the result of authentication procedure but perform remote provisioning. 2) DCS interacts with SO-SNPN and Network Function in SO-SNPN (Subscription Owner SNPN) is involved in the authentication procedure. As a result, the SO-SNPN is aware of the result of authentication procedure and perform remote provisioning. SA WG3 needs to evaluate the two above mechanisms from security perspective and provide feedback.

Editor's note:	The decision on whether primary authentication is required during initial access to the O-SNPN is dependent on SA WG3 feedback; until this feedback is received, it is assumed that such authentication is required.

-	The SNPN, which directly interacts with DCS, determines the corresponding DCS identity or address/domain based on the input from the UE.

Editor's note:	It is FFS whether some alternative way to identify the DCS is to be added.

-	The NG-RAN of the Onboarding network includes information in the SIB so that the UE can discover and select an appropriate O-SNPN.

-	Upon registration for Onboarding, the UE provides an indication at RRC level that the connection request is for onboarding. This indication allows NG-RAN to select an appropriate AMF and possibly handle RAN-level congestion.

-	Upon registration for Onboarding, the UE provides an indication at NAS level that the registration request is for onboarding. This indication allows AMF to e.g. select an appropriate SMF and perform other onboarding-related configuration.

-	Using PLMN credentials for UE onboarding and PLMN as Onboarding Network (ON) is already possible and does not require any further standardisation work.

-	Onboarding network should support functionality to restrict usage to only on-boarding service.

-	The UE shall initiate de-registration from the on-boarding network after finishing the remote provisioning or the on-boarding network shall initiate the de-registration after successful completion of onboarding or based on timer configured for on-boarding registration.

Editor's note:	Further conclusions how to enable on-boarding registration is FFS e.g., based on default ON profile at UDM (e.g. When PLMN is used) or UE indication or local policy.

Remote provisioning for SNPN credentials (Component 2 of KI#4)

-	Usage of a PLMN as Onboarding Network for a UE equipped with a USIM shall be possible. The SNPN credentials can be transmitted to UE via UP connectivity. The UE shall be configured with Default credentials in USIM to register with a PLMN where the UE can register with the Default credentials in order to communicate with the provisioning server;

-	When User Plane is used for provisioning of SO-SNPN credentials, the User plane remote provisioning protocol used and how the UE downloads the SNPN credential from the Provisioning Server (PS) after PDU session establishment in the O-SNPN is out of scope of SA2;

-	Control Plane remote provisioning based on UE Parameters Update Procedure as defined in TS 23.502 [6] can be used for provisioning of SO-SNPN credentials;

Editor's note:	SA WG3 feedback will need to be taken into account for including of the CP based provisioning.

Editor's note:	It is FFS whether the UE Parameters Update container can be delivered to the UE during the registration procedure.

-	For the provisioning of IMSI accompanied by AKA credentials, GSMA RSP is used, Provisioning Server (PS) can provision the credential to UE over User Plane (UP) connectivity;

-	For the provisioning of Non-3GPP credentials, the credentials can be provided to UE over UP connectivity;

-	It shall be possible to pre-configure the Provisioning Server (PS) address, SO-SNPN identity, neither or both used to access PS on the UE, and it shall also be possible that the O-SNPN provides the PS address used to access PS to the UE after successful authentication and authorization. The PS address used to access PS from the O-SNPN shall be integrity protected. The PS address used to access PS provided by the network is prioritized, if configured and overrides any PS address, used to access PS stored in the UE. Configuration of PS address used to access PS from the onboarding configuration data to the UE can be supported using one of the following methods:

a.	SMF may deliver onboarding configuration data as part of extended Protocol Configuration Options (PCO) in PDU Session Establishment Response to UE. This is similar to use of PCO to configure Autoconfiguration server for UE in Wireless and Wireline Convergence (TR 23.716 [28] clause 6.10).

b.	Alternatively, onboarding configuration data may be configured in the UE during Registration Procedure using UE Route Selection Policy (URSP) that may be subject UE capabilities. As part of UE initial registration (based on received UE capability information) AMF indicates to PCF that UE has requested restricted/provisioning registration. The PCF may initiate UE Policy delivery using UE Route Selection Policies (URSP), for instance to trigger UE after successful registration to request establishment of specific type of PDU Session limited to onboarding purposes only.

c.	In addition, onboarding configuration data may be configured in the UE using service specific policies subject to UE capabilities similar to what is used for V2X communications as specified in TS 23.287 [29] clause 5.1.1 for ways how parameters may be made available to the UE and TS 23.287 [29] clause 6.2.5 for AF-based service parameter provisioning and TS 24.587 [30] clause 5.2.4 for configuration parameters such as validity timer, server address and geographical area.

-	It is assumed that the UDM (or AAA) of SO-SNPN is provisioned with UE credentials/subscription data when remote provisioning is successfully performed.

Editor's note:	Whether the specific procedure for provisioning of UDM (or AAA) with the UE credentials/subscription data is in scope of 3GPP is FFS.

-	Upon successful establishment of restricted access PDU session, if the UE still does not have a PS address, the device uses a well-known FQDN to perform PS discovery.

NOTE:	SA WG3 may evaluate these mechanisms and provide guidance on appropriateness of use for SNPNs.

UE Onboarding for PNI-NPN (Component 1 of KI#4)

-	No enhancement for the UE onboarding (component 1 of KI#4) with PLMN credentials used for primary authentication and PLMN network selection are needed for the case of PNI-NPN credentials provisioning.

Remote provisioning for PNI-NPN credentials (Component 2 of KI#4)

-	At least network initiated remote provisioning of credentials to allow access to PNI-NPN services should be supported in Rel-17;

-	Both procedures using Control Plane and using User Plane protocols after establishing PDU session shall be enabled for remote provisioning the PNI-NPN credentials used for NSSAA and/or PDU Session secondary authentication;

Editor's note:	SA WG3 feedback for the suitability of the procedure will need to be taken into account.

Editor's note:	whether an extra security layer for protection of credentials between PS and UE is needed should be decided by SA WG3.

-	For User Plane remote provisioning:

-	The protocol for provisioning of PNI-NPN credentials used for NSSAA and/or PDU Session secondary authentication, i.e. how the UE download the NPN credential from the PS after PDU session establishment in PNI-NPN, is out of scope of SA WG2;

-	The PS address and DNN/NSSAI used to access PS may be provided to the UE during or after the Registration procedure;

Editor's note:	How the PS address is provided to the UE is FFS.

Editor's note:	The vertical may verify the UE before PNI-NPN credential is provisioned to UE, and how this is done should be decided by SA WG3.

-	Upon successful remote provisioning of the UE, the UE Subscription Data in the UDM/UDR may be updated to enable the access to the PNI-NPN.

Editor's note:	for PNI-NPN credentials remote provisioning, whether the 3GPP operator could decide to update the UE Subscription Data (e.g., S-NSSAI, DNN, CAG information) in the UDM/UDR used to access to the PNI-NPN based on the input from the vertical which may be outside 3GPP operator domain should be decided by SA WG3.

* * * * End of Change * * * *