SA WG2 Meeting #141E	S2-2007481

12 October - 23 October 2020, Elbonia

Source:	Nokia, Nokia Shanghai Bell, NTT DOCOMO

Title:	KI#2, Interim conclusion on solutions addressing NWDAF interactions in hierarchical NWDAF architecture

Document for:	Discussion/Approval

Agenda Item:	8.1

Work Item / Release:	FS_eNA_Ph2 / Rel-17

Abstract of the contribution: Proposes interim conclusions derived from the oberservations related to Key Issue #2.

# 1		Discussion

Based on the evaluation in S2-2007479, it is proposed that conclusions should agreed per category.

# 2	Proposal

It is proposed to include the following conclusions into TR 23.700-91.

*** Change (all new text) ***

## 8.2	Key Issue #2: Multiple NWDAF instances

### 8.2.1	Hierarchical NWDAF architecture (category #1)

The following principles are proposed for the normative work on "NWDAF interactions in hierarchical NWDAF architecture" and "Analytics aggregation in hierarchical NWDAF architecture":

- NWDAF architecture should allow for flexible number of layers.

- Extend the NWDAF Profile in NRF to include the new NWDAF capabilities and information (e.g. Aggregator functionality) such that the NWDAF service consumer can select the preferred NWDAF in case multiple NWDAFs are returned.

- Specify the capability for NWDAFs to support the aggregation of Analytics.

- Support the exchange of analytics metadata between NWDAFs required for the aggregation of Output Analytics.

- NWDAFs use the discovery mechanism from NRF as defined in clause 6.3.13, TS 23.501 [x] to identify other NWDAFs with certain capabilities (e.g. Analytics aggregation) and covering certain area of interest (e.g. providing data / analytics for specific TAI(s)).

- Avoid changes on the NWDAF service consumer (other than another NWDAF) beyond the change in principle b).

- Maintain the NRF behaviour of Rel-16, e.g., do not change the discovery mechanism provided by NRF.

### 8.2.2	Re-selection of NWDAF (category #2)

Editor's Note: placeholder for conclusions/recommendations related to category "re-selection of NWDAF"

### 8.2.3	Specific aspects of NWDAF interactions (category #3)

Editor's Note: placeholder for conclusions/recommendations related to category "specific aspects of NWDAF interactions"

*** End Change ***