3GPP TSG-WG SA2 Meeting #141E e-meeting 	S2-2007528

Elbonia, Oct 12 – 23, 2020	(revision of S2-200xxxx)

# Title:	[DRAFT] LS Reply on Counter of UEs Registering Network Slice

# Release:	Release 17

# Work Item:	FS_5MBS

Source:	[Nokia to be] SA2

To:	SA4

Cc:	CT4

Contact Person:

Name:	Thomas Belling

Tel. Number:

E-mail Address:	Thomas DOT Belling AT Nokia DOT com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

# Attachments: S2-2007297 (approved PCR against TR 23.757 on update of solution 33)

1. Overall Description:

SA2 is studying architectural enhancements for 5G multicast-broadcast services in TR 23.757.

Baseline architectures Architecture 1 in the TR contains an MSF-U controlled by an MSF-C, and architecture 2 contains a MBSU controlled by an MBSF for similar tasks. Those entities can be regarded as an evolution of the BMSC.

Solution 33 in the TR addresses the interactions between MSF-C/MBSF and MSF-U/MBSU. It proposes to extend PFCP (see TS 23.501 [2] subclause 5.8.2.11 for stage 2 requirements and TS 29.244 [17] for stage 3). This protocol is used successfully in the 3GPP ecosystem and provides capabilities e.g. for packet filtering, forwarding, repacketization of PDUs (i.e. manipulation of transport layer), event notifications and billing. It lacks media manipulation capabilities (for transcoding, FEC and ROHC) and will require related extensions; solution 33 contains proposals for a framework for such extensions.

Detailed requirements for media processing go beyond the SA2 expertise and require coordination with SA4.

A possible work split could be that SA2 defines high-level PFCP extensions to provide a framework to enable media processing (e.g. in TS 23.501 where the current stage 2 requirements for PFCP are defined), SA4 defines detailed stage 2 requirements for media processing and PFCP stage 3 extensions are defined on that basis either by CT groups (most likely CT4) or SA4.

2. Actions:

To SA4 group.

ACTION: 	SA2 asks SA4 to comment on the proposed work split for the definition of stage 2 interactions between MSF-C/MBSF and MSF-U/MBSU.

3. Date of Next TSG SA WG2 Meetings:

TSG-SA2 Meeting #142		16 – 20 Nov 2020		Elbonia