SA WG2 Meeting #141E	S2-2006998r03

Oct 12 – Oct 23, 2020, Elbonia

Source:	OPPO

Title:	KI#1, update of paging cause solution and conclusion proposal

Document for:	Approval/Discussion

Agenda Item:	8.10

Work Item / Release:	FS_MUSIM / Rel-17

Abstract of the contribution: The solution reuse the mechanism of how AMF determine paging cause, except only 1 bit indication is added in paging if AMF decides it is an important service based on operator policy (by default voice is important service).

# 1. Introduction

As discussed in S2-2006997, we propose that:

Leverage principles in solution#1, #2, and #10 as a way forward to make the paging cause indication really useful and reasonable, specifically:

1) 1-bit indication is added in paging if DL data is corresponding to important service (by default “voice service” is important service);

2) Based on receiving the paging, UE determines to negotiate a absence time to responds the paging in USIM-2 while keep connected in USIM-1;

3) For determination of important service, UE may provide service list (as a filtering rule) to tell network which service(s) is important service, otherwise by default voice service is always the important service.

The change proposal is described as below.

# 3. Proposal

* * * * First Change * * * *

## 6.x	Solution #x Solution #x: ”Important Service” paging cause

### 6.x.1	Introduction

The solution applies to Key Issue #1 "Handling of MT service".

The solution reuses the mechanism of Solution #1 regarding how AMF determines paging cause, except that only a 1 bit indication is added in the paging message if AMF decides that the paging is for an important service based on operator policy (by default voice is important service)

### 6.x.2	Functional Description

A1 bit indication is needed to indicate the important service i.e. voice service or others as UE requested (or based on operator policy). Optionally, AMF may determine services other than voice service as important services when the UE (NAS message) or UDM (subscription) piggyback the important service(s). However “Voice service” is always considered as the important service by default.

Table 6.x.2.x: Important Service indication

A

|  | Description |
| --- | --- |
| Important Service indication (1 bit) | The downlink traffic is corresponding to the important service (by default it is voice service). UE determines to respond to the paging based on the indication. |

AMF determines it is an important service based on HPLMN/DNN/5QI configuration and the ARP/PPI received from the SMF, or based on operator policy. Similar mechanism applies to NG-RAN for UE in RRC-Inactive and to MME for UE in EPS. How the AMF (or MME) determines the cause is the same as the mechanism in Solution #1.

NOTE-1: In future release the UE can negotiate additional services with the network that need to be considered as “important service”.

### 6.x.3	Impacts on services, entities and interfaces

UE, AMF, MME, RAN:

-	Similar impact as in Solution #1 for the case with only one Paging Cause.