3GPP TSG-WG SA2 Meeting #141E	S2-2007553r11

Elbonia, October 12 – 23, 2020	(Revision of S2-200xxxx)

Source:	NTT DOCOMO, Lenovo, Motorola Mobility, ZTE

Title:	Key Issue #1 solution evaluation and interim conclusions

Document for:	Approval

Agenda Item:	8.4

Work Item / Release:	FS_eNS_ph2 / Rel-17

Abstract of the contribution: This contribution evaluates solutions addressing the KI#1.

# Discussion

The following set of evaluation criteria are used to evaluate the proposed solutions for KI#1 to support network slice related quota on the maximum number of UEs:

1)	Comply to the working assumptions and requirements as described in clause 4 of TR23.700-40.

2)	Address all the issues identified in the KI#1 as described in clause 5.1.

3)	Impact to the UE

4)	Impact to the CN, the serving PLMN, the home PLMN, e.g. AMF, NSSF, UDM, CHF, PCF in term of signalling interfaces, control flows and internal logic etc. including changes to the semantics of existing services/parameters.

5)	Impact to (R)AN

6) 	Impact to O&M

# Proposal

It is proposed to capture the summary of the analysis and the table of solution comparison and the interim conclusions in the TR23.700-40.

********************************  START OF 1st CHANGES **************************************

# 7	Overall Evaluation

## 7.1	Evaluation on solutions of KI#1

From all 10 solutions proposed for KI#1 (Solution #1-#4, #8, #9, #15, #18, #19, #38), some may have a complete solution and some not. But overall, we can summarize that there are three main functionalities for supporting quota management on the maximum number of UEs as described below.

- NW Slice quota information storage functionality: This functionality is responsible for storing a NW Slice quota information, which includes one or more of the following information.

- An indication whether a S-NSSAI is subject to the network slice quota checking

- The maximum number of UEs for the S-NSSAI (either the global quota of the overall maximum number of UEs in a S-NSSAI or the local quota that is a subset of global quota)

- NW Slice quota management functionality: This functionality is responsible for managing and updating NW Slice quotas (either the global quota or the local quota) of the maximum number of UEs in a S-NSSAI, which includes one or more of the following functionalities.

- Monitoring for counting, collecting and updating the number of UEs that have been registered for a S-NSSAI that is subject to the network slice quota checking.

- Sending an indication to storage or enforcement function whether the network slice local quota has been reached.

- NW Slice quota enforcement functionality: This functionality is responsible for enforcing a network slice SLA, which includes one or more of the following functionalities

- Accept or reject the Requested S-NSSAI by taking the network slice quota and the current monitored number of the registered UEs into account.

- In case of the network slice quota (e.g. local quota that is a subset of global quota) has been reached, the function may request the update of the quota or rejects the request. In case of rejection, the function may provide a rejection cause and a back off time.

Table 7.1-1 illustrates 5GS system impacts for all solutions proposed for the KI#1. Moreover, it also compares where to place the afore-mentioned network slice quota functionalities in the 5G system.

Table 7.1-1: Key impacts of the solutions

|  | UE Impact | RAN Impact | 5GC CN Impact |  |  |  |  |  | Notes |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Existing NF (Note 8) |  |  | New NF or new service operation in existing NF |  |  |  |
|  |  |  | NW Slice Quota Storage | NW Slice Quota management | NW Slice Quota Enforce | NW Slice Quota Storage | NW Slice Quota Managment | NW Slice Quota Enforce |  |
| Sol#1 | Yes | No | UDR, PCF | PCF | PCF AMF | - | - | - | (Note 1) (Note 5) |
| Sol#2 | Yes | No | - | - | AMF | NSQ |  | NSQ | (Note 1) (Note 5) |
| Sol#3 | Yes | No | NSSF | NSSF | NSSF, AMF | - | - | - | (Note 1) (Note 5) |
| Sol#4 | Yes | No | NWDAF |  | AMF, PCF (roaming case) | - | - | - | (Note 1) (Note 5) |
| Sol#8 | Yes | No | O&M, AMF | O&M, AMF | OAM AMF | - | - | - | (Note 1) (Note 2) (Note 5) |
| Sol#9 | Yes | No | UDM, NWDAF | NWDAF, CHF | NWDAF AMF | - | - | - | (Note 1) (Note 5) |
| Sol#15 | Yes | No | - | - | AMF | - | - | - | (Note 3) |
| Sol#18 | No | No | SQM(NSSF for global) AMF for local- | AMF NSSF | AMF | - |  | - | (Note 4) (Note 5) (Note 7) |
| Sol#19 | No | No | NWDAF |  | - | QCF |  | QCF QEF | (Note 4) (Note 6) (Note 7) |
| Sol#38 | No | No | CHF | CHF | AMF, CHF | - | - | - | (Note 1) (Note 2) (Note 5) |
| NOTE 1: Solution requires a UE to support a (new/existing) rejection cause and a (new/existing) back-off timer due to the network slice quota has been reached. NOTE 2: Solution has an impact on O&M to support either a network slice quota monitoring and/or a network slice quota distribution (if any). Multi-vendor case is unclear on how it works. NOTE 3: Solution only addresses the aspect of back-off timer to be sent to the UE for network slice quota enforcement. NOTE 4: Solution does not describe whether the UE should be aware of a rejection cause due to a network slice quota has been reached. NOTE 5: Solution requires a change in both the H-PLMN and the V-PLMN to support a network slice quota monitoring and a network slice quota enforcement. NOTE 6: No descriptions of roaming aspect NOTE 7: Although the solution proposes a new NF, it also mentions that this new NF could be deployed together with existing NF. In such case, no new NF is needed. NOTE 8: Even when the existing NF is reused, the new NF service may need to be introduced. |  |  |  |  |  |  |  |  |  |

From Table 7.1-1 above, one can derive a commonality among those solutions as following:

-	All solutions do not require changes in RAN

-	All solutions propose to store an information related to the network slice quota information in the CN and the UE is not aware of it.

- 	All solutions propose to monitor the number of UEs accessing a network slice in the CN and to enforce the quota in the CN. Difference among those solutions are a) where to store the network slice quota information, b) where to place the network slice quota management and the network slice quota enforcement. There are two main approaches, one is to put them into an existing 5GC network function or introducing a new network function/new service operation to be deployed in the existing 5GC network function.

- 	Almost all solutions imply some form of interaction by the AMF with another function which performs the counting.

Furthermore, there are some other aspects that we could also draw some remarks, for example,

-	Rejection cause + Back-off timer: When a network slice quota in terms of number of UEs is overflown, and a CN NF sends a rejection to the UE requesting for a S-NSSAI, it is up to the Stage 3 to determine whether a new rejection cause or an existing rejection cause should be used. To suppress further signalling load for a subsequent request for the same S-NSSAI, the CN NF may provide a back-off timer to the UE. The CN NF responsible for sending a rejection for the requested S-NSSAI and optionally the back-off timer for the rejected S-NSSAI is the AMF.

********************************  START OF 2nd CHANGES **************************************

# 8	Conclusions

Editor's note:	This clause will capture conclusions from the study.

## 8.1	KI#1 Interim conclusion

To enable a 5GS to support network slice related quota on the maximum number of UEs, no change is required in the RAN. The following new functionalities in the 5GS are needed, i.e.

-	Storing of network slice related quota information: If a network slice is subject to a network slice quota checking on a maximum number of UEs, it is assumed that the O&M should have for this network slice a) the information that it is subject to the network slice related quota checking, and b) the information of the quota of maximum number of UEs (either the global quota or the local quota that is a subset of global quota). To enable the network slice related quota enforcement, this information is configured and stored to one or more network functions in 5GC.

Editor's note:	It is FFS which network function located in 5GC needs to be configured to store the network slice related quota information and how it gets the network slice related quota information.

.

-	Managing and updating the network slice related quota on maximum number of UEs registered for the network slice: This functionality is part of the 5GC and it manages the NW Slice quota of maximum number of UEs in a S-NSSAI, monitors the current number of UEs being registered for the network slice subject to a network slice quota checking .

Editor's note:	It is FFS which network function located in 5GC should manages the NW Slice quota of the maximun number of UEs in a network slice.

-	Enforcing the network slice related quota on the maximum number of UEs: This functionality is part of the 5GC and it controls the registration request on the S-NSSAI subject to the quota management. In case, the NW slice related quota on the maximum number of UEs is reached, this functionality in 5GC may report to the quota management function, may progress the registration with a differentiated charging. In case of rejection the function may provide a rejection cause and optionally with a back-off timer.

Editor’s note: It is FFS to select an appropriate NW Slice enforcement function among 5GC NFs (new or existing) shown in Table 7.1-1 by comparing their advantages

NOTE 3:	Whether to use an existing rejection cause or a new rejection cause, this is to be determined in Stage 3.