SA WG2 Meeting #141e	S2-2007491r01

# Oct 12th – 23rd, 2020 ; Elbonia                   		 (revision of S2-200xxxx)

Source: 	Nokia, Nokia Shanghai Bell

Title: 	New Solution #X: Determining the slice of the PDU Session supporting relayed traffic

Document for: 	Agreement (P-CR)

Agenda Item: 	8.8

Work Item / Release:	FS_5G_ProSe / Rel-17

Abstract of the contribution: New Solution #X: Determining the slice of the PDU Session supporting relayed traffic

# 1 Discussion

If slicing is supported, then procedures for communicating the required NSSAI between the Remote UE and Relay UE and the network are needed. This new solution adds that capability on top of the existing L3 relay solutions and especially relay authorisation in Solutions #46 and #47.

This solution includes two mutually exclusive alternatives.

# 2 Proposal

It is proposed to update TR 23.752 as follows

FIRST CHANGE

## 6.0	Mapping of Solutions to Key Issues

Table 6.0-1: Mapping of Solutions to Key Issues

|  | Key Issues |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Solutions | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 |
| 1 | X |  |  |  |  |  |  | X |
| 2 | X |  |  |  |  |  |  | X |
| 3 | X |  |  |  |  |  |  |  |
| 4 | X | X |  |  |  |  |  |  |
| 5 |  | X |  |  |  |  |  |  |
| 6 |  |  | X |  |  |  |  |  |
| 7 |  |  | X |  |  |  |  | X |
| 8 |  |  |  | X |  |  |  |  |
| 9 |  |  |  | X |  |  |  |  |
| 10 |  |  |  | X |  |  |  |  |
| 11 |  |  |  | X |  |  |  |  |
| 12 |  |  |  |  | X |  |  |  |
| 13 |  |  |  |  |  |  | X |  |
| 14 |  |  |  |  |  |  | X |  |
| 15 |  |  |  |  |  |  | X |  |
| 16 |  |  |  |  |  |  |  | X |
| 17 |  |  |  |  |  |  |  | X |
| 18 | X |  |  |  |  |  | X |  |
| 19 | X |  | X |  |  |  |  |  |
| 20 |  | X |  |  |  |  |  |  |
| 21 |  | X |  |  |  |  |  |  |
| 22 | X | X |  |  |  |  |  |  |
| 23 |  |  | X |  |  |  |  |  |
| 24 |  |  | X |  |  |  |  |  |
| 25 |  |  | X |  |  |  |  |  |
| 26 |  |  | X |  |  |  |  |  |
| 27 |  |  | X |  |  |  |  |  |
| 28 | X |  | X |  |  |  |  |  |
| 29 |  |  | X |  |  |  |  |  |
| 30 |  |  | X |  |  |  |  | X |
| 31 |  |  |  | X |  |  |  |  |
| 32 |  |  |  | X |  |  |  |  |
| 33 | X |  |  | X |  |  |  |  |
| 34 |  |  |  |  |  |  | X |  |
| 35 |  |  | X |  |  |  |  | X |
| 36 |  |  |  | X |  |  |  | X |
| 37 |  | X |  |  |  |  |  |  |
| 38 |  |  | X |  |  |  |  |  |
| 39 | X |  |  |  |  |  |  |  |
| 40 |  |  | X |  |  |  |  |  |
| 41 |  |  | X |  |  |  |  |  |
| 42 |  |  | X |  |  |  |  |  |
| 43 |  |  | X |  |  |  |  |  |
| 44 |  |  | X |  |  |  |  |  |
| 45 |  |  | X |  |  |  |  |  |
| 46 |  |  | X |  |  |  |  |  |
| 47 |  |  | X |  |  |  |  |  |
| 48 | X |  | X |  |  |  |  |  |
| 49 |  |  |  | X |  |  |  |  |
| 50 |  |  |  | X |  |  |  |  |
| 51 |  |  |  |  | X |  |  |  |
| 52 |  |  | X |  |  |  |  |  |

NEXT CHANGE

## 6.46	Solution #46: UE-to-Network Relay authorization

### 6.46.1	Description

This is a solution for key issue #3, UE-to-Network Relay.

The solution is intended to complement the baseline L3 UE to network Relay solutions when relay authorisation is needed e.g. in solutions described in clause 6.6 and Solution #23 "End-to-End security for Layer-3 UE-to-Network Relay using N3IWF" described in clause 6.23. See also Figure 6.46.1-2 below.

A 5G ProSe UE-to-Network relay is called "Relay UE" in this solution.

Figure 6.46.1-1: Architecture model using 5G ProSe 5G UE-to-Network L3 Relay solution (Generic case)

Figure 6.46.1-2: Architecture model for Layer-3 UE-to-Network Relay with support of N3IWF

When the Relay UE and the Remote UE have a subscription to the same HPLMN, the solution works as follows;

1.	The Relay UE registers with 5GS and obtains service for itself (as in R16 specification).

2.	The Remote UE tries to establish a PC5 connection to the Relay UE. During the PC5 establishment, the Remote UE provides its SUCI to the Relay UE when it requests UE-to-network relaying from the Relay UE over PC5.

3.	The Relay UE contacts its AMF and requests (NAS message) an authorization for relaying the remote UE providing the SUCI of the remote UE.

NOTE 1:	The stage 3 NAS message to use is to be defined by CT WG1.

4.	If AUSF corresponding to the Remote UE's target PLMN is not discovered already, the AMF in Intermediate PLMN discovers AUSF as specified in TS 23.501 [6] clause 6.3.4 and sends a Request to AUSF for authorization, providing the SUCI of the Remote UE.

The checking whether the Remote UE is authorised to use UE-to-Network relaying may consider the following aspects:

a)	UDM Subscription data for the Remote UE may contain information on which UE (possibly any, members of an IMSI Group, a list of SUPI or GPSI) the Remote UE accepts to use as a Relay UE.

b)	HPLMN may have policies to fetch from a third-party AAA the information whether the Remote UE (identified by its GPSI) accepts to be relayed by a Relay UE identified by its GPSI.

During this step the HPLMN may control whether relaying via the Intermediate PLMN serving the Relay UE is allowed.

NOTE 2:	The Intermediate PLMN need not be the same as the Remote UE target PLMN. If the PLMN identified by Remote UE SUCI differs from the MCC and MNC of the Intermediate PLMN, then the AMF needs to discover AUSF in the target PLMN of the Remote UE.

Similar operations apply towards the Relay UE HPLMN to authorise its capability to act as a Relay UE.

5.	The AUSF authenticates the Remote UE via the AMF. The AUSF uses the SUCI of the Remote UE to get the necessary credentials.

PC5 related security parameters are derived during the authentication procedure. This may be useful in the generic configuration of Figure 6.46.1-2 where no secured (IPSec) tunnel protects the traffic over PC5

6.	Optionally, the Remote UE may use the procedure in 6.46.2.2 as part of its PDU Session Establishment in order to identify which NSSAIs can be supported in the Relay Network (Underlay network). Based on this knowledge, the Remote UE may prune its NSSAI(s) and only request NSSAIs that are compatible with those supported by the Relay Network.

### 6.46.2	Procedures

### 6.46.2.1	Relay authorisation

The connection of a Remote UE via a Relay UE works according to high level steps described in to Figure 6.46.2.1-1.

Figure 6.46.2.1-1: UE-to-Network Relay authorization when Relay UE and Remote UE have subscription in different PLMN (s)

1.	The Relay UE registers (see TS 23.502 [8] clause 4.2.2.2).

2.	Remote UE establishes PC5 connection towards the Relay UE. When requesting UE-to-Network relaying over PC5, the Remote UE provides its SUCI to the Relay UE.

NOTE 1:	How the remote UE discovers the Relay UE is independent of this solution. Discovery can be based on methods defined in TS 23.287 [5] or in other solutions of this TR.

3.	The Relay UE requests authorization for UE-to-Network relaying from its serving AMF and provides the SUCI of the Remote UE.

NOTE 2:	It assumed that UE-to-Network relay authorisation is NAS signalling and it is expected that CT groups specify the stage 3 messages.

4.	The AMF requests for authorisation for the Remote UE to use UE-to-Network relaying via the Relay UE. The AMF includes the SUCI of the Remote UE and the GPSI of the Relay UE.

5.	The AUSF of the Remote UE may authenticate the Remote UE.

The authentication is signalled transparently via the Relay UE and the AMF serving it. Part of the signalling may be encrypted and can be decrypted by the Remote UE and the AUSF.

At the end of the authentication procedure the AUSF has got the SUPI and GPSI of the Remote UE;

Both the Remote UE and its AUSF derive security (e.g. ciphering) parameters during the authentication of the Remote UE for later use for PC5 security.

6.	The AUSF of the Remote UE checks whether the Remote UE identified by Remote UE GPSI is authorised to use UE-to-Network relaying via the Relay UE identified by Relay UE GPSI. The AUSF includes the GPSI of both Relay UE and Remote UE. Two alternatives can be supported:

6a	The AUSF can request the UDM to verify against Subscription data of the Remote UE whether the Remote UE is authorised to use UE-to-Network relaying over the Relay UE.

6b	Based on HPLMN policy, the HPLMN can delegate it to a third party AAA server (identified by the domain part of the GPSI of the Remote UE) to verify whether the Remote UE (identified by its GPSI) is authorised to use UE-to-Network relaying via Relay UE (identified by its GPSI).

If authorisation succeeds, the AUSF continues in step 8. Otherwise, the AUSF indicates authorisation failure to the AMF, which rejects the UE-to-Network relaying request based on the response it received from the AUSF.

7.	The AUSF responds to Relay authorisation Request and provides Result and security (e.g. ciphering) parameters and the Remote UE GPSI for the Intermediate PLMN.

8.	The AMF requests for authorisation for the Relay UE to support UE-to-Network relaying for the Remote UE. The AMF includes the Relay UE GPSI and the Remote UE GPSI.

If the Relay UE is authorised to act as Relay UE for the Remote UE, the AMF continues in step 5. Otherwise, the AMF rejects the UE-to-Network relaying request.

9.	The AMF sends the relay authorisation including the authorisation result and the security (e.g. ciphering) parameters.

### 6.46.2.2	Slice Mapping at the PDU session Establishment Impacts on services, entities and interfaces

### 6.46.2.2.0	Overview

This clause aligns mapping of slice requests from the Remote UE with the slices that can be supported by the Relay UE and the 5GS network serving the Relay UE in N3IWF relaying solutions including #46 and #47.

2 variants are described:

-	Slice mapping is done at the PDU Session Establishment request from Relay UE (clause 6.46.2.2.1): there is no further signalling between Relay UE and its AMF than the PDU Session Establishment request issued by the Relay UE to relay traffic of the Remote UE

-	Slice mapping is done in a dedicated procedure between the Relay UE and its AMF that takes place before a possible PDU Session Establishment request from Relay UE (clause 6.46.2.2.2). This solution allows to share the same relaying PDU Session between different Remote UE;

Only one of these variants is to be selected for the normative phase. The solution proposes to select the variant described in clause 6.46.2.2.1

A Remote UE  desires to access slices at the overlay network (e.g. slice A and slice B),  i.e. at the network that provides actual service to this Remote UE, but the Remote UE needs the connectivity of an underlay network to reach the N3IWF providing access to its overlay network.

In order to reach the N3IWF over the underlay network, the Remote UE needs to ensure a PDU Session has been established at the underlay network. This PDU Session

-	is unique regardless of the number of slices needed by the Remote UE

-	should be associated with a slice from the underlay network that best meets the slicing requirements of the Remote UE desires to access, (slice A and slice B in the example)

To support this feature, the solution assumes:

-	The Remote UE sends a request to the Relay UE over PC5 to obtain connectivity and it provides its intended requested NSSAI listing all S-NSSAIs (slice A and slice B  in the  example) that will be presented as requested NSSAI within the NAS Registration request message that it plans to send to the overlay network (5GC/AMF of the Remote UE).

-	Then the Relay UE sends a request (via NAS) to its AMF to translate the intended requested NSSAI received over PC5 (from the Remote UE) into the (unique) S-NSSAI to be used by the PDU Session that is being used by the Relay UE for the relayed traffic of the Remote UE.

-	The subscription of the Relay UE and/or the capabilities of the access where it is accessing to the (underlay) network may not allow to support all slices within the intended requested NSSAI received over PC5 (from the remote UE). The AMF of Relay UE answers back with the Remote UE slices it can support (NSSAI Allowed for relaying) that the Relay UE forwards to the to the Remote UE. Based on this, the Remote UE may decide to prune out slices from its list of Requested S-NSSAI when issuing a Registration request to the overlay network (no need to include NSSAI in the Registration request sent to the overlay network for a slice that is not supported by the underlay network).

#### 6.46.2.2.1	Slice mapping at the PDU Session Establishment request from Relay UE

The figure 6.26.2.2.1-1 focuses on Slice mapping at the PDU Session Establishment request from Relay UE. It assumes that there is a PDU session for UE to network traffic relaying per remote UE; The procedure goal is to inform Remote UE which NSSAIs can be supported in the Relay UE and Relay network.

Upon PC5 request from the Remote UE, the Relay UE requests a PDU Session Establishment for the traffic of the Remote UE, providing the PLMN ID and the intended requested NSSAI received over PC5 instead of a S-NSSAI in case of a generic PDU Session. The AMF selects the appropriate S-NSSAI for the PDU Session, that is provided to the SMF and is eventually included as part of the PDU Session accept sent to the Relay UE. The whole procedure may take place at the same time as the Relay authorization procedure described in solution #46 and #47;

Figure 6.46.2.2.1-1: Slice mapping at the PDU Session Establishment request from Relay UE

1	The Relay UE registers and gets service (PDU Sessions) for itself. The Relay UE registration may allow the UE to access to slices (S-NSSAI) eligible to support the traffic of Remote UE(s).

2	The Remote UE tries to establish a PC5 connection to the Relay UE. During the PC5 establishment the Remote UE provides its PLMN ID and its intended Requested NSSAI.

NOTE 1:	How the remote UE discovers the Relay UE is independent of this solution. Discovery can be based on methods defined in TS 23.287 [5] or in other solutions of this TR.

NOTE 2:	The remote UE may also provide other information as described in solution #46/#47 of this TR.

3	The Relay UE sends a PDU Session establishment request providing the PLMN ID and the intended Requested NSSAI received from the remote UE.

NOTE 3:	The PLMN ID and the intended Requested NSSAI are NAS information that the AMF can handle like the target S-NSSAI of a PDU Session.

As described in solution #46/#47 the PDU Session establishment may also contain other information to be used to carry out an authorization for relaying the Remote UE.

4 	The AMF may proceed with other authorization checks as described in in solution #46/#47 of this TR.

Based on the PLMN ID and on the intended Requested NSSAI of the Remote UE, on local configuration, and on Allowed NSSAI of the Relay UE, the AMF determines the S-NSSAI to be used by the PDU Session of the Relay UE to carry traffic of the Remote UE and determines the corresponding set of S-NSSAI(s) (of the overlay network i.e. of the Remote UE) Allowed for relaying (this is a subset of the intended Requested NSSAI of the Remote UE).	.

NOTE 3:	as the AMF takes into account the Allowed NSSAI of the Relay UE, it takes into account the subscription of Relay UE as well as the capabilities of the 5G RAN serving the Relay UE; The local configuration of the AMF uses the PLMN ID of the Remote UE to translate the intended Requested NSSAI of the Remote UE into a list of S-NSSAI of the underlay network (of the 5GC of the Relay UE).

5	The AMF proceeds with the establishment of the PDU Session (PDU Session establishment as defined in 23.502 § 4.3.2) with the S-NSSAI determined above.

The AMF answers to the request sent by the Relay UE in step 3, providing (on top of information already defined in 3GPP TS 23.502 / 24.501) the set of S-NSSAI(s) (of the overlay network i.e. of the Remote UE) Allowed for relaying;

6	The Relay UE answers to the Remote UE. The Relay provides the set of S-NSSAI(s) (of the overlay network i.e. of the Remote UE) Allowed for relaying.

7	Based on the set of S-NSSAI(s) Allowed for relaying received from the Relay UE, the Remote UE prunes out slices from its Requested NSSAI when (later on) issuing a registration request to the overlay network;

The Remote UE can then select a N3IWF and via this N3IWF registers on its own network (Overlay network) and requests PDU Sessions from the overlay network.

#### 6.46.2.2.2	Slice mapping using a NAS dedicated signalling

In this version of the procedure, Upon PC5 request from the Remote UE, the Relay UE requests a dedicated authorization procedure, providing the PLMN ID and the intended requested NSSAI received over PC5. The AMF selects the appropriate S-NSSAI for the PDU Session, that is provided back to the Relay UE. The whole procedure may take place at the same time as the Relay authorization procedure described in solution #46 and #47;

Then the Relay UE determines whether:

-	to initiate a PDU Session Establishment request for the traffic of the Remote UE, or

-	to reuse an existing PDU session for the traffic of the Remote UE

Figure 6.46.2.2.2-1: Slice mapping using dedicated NAS message

Steps 1 and 2 are as defined for Figure 6.46.2.2.1-1

3 	The Relay UE sends a dedicated NAS message requesting relaying authorization, providing the PLMN ID and the intended Requested NSSAI received from the remote UE.

4	As step 4 of Figure 6.46.2.2.1-1

5a 	the AMF answers to the Relay UE request in step 3 , providing (possibly on top of information already defined in solution #46 / #47) the S-NSSAI for the PDU Session of the Relay UE to carry traffic of the Remote UE and the set of S-NSSAI(s) (of the overlay network i.e. of the Remote UE) Allowed for relaying;

5b	The Relay UE the Relay UE may:

-	initiate a PDU Session Establishment request for the traffic of the Remote UE, or

-	reuse an existing PDU session for the traffic of the Remote UE

Then a PDU Session establishment / modification procedure may take place as defined in 23.502 per R16 specifications

6	As step 6 of Figure 6.46.2.2.1-1

7	As step 7 of Figure 6.46.2.2.1-1

### 6.46.3	Impacts on services, entities and interfaces

This solution impacts the following system entities.

Remote UE:

-	Capability to provide SUCI for UE-to-Network Relay UE for authorisation

-	Capability to support the authentication procedure via Relay UE

-	(NSSAI pruning only) PC5 change to exchange slicing information between Remote UE and Relay UE: the Remote UE provides its PLMN ID and the intended Requested NSSAI(s) in the PC5 establishment request.

Relay UE:

-	Capability to request authorisation for Remote UE identified by SUCI

-	Capability to relay the authentication procedure towards the Remote UE

-	(NSSAI pruning only) Configured to establish a PDU session for relaying providing the PLMN ID and the intended Requested NSSAI(s) received from the remote UE.

-	(NSSAI pruning only) PC5 change to exchange slicing information between Remote UE and Relay UE

AMF:

-	Capability to request UE-to-Network relay authorisation from Remote PLMN

-	Capability to request UE-to-Network relay authorisation from Intermediate PLMN

-	(NSSAI pruning only) Configured to determine the S-NSSAI to be used by the PDU Session of the Relay UE to carry traffic of the Remote UE and the set of S-NSSAI(s) (of the overlay network i.e. of the Remote UE) Allowed for relaying

AUSF:

-	Capability to interact with third party AAA server if PLMN operator delegates relay authorisation to third party.

-	PC5 change to exchange slicing information between Remote UE and Relay UE: the Remote UE provides its PLMN ID and the intended Requested NSSAI(s) in the PC5 establishment request.