3GPP TSG-WG SA2 Meeting #141E	S2-2007024r01

Oct 12 – Oct 23, 2020, Elbonia	(revision of S2-2005399, merge of S2-2007385 and S2-2007580)

Source:	China Mobile, CATT

Title:	KI #1: Interim conclusions

Document for:	Approval

Agenda Item:	8.1

Work Item / Release:	FS_eNA_ph2 / Rel-17

Abstract: This contribution proposes interim conclusions on Key Issue #1.

# 1. Introduction

This contribution proposes interim conclusions on KI #1: Logical decomposition of NWDAF and possible interactions between logical functions.

# 2. Text Proposal

It is proposed to capture the following new text in TR 23.700-91.

* * * * Start of Changes * * * *

# 8	Conclusions

Editor's note:	This clause is intended to list conclusions that have been agreed during the course of the study item activities. This should also capture the guiding principles and documentation approach for creating CRs to normative specifications within the responsibility of SA WG2.

## 8.1	Key Issue #1: Logical decomposition of NWDAF and possible interactions between logical functions

Interim conclusion:

NWDAF can be composed of Training Function and Analytics (Inference) Function:

-	 The Training Function could expose training service (e.g. providing trained model to the NWDAF);

-	 The Analytics (Inference) Function could exposeanalytics service (e.g. providing analytics information to the NF or OAM).

A NWDAF can be decomposed into Model Training Function (MTF) and/or Analytics Function (AnF). These decomposed functions (i.e. MTF and AnF) can be:

- Standalone functions with a standardized interface(s) between these functions and NF consumers; or

- Colocated with the NWDAF.

NOTE 1:	It can also be possible to identify other standalone NF(s) based on other KI results, e.g. Data Dollection function from KI#11.

For the NWDAF functionality decomposition method and procedure , solution 7 is selected as a basis for the normative work.

Messaging framework can be used between the decomposed functions.

The Analytics exposure by the AnF continues to be offered by NWDAF as in Rel-16.

NOTE 2: The definition of model provisioning service and its parameters between the MTF and AnF is dependant of the conclusion of Key Issue #19: Trained data model sharing between multiple NWDAF instances.

Pre-trained ML model storage and provisioning to NWDAF by the MTF is out of the scope of 3GPP.

MTF registers the supported ML model information to NRF. If AnF does not have a proper ML model for the requested analytics, the AnF will discovery the MTF from NRF to collect the required ML model.

* * * * End of Changes * * * *