3GPP TSG-WG SA2 Meeting #141E e-meeting	S2-2007114

October 12 – 23, 2020	(merge S2-2007246 and S2-2007114)

Source:	Ericsson

Title:	KI#4- Conclusion

Document for:	Approval

Agenda Item:	8.1

Work Item / Release:	FS_eNA_ph2 / Rel-17

Abstract of the contribution: This contribution proposed some high level conclusion for KI#4.

# Discussion

Proposal to add some high level conclusions on KI#4 with the aim to identify those aspects that require normative work in Rel-17.

# Proposal

It is proposed to include the conclusion in the eNA_ph2 TR

******************************** START OF CHANGES **************************************

## 8.4 	Key Issue #4: Remaining aspects on how to ensure that slice SLA is guaranteed

For KI#4, it is proposed that the following aspects from Solution #2 are considered for the normative work for slice load distribution mechanism based on determination by NWDAF of slice service experience and slice load analytics:

- On how to leverage the Analytics Id on Observed Service Experience and Slice Load: existing Analytics on Observed Service Experience for a Network Slice are reused as defined in Rel-16 TS 23.288 [5] and Analytics on Slice load needs enhancements to define the resource usage within the slice, and the number of UE registrations and the number of PDU sessions established in the slice.

- The input data and output analytics for the Analytics Id on Observed Service Experience remain unchanged. The input data and output analytics for Slice Load analytics are defined in Solution#2 clause 6.2.1.3.1 and 6.2.1.3.2.

- The NWDAF service consumers are NSSF,AMF and optionally OAM; leveraging the analytics information, NSSF and AMF can take actions as instructed in the Network Slice or Network Slice instance Restrictions not to accept any more registration or PDU sessions until this restriction is lifted.

- The NWDAF subscribes to OAM to be informed on the creation or modification of a slice or slice instance as described in clause 6.2.1.2. Feasibility of the procedure to be checked with SA WG5.

******************************** END OF CHANGES **************************************