3GPP TSG-WG SA2 Meeting #141E	S2-2007387r01

Elbonia, October 12 – 23, 2020	(revision of S2-2007387+6860+7015+7278+7292+7642+7724)

Source:	Qualcomm Incorporated

Title:	Evaluation for Key Issue #8: UE data as an input for analytics generation

Document for:	Approval

Agenda Item:	8.1

Work Item / Release:	FS_eNA_Ph2 / Rel-17

Abstract: This contribution proposes the evaluation for KI#8.

# 1. Introduction

This contribution proposes the evaluation for KI#8: UE data as an input for analytics generation.

# 2. Text Proposal

It is proposed to capture the following new text in TR23.700-91.

* * * * Start of Changes * * * *

# 7	Overall Evaluation

Editor's note:	This clause will provide evaluation of different solutions.

## 7.1	Key Issue #8: UE data as an input for analytics generation

According to table 6.0-1, solution #27, #28, #29, #62, #63, #64 and #65 are proposed for Key Issue #8.

There are two main aspects for Key Issue #8, one is the UE data collection procedure and the other is what parameters are feasible to be collected from UE.

For UE data collection procedure, there are two sets of solutions proposed.

User plane solution set:

- Solution #27: UE establishes a user plane connection to MNO AF which is the entity operated by operator. Application layer data request and data report are performed between UE and MNO AF via user plane. The MNO AF provides the data after possible processing like aggregation, normalization, etc. to the NWDAF using Naf_EventExposure.

- Solution #64: UE establishes a user plane connection to Application Server, Application layer data request and data report are performed between UE and Application Server via user plane. Application Server forwards the data after possible processing to a MNO AF which in turn exposes it to the NWDAF using Naf_EventExposure.

- Solution #29 refers the UE data collection procedure to solution #27.

Control Plane solution set:

- Solution #28: The data request message is encapsulated in a transparent container sent from NWDAF to AMF and then AMF forwards the container to UE via NAS message. UE data reporting is also encapsulated in a transparent container sent to AMF via NAS message and AMF forwards the container to NWDAF.

Other solutions do not explicitly propose the data collection procedure and only refer to the conclusion of the key issue.

For user plane solution, the data collection / report procedure is performed in application layer and treated as data transmission in PDU session, for control plane solution, it increases the NAS siganlling transmission which has more impact on both UE and network.

For the proposed parameters that collected from UE, there are several different proposals.

- Application layer information:

  - solution #27 and solution #64, general solutions for application layer parameters. The solutions support both 3GPP defined service and non-3GPP defined service. For the non-3GPP defined service, which application parameters that collected from UE does not need to define in 3GPP spec.

  - Solution # 63: Service experience contribution.

- Mobility related information: solution #29, which includes the route, destination, average speed information.

- Application status information: solution #29, which indicates flow of the application is in a foreground status or background status. It is not clear how does this information relevant to the network load balance analytics. Network allocates resources for the application is based on the QoS parameters allocated for the QoS flow, the foreground status and background status shall not impact the network resource allocation.

- Policy enforcement related information:

  - Solution #28: UE reports the applied RSD information to NWDAF, PCF can use the analytics report to allocate more proper RSD for UE.

  - Solution # 62: NWDAF collects data from the UE about which of several WLANSP rules and groups of selection criteria is directly involved in the WLAN selection. It also collects data that can measure the quality and performance of the WLAN connection from other 5GC NFs. NWDAF reveals the correlation between WLANSP that UE applied when selecting the WLAN and the resulting WLAN performance. The PCF can use the analytics results to update the WLANSP to improve WLAN performance.

The justification of collecting these policy enforcement related information is not clear,  there are multiple inputs and situations that would affect the applicability of the URSP rules, some rules may not be valid due to network access availability, the URSP enforcement may also depends on the URSP engine implementation. The UE should be provided with the superset of rules that is expected to be used in a particular region beforehand. If the UE does not apply a specific URSP rule does not mean the allocated rule is not going to be matched in the future. The proposal of URSP enforcement in solution #28 and the Applied WLANSP rule in solution #62 does not justify the useful for the proposed analytics feature. For solution #62, it is already supported to collect the WLAN SSID by MDT/SON solution, NWDAF can analytics the result based on the already supported parameters from OAM and UPF and there is no need to collect other proposed parameters from UE.

Solution #65 considers the trigger about UE data collection, UE reports the supported type of data to AMF during registration, NWDAF will collect the number of UEs that supporting a specific type of data collection and decide whether start or stop data collection from UEs. It is not clear whether the proposed solution can be applied to the user plane solution and what are the type of data that reported from the UE.

* * * * End of Changes * * * *