SA WG2 Meeting #141-e	S2-2007469

October 12th -23rd, Online	(revision of)

Source:	Nokia, Nokia Shanghai Bell

Title:	Overall MUSIM interim conclusions proposal

Document for:	Approval

Agenda Item:	8.10

Work Item / Release:	FS_MUSIM

Abstract of the contribution: xxx

# Discussion

This paper attempts to provide some interim conclusion that considers these aspects:

- For E-UTRA a NAS based solutions are required. These should also be available for the 5GS (no reason to disallow them in 5GS), even if then we agreed to support some RRC / NR radio interface impacting solutions as an alternative for the 5GS. Also, we assume no EUTRA impact applies to both EPS and 5GS.Should TSG RAN WGs come to a different conclusion, some NR-specific solution that are impacting the RRC level could also be adopted for EUTRA connected to the 5GS.

- For paging collision, we focus on a proposal that has impact just on RAN/Core interface and UE but no impact on the radio interface. Whether RAN also defines other approaches should not be in the way of supporting this approach in both EPS and 5GS.

- We do not propose a conclusion per KI, rather we prefer defining a comprehensive conclusion covering all aspects. This should remove the diatribe on whether certain solutions should be evaluated and concluded on as part of a KI or another.

# Proposal

The following clarifications are proposed to be added to TR 23.761

# Proposed changes

# 8	Conclusions

## 8.1	Key Issue #1: Handling of Mobile Terminated service with Multi-USIM device

The following interim conclusions are agreed for the baseline functionality:

- 	Paging filtering in the CN and RAN based on filters indicated by UE in MUSIM assistance information. Paging filters are set based on local UE policies and user settings and preferences.

Editor's note:	The granularity of services indicated with the paging filters is FFS.