SA WG2 Meeting #141E	S2-2007481r11

12 October - 23 October 2020, Elbonia

Source:	Nokia, Nokia Shanghai Bell, NTT DOCOMO, Samsung, ETRI

Title:	KI#2, Interim conclusion on solutions addressing NWDAF interactions in hierarchical NWDAF architecture

Document for:	Discussion/Approval

Agenda Item:	8.1

Work Item / Release:	FS_eNA_Ph2 / Rel-17

Abstract of the contribution: Proposes interim conclusions derived from the oberservations related to Key Issue #2.

1		Discussion

Based on the evaluation in S2-2007479, it is proposed that conclusions should agreed per category.

2	Proposal

It is proposed to include the following conclusions into TR 23.700-91.

*** Change (all new text) ***

8.2	Key Issue #2: Multiple NWDAF instances

8.2.1	Hierarchical NWDAF architecture (category #1)

A single option in normative phase is preferred. Solutions or parts of solutions described in evaluation clause 7.2.2 shall be used to create one option with possible multiple deployment variants. The following principles are proposed for the normative work on "NWDAF interactions in hierarchical NWDAF architecture" and "Analytics aggregation in hierarchical NWDAF architecture":

- NWDAF architecture should allow for a hierarchy with flexible number of layers.

- Extend the NWDAF Profile in NRF to support registering NWDAF capabilities and information for hierarchy such that the NWDAF service consumer can select the preferred NWDAF in case multiple NWDAFs are returned based on its implemented selection criteria and registered capabilities and information for hierarchy in NRF.

- Specify the capability for certain NWDAFs to support the aggregation of Analytics (per analytics ID) from other NWDAFs.

- Support the exchange of analytics metadata between NWDAFs when required for the aggregation of Output Analytics.

- NWDAFs use the discovery mechanism from NRF as defined in clause 6.3.13, TS 23.501 [x] to identify other NWDAFs with certain capabilities (e.g. Analytics aggregation), covering certain area of interest (e.g. providing data / analytics for specific TAI(s)).

- Avoid changes on the NWDAF service consumer (other than another NWDAF) beyond the change in principle b).

- Maintain the NRF behaviour of Rel-16, e.g., do not change the discovery mechanism provided by NRF.

- NWDAFs can be co-located with a specific set of NFs within 5GC, whereby the NWDAFs may collect data from that set of NFs or generate analytics based on input data coming from that set of NFs. In this case, registration and discovery mechanisms are the same as in bullet items #b and #e.

8.2.2	Re-selection of NWDAF (category #2)

Editor's Note: placeholder for conclusions/recommendations related to category "re-selection of NWDAF"

8.2.3	Specific aspects of NWDAF interactions (category #3)

Editor's Note: placeholder for conclusions/recommendations related to category "specific aspects of NWDAF interactions"

*** End Change ***