3GPP TSG-SA2 Meeting #141-e	S2-2007562r1

# Online, , 12th Oct 2020 - 23rd Oct 2020

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.502 | CR | 2400 | rev | 1 | Current version: | 16.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Corrections to Emergency Configuration Data for enable the selection of diffrent emergency gateways based on UE capabilities |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai-Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S2 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_Ph1, TEI16 |  |  |  |  |  | Date: |  |  | 2020-08-13 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Clause 4.11.0a.4 (PGW Selection for System interworking procedures with EPC) states When the UE performs emergency Attach or requests to establish an emergency PDN connection, the MME determines, based on the UE's support for 5GC NAS and local configuration, if an emergency PGW-C+SMF or a standalone emergency PGW-C should be selected, which also means that an emergency PGW-C+SMF may need to be configured in Emergency Configuration Data in the MME. The above is for PGW selection when UE initiates emergency service over EUTRAN. The same principle shall apply to PGW selection when UE initiates emergency service over EPC/ePDG, however, the necessary impact is missing in clause 4.11.4.3.3a “Initial Attach for emergency session (GTP on S2b)” |
| Summary of change: |  | Clarify in clause 4.11.4.3.3a that a separate emergency PDN GW-C/SMF identity may need to be configured. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Emergency PGW-C+SMF is not possible to be selected when UE initiate emergency service from EPC/ePDG thus mobility to 5GS may not be possible. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.11.4.3.3a |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

1st change

##### 4.11.4.3.3a	Initial Attach for emergency session (GTP on S2b)

The procedure in TS 23.402 [26] clause 7.2.5 applies with the following modification or modification:

-	Step 3 (Create Session Request): ePDG determines if interworking with 5GC is supported based on UE's 5G NAS capability and local configuration. If the ePDG should select an emergency PGW-C+SMF than a standalone emergency PGW-C, an emergency PGW-C+SMF identity needs to be added to the Emergency Configuration Data as specified in clause 4.5.7.2.3 of TS 23.402 [26].

In PGW-C+SMF, only one S-NSSAI is configured for the emergency APN.

-	Step 6 (Create Session Response), compared to step D.1 of clause 4.11.4.3.3, PGW-C+SMF does not include S-NSSAI in PCO for emergency PDN connection.

End of changes