3GPP TSG-WG SA2 Meeting #141E e-meeting	S2-2008093

Elbonia, October 12 – 23, 2020	(revision of S2-2007530r05)

Source:	Samsung

Title:	KI#1, Sol#6: adding leave and release procedures

Document for:	Approval

Agenda Item:	8.9

Work Item / Release:	FS_5MBS / Rel-17

Abstract of the contribution: This contribution proposes to add MBS Service leave and release procedures to solution #6.

Discussion

MBS Service leave and release procedures are added.

Proposal

It is proposed to add the following changes into TR 23.757.

******************************** START OF CHANGES*********************************

6.6	Solution #6: Multicast service initiation

6.6.1	Functional description

This solution addresses Key Issue 1 and proposes a multicast service initiation procedure based on the MBS reference architecture alternative 2 (see clause A.2).

When the AF requests the MBSF to provide a multicast service, the AF provides the multicast service related information that may include service type, target service area, target service UE group, QoS requirement information, where the service type indicates which kinds of MBS services e.g. public safety or V2X services, etc., and target service area indicate the area in which the contents provider wants to provide service.

NOTE:	Based on the service type, target service area etc., an appropriate MBSF/MBSU can selected.

The MBSF setups the MBS service session for the multicast service.

AF or MBSF announces the MBS services, which deliver the MBS service specific parameters e.g. DNN info, MBS service session ID, target service UE group ID, target service area, etc. to UEs. Then, the UE in MBS service area may join the multicast service for the MBS service session via PDU session establishment/modification procedure, the MB-SMF may verify the UE by its SM subscription information and notify the UE's multicast joining request to the MBSF.

The MBSF request the MB Session setup towards the MB-SMF(s) involved in the MBS session, which may get authorization for the QoS profile from the PCF.

The MB-SMF sets up the MB Session based on the MBS service session ID, target service UE group ID, etc., and associates with the PDU session. During the PDU session establishment/modification procedure, the MB-SMF sets up the shared N3 tunnel between NG-RAN and MB-UPF and the UE obtains the radio resource information to receive the MBS traffic. If the UE wants or is out of MBS area, the UE may get the multicast service traffic via individual delivery as a regular PDU session.

When an MB-SMF manages association of a MB Session and PDU session(s) of some UEs, if the shared N3 tunnel is activated then the MB-SMF may deactivate or release the associated PDU session(s) of the UEs per operator's policy.

6.6.2	Procedures

6.6.2.1	MBS Session initiation procedure

Figure 6.6.2.1-1: MBS session initiation procedure for multicast service

The figure depicts MBS session initiation procedure for multicast service as follows.

1.	Third party contents provider (or AF) requests MBS service(s) to an appropriate MBSF (in case that AF is not in the trust domain, the MBSF is selected via NEF) with service characteristics (e.g. service type, optional target service area, target service UE group, etc.), where the service type indicates which kinds of MBS services e.g. public safety or V2X services, etc., and target service area indicate the area in which the contents provider wants to provide service.

NOTE X:	Based on the service type, target service area etc., AF may select an appropriate MBSF/MBSU.

2.	MBSF may check whether the AF is authorized to start the MBS service with PCF.

3.	If the MBS service is authorized, an MBS service session is setup and its information (e.g. MBS service session ID, address information of the MBSU, etc.) is managed by the MBSF.

4.	The MBSF notifies the MBS service session information to the content provider or AF.

5.	MBS service announcement can be delivered to UEs in the target service area on application level, which includes the MBS service specific parameters e.g. DNN, MBS service session ID, target service UE group ID, etc. by AF or MBSF.

For UE joining the multicast group:

6.	If the UE belongs to target service UE group, the UE may join the multicast group via control plane (i.e. PDU session establishment or modification procedure, where  an MB-SMF is selected during the PDU session establishment procedure based on its DNN info, [if available MBS service session ID], target service area, etc. which are obtained in step 5), which is verified based on the UE SM subscription information. In addition, the SMF may notify the MBSF of the UE joining in the multicast group.

NOTE Y: 	The PDU session for UE joining procedure is assumed as controlled by MBS-capable SMF (i.e. MB-SMF), which can be achieved e.g. by DNN info etc. obtained by MBS service announcement.

For MB Session setup:

7.	MBSF requests an MB session setup to MB-SMF(s), which are selected based on their coverage and the target service area.

8.	The MB-SMF may get service authorization including authorized QoS info from PCF, which is authorized for the target service UE group.

9 & 10.	The MB-SMF establishes a transport tunnel between MBSU and MB-UPF for shared delivery of the MBS data.

11.	If the UE has joined, the MB-SMF associates the PDU session with the corresponding MB Session, based on the target service UE group, MBS service session ID, etc.

In the case that shared N3 tunnel does not exist yet for the MBS service area:

12-14.	For data transport of shared delivery, the MB-SMF establishes shared N3 tunnel with the NG-RAN(s) to the MB-UPF selected in the step 9 via the serving AMF. NG-RAN decides delivery mode of the MBS data i.e. point-to-point delivery or point-to-multipoint delivery e.g. based on the UE population sharing the target service UE group ID, MBS service session ID, etc, which are sent from the MB-SMF in the step 12. The remaining PDU session establishment or modification procedure is similar as the one defined in TS 23.502 [8], with the changes that the NG-RAN notifies the information on the RAN resources for point-to multipoint mode to the UE via RRC signalling.

6.6.2.2	MBS Session leave procedure

When UE is not using an MBS session anymore, UE notifies the network via the MBS Session Leave procedure.

Figure 6.6.2.2-1: MBS session leave procedure for multicast service

1.	The UE has joined multicast service as specified in clause 6.6.2.1.

2.	The UE decides to leave the multicast service and sends the PDU Session Modification Request which includes the information about the multicast service to be leaving.

3.	The MB-SMF notifies the MBSF, the UE leave the multicast session. If the UE is the last UE in this multicast session in the MB-SMF, the MB-SMF sends MBS session release request to MBSF. If MBSF receives MBS session release request, it requests the MBSU to release the tunnel between MBSU and MB-UPF.

NOTE X: subsequential UE joining procedure follows step 6~14 in the subclause 6.6.2.1.

4.	The MB-SMF initiates the PDU session modification procedure to release the QoS flow of PDU session of UE corresponding to the MBS session which UE is leaving.

5.	The MB-SMF initiates the MB session modification to NG-RAN. If the UE is the last UE for the MB session, the MB-SMF sends MB session release request to NG-RAN.

NOTE Y: Whether a separate message for releasing the MB Session can be determined during the normative phase.

6.	The NG-RAN release the UE MBS context.

7.	The NG-RAN sends MB session modification response to MB-SMF. And if the UE is the last UE for the MB session, the MB-SMF requests the MB-UPF to release the tunnel for the MB session.

6.6.2.3	MBS Session release procedure

The MBS Session release procedure is used to delete all the MBS session context for media delivery of an multicast service.

Figure 6.6.2.3-1: MBS session release procedure for multicast service

1.	An multicast service is initiated and the UE has joined multicast service as specified in clause 6.6.2.1.

2.	The Content provider/AF decides to terminate this multicast service. It sends the MBS service release request to MBSF.

3.	The MBSF releases the MBS session and the tunnel between MBSU and MB-UPF, and notifies the MBS session release to the Content provider/AF. Step 3 may follow step 5.

4.	The MBSF requests the MB-SMF(s) the MB session release.

5. 	The MB-SMF releases the MB session. Step 5 may follow step 9

6.	The MB-SMF initiates the PDU session modification procedure to release the QoS flow of PDU session of UE corresponding to the MBS session which UE has joined.

7.	The MB-SMF initiates the MB session release to NG-RAN.

8.	The NG-RAN releases all the MBS context of this MB session.

9.	The NG-RAN sends MB session release response to MB-SMF, and the MB-SMF releases the tunnel between MB-UPF and NG-RAN.

6.6.3	Impacts on services, entities and interfaces

Editor's note:	This clause describes impacts to services, entities and interfaces.

MBSF:

-	The MBSF generates and manages MBS service context and request an MB session setup to the MB-SMF

-	The MBSF controls the MBSU for user plane management, which is a media anchor of MBS traffic in 5GS.

MB-SMF:

-  The MB-SMF setup the MB session, which is used for the transport of shared delivery.

-	The MB-SMF manages the MB session context for shared delivery, and associate & manages together the UE's PDU session context for individual delivery.

-  The MB-SMF send the association information b/w shared delivery and individual delivery to the NG-RAN.

AMF:

-	The AMF is enhanced to support MB session management b/w NG-RAN and MB-SMF.

MB-UPF:

-	It supports shared tunnel for shared delivery method.

NG-RAN:

-	It supports shared tunnel for shared delivery method.

-	The NG-RAN receives the association information from the MB-SMF and stores it, which may identify the UEs to receive MBS data traffic via shared delivery method.

-  The NG-RAN may decide RAN delivery mode for MBS data traffic from shared delivery method i.e. PTP vs. PTM

N3:

-	A shared tunnel on this interface is established for an MB session.

UE:

-	UE need to indicate its joining on the MBS service as part of the NAS message (e.g., PDU Session establishment/modification request).

******************************** END OF CHANGES **************************************