3GPP TSG-WG SA2 Meeting #142E	S2-2008570

Elbonia, November 16 – 20, 2020	(revision of S2-20xxxx)

Source:	Qualcomm Incorporated

Title:	Conclusion for Key issue #3

Document for:	Approval

Agenda Item:	8.10

Work Item / Release:	FS_MUSIM / Rel-17

Abstract: This contribution proposes the discussion and conclusion about Busy Indication.

# 1. Discussion

One controversial issue for KI#3 is about the RAN-triggered leaving procedure and NAS-triggered leaving procedure.

Based on RAN’s discussion, the option 5 is not in the scope, so for option 5, only NAS-triggered leaving procedure is allowed.

Proposal 1: For leaving in E-UTRAN/5GS access, the NAS-triggered leaving procedure is recommended to be supported.

For NR/5GS case, if UE leaves for a short determine time, e.g., periodic RNAU, periodic RAU, paging for SMS (if it is supported by paging cause), UE can initiate a RRC-leaving procedure and goes to RRC_Inactive mode, in this case there is no need by the UE sends a expected leaving time to RAN. If the periodic RNAU timer is expired and UE is not able to resume before the periodic RNAU timer expiration, RAN will change UE from RRC_Inactive to RRC_Idle mode.

For NR/5GS case, if UE leaves for a undetermined time, e.g. paging for voice, it is more proper for UE sends a NAS-leaving procedure and goes to IDLE mode, UE can include the PDU session as an assistant information to AMF for the 5GC to suppression of MT data/signalling delivery.

Proposal 2: For NR/5GS case, if UE leaves for a short determine time, e.g., periodic RNAU, periodic RAU, paging for SMS (if it is supported by paging cause), the RRC-triggered leaving procedure is recommended to be supported, UE will enter RRC_Inactive mode after leaving procedure.

Proposal 3: For NR/5GS case, if UE leaves for an undetermined time, e.g. paging for voice, the NAS-triggered leaving procedure is recommended to be supported, UE will enter RRC_Inactie mode after leaving procedure.

# 2. Proposal

* * * * Start of Changes * * * *

## 8.3	Conclusions for Key Issue #3: Coordinated leaving for Multi-USIM device

Based on the evaluation in clause 7.3 the following interim conclusions are agreed for the baseline functionality:

-	For leaving in E-UTRA/EPS access, the NAS-triggered leaving procedure is recommended to be supported.

-	For leaving in E-UTRA/5GS access, the NAS-triggered leaving procedure is recommended to be supported..

-	For leaving in NR/5GS access,

-	If UE leaves for a short deterministic time, e.g., periodic RNAU, periodic RAU, paging for SMS (if it is  supported by paging cause), the RRC-triggered leaving procedure is recommended to be supported, UE will enter RRC_Inactive mode after leaving procedure.

-	If UE leaves for an indeterministic time, e.g. paging for voice, the NAS-triggered leaving procedure is recommended to be supported, UE will enter RRC_Inactie mode after leaving procedure.

-	If UE is in RRC_Inactive mode, the RRC-triggered resume procedure is recommended to support after UE back to the access.

-	If UE is in IDLE mode, the NAS-triggered resume procedure is recommended to support after UE back to the access.

-	For NAS level leaving procedure (5GS and EPS case), the UE provides leaving indication to the CN and the UE may provide assistance information to the network in the leaving procedure regarding MT data/signalling handling in the CN.

-	The assistance information may include information to temporarily restrict/filter MT data in this network while the UE has left:

-	An indication that the UE should only be paged for IMS voice,

-	An indication that the UE should not be paged at all.

-	The PDU Sessions that are subject to suppression of MT data/signalling delivery.

Editor's note:	Whether resume the MT data delivery and stop the paging filtering when the UE returns shortly, e.g. in order to send the mobility registration, busy indication, is FFS.

NOTE:	The details on the conclusion of the leaving procedure at UE side, including whether there is acknowledgement to the leave/release request message (RRC and/or NAS), will be synched with CT WG1 and RAN WGs in normative work.

* * * * End of Changes * * * *